3GPP TSG-RAN WG4 # 104-e	R4-2212448

Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.17.4.1.2

Source:	MediaTek Inc.

Title:	Discussion on demodulation performance requirements definition for Rel17 multi-TRP

Document for:	Discussion

# Introduction

In RAN4#103-e, the detail for the simulation assumption for multi-TRP PDCCH with FDM repetition scheme was agreed [1]. We provide discussion with simulation results in this contribution.

# Discussion

The following options in Issue 1-1-1 and 1-2-1 were agreed for further discussion in the previous meeting [1] and are discussed in this document.

Issue 1-1-1: Whether to define PDCCH requirement for multi-TRP repetition transmission schemes

Agreements:

- Introduce PDCCH requirements for multi-TRP repetition transmission

  - Option 1: With UE soft-combining

  - Option 2: Without UE soft-combining

There exist several UE implementation assumptions that may or may not include UE soft-combining. It is known that performance with UE soft-combining is better than without UE soft-combining. If requirements would be defined based on UE soft-combining assumption that may be a problem to UEs not implementing soft-combining. Therefore, we are proposing to define requirements without UE soft-combining assumption.

Proposal #1: We support Option 2 to define PDCCH requirement for multi-TRP repetition transmission scheme without UE soft-combing assumption.

Issue 1-2-1: PDCCH requirement setup

Agreements:

- Aggregation level

  - Option 1: AL=2

  - Option 2: AL=4

  - Decide aggregation level base on simulation results in next meeting.

- Operating SNR condition for selection AL

  - Option 1: SNR > [-4] dB

  - Other options are not precluded

- SNR setting for each TRP

  - Option 1: Balance SNR (no blocking considered)

    - The SNRs for TRP #1 and TRP #2 are assumed to be balanced with a scaling factor of 1/sqrt(2) for the transmitted signal from each TRP

  - Option 2: Blocked TRP testing

    - Blocked TRP testing for Option 2 in Issue 1-1-1 (without UE soft-combining): Probability of blocking p=10% with X=10 dB blocking as a starting point

  - Other options are not precluded

- Time offset/Frequency offset for each TRP

  - Same value as single-DCI PDSCH with FDM scheme as starting point

    - 15KHz SCS: timing offset =-0.5us, frequency offset=200Hz

    - 30KHz SCS: timing offset =-0.25us, frequency offset=300Hz

The simulation settings are based on the agreed assumptions from 103-e WF [1] and 102-e WF [2] documents and shown in Table 1. Multi-transmission point (TRP) PDCCH repetition is simulated with agreed Aggregation Levels (AL) 2 and 4 with number of receiver antenna 2 and 4 as suggested in 103-e WF [1].

Table 1: Simulation settings.

| Parameter | Value | Value |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| Repetition transmission schemes | FDM | FDM |
| CBW | 10 MHz | 40 MHz |
| CORESET RB | 24 | 48 |
| CORESET duration | 2 | 2 |
| Aggregation level for repetition | 2/4 | 2/4 |
| Propagation Condition | TDLA30-10 | TDLA30-10 |
| Timing offset | -0.5us | -0.25us |
| Frequency offset | 200Hz | 300Hz |
| Antenna configuration | 2x2, 2x4 | 2x2, 2x4 |
| CCE to REG mapping type | nonInterleaved | nonInterleaved |
| REG bundle size | 6 | 6 |
| Payload bits(without CRC) | 39 | 41 |
| Test metric | 1% of Pm-dsg (%) | 1% of Pm-dsg (%) |

The first set of simulation results are collected in Table 2 where SNR setting for each TRP uses Option 1 Balance SNR. This means the SNRs for TRP #1 and TRP #2 are assumed to be balanced with a scaling factor of 1/sqrt(2) for the transmitted signal from each TRP.

The second set of simulation results are collected in Table 3 where SNR setting for each TRP uses Option 1 Blocked TRP. This means one TRP is assumed to be blocked by 10dB with probability of 10%. In these simulations SNRs for TRP #1 and TRP #2 are not scaled by 1/sqrt(2) as in Table 2 results.

Table 2: Simulation results of SNR setting for each TRP using Option 1 Balance SNR

| Duplex scheme | Antenna config | Aggregation level | SNR [dB] with UE soft-combining | SNR [dB] without UE soft- combining |
| --- | --- | --- | --- | --- |
| FDD | 2x2 | 2 | 2.41 | 4.03 |
| FDD | 2x2 | 4 | -0.75 | 0.68 |
| FDD | 2x4 | 2 | -1.50 | 0.07 |
| FDD | 2x4 | 4 | -4.17 | -2.86 |
| TDD | 2x2 | 2 | 2.57 | 4.15 |
| TDD | 2x2 | 4 | -0.83 | 0.58 |
| TDD | 2x4 | 2 | -1.40 | 0.11 |
| TDD | 2x4 | 4 | -4.22 | -2.87 |

Table 3: Simulation results of SNR setting for each TRP using Option 2 Blocked TRP

| Duplex scheme | Antenna config | Aggregation level | SNR [dB] with UE soft-combining | SNR [dB] without UE soft- combining |
| --- | --- | --- | --- | --- |
| FDD | 2x2 | 2 | 1.36 | 2.68 |
| FDD | 2x2 | 4 | -1.66 | -0.92 |
| FDD | 2x4 | 2 | -2.34 | -1.60 |
| FDD | 2x4 | 4 | -4.95 | -4.61 |
| TDD | 2x2 | 2 | 1.56 | 2.97 |
| TDD | 2x2 | 4 | -1.85 | -0.97 |
| TDD | 2x4 | 2 | -2.05 | -1.45 |
| TDD | 2x4 | 4 | -4.28 | -4.24 |

For SNR definition we think that using Option 1 with balanced SNR is simple and adequate method to verify multi-TRP Rel-17 functionality and performance. Therefore, we do not see need for more complicated Option 2 Blocked TRP test methodology.

Proposal #2: We support Option 1 for SNR setting for each TRP that corresponds balanced SNR.

For aggregation level definition we think that using higher AL4 especially with 4 receiver antennas requires quite low SNR requirements and therefore we would slightly prefer AL2. However, required SNR of AL4 is within reasonable levels and is acceptable for us if other companies mostly prefer that option.

Proposal #3: We prefer Option 1 for aggregation level that corresponds value 2.

# Conclusion

In this paper we provided the view on the requirements definition for multi-TRP Rel-17 enhancements. The following observations and proposals are made:

Proposal #1: We support Option 2 to define PDCCH requirement for multi-TRP repetition transmission scheme without UE soft-combing assumption.

Proposal #2: We support Option 1 for SNR setting for each TRP that corresponds balanced SNR.

Proposal #3: We prefer Option 1 for aggregation level that corresponds value 2.

# Reference

- R4-2210671, “WF on demodulation requirement for Enhancement on Multi-TRP”, Huawei, HiSilicon

- R4-2207207, “WF on demodulation requirement for Enhancement on Multi-TRP”, Huawei, HiSilicon