3GPP TSG-RAN4 Meeting #104-e	R4-2212450

# Electronic Meeting,  15 - 26 August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of A-MPR for NS_05 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_PC2_UE_FDD |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current NS_05 A-MPR requirements in region A for PC3 and PC2 do not allow sufficient margin for practical APT/ET PA implementations, while ample margin is observed in region C for PC2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Further divide region A into three partitions. Add extra margin to the A-MPR values in one of the partitions and reduce margin from another partition. Additionally, reduce the margin in region C for PC2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE may fail to meet the A-MPR requirements in region A, while the system may lose potential performance gain in region C.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<Start of changes>>>

#### 6.2.3.4	A-MPR for NS_05 and NS_05U

Table 6.2.3.4-1: A-MPR regions for NS_05 and NS_05U (Power Class 3)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 | 1922.5 ≤ FC < 1927.5 | < 1.62 MHz/12/SCS | > 2.52 MHz/12/SCS | A3 |  |  |  |  |  |  |
| 10 | 1925 ≤ FC < 1935 | ≤ 1.62 MHz/12/SCS | > 4.5 MHz/12/SCS | A9 | > 1.62 MHz/12/SCS ≤ 3.60 MHz/12/SCS | > 5.4 MHz/12/SCS | A7 | ≥ 7.2 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
|  |  | ≤ 0.9 MHz/12/SCS | ≤ 4.5 MHz/12/SCS | A1 |  |  |  |  |  |  |
|  |  | > 0.9 MHz/12/SCS ≤ 1.62 MHz/12/SCS | ≤ 4.5 MHz/12/SCS | A10 |  |  |  |  |  |  |
| 10 | 1935 ≤ FC < 1945 |  | > 4.5 MHz/12/SCS | A4 |  |  |  |  |  |  |
| 15 | 1927.5 ≤ FC < 1932.5 | ≤ 3.24MHz/12/SCS | > 4.68 MHz/12/SCS | A9 | > 3.24 MHz/12/SCS ≤ 5.40 MHz/12/SCS | > 8.1 MHz/12/SCS | A7 | ≥ 10.08 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
|  |  | ≤ 1.8 MHz/12/SCS | ≤ 4.68 MHz/12/SCS | A1 |  |  |  |  |  |  |
|  |  | > 1.8 MHz/12/SCS ≤ 3.24 MHz/12/SCS | ≤ 4.68 MHz/12/SCS | A10 |  |  |  |  |  |  |
| 15 | 1932.5 ≤ FC < 1942.5 | < 1.62 MHz/12/SCS | > 0 | A1 |  |  |  | ≥ 12.24 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
| 15 | 1942.5 ≤ FC < 1947.5 |  | > 7.2 MHz/12/SCS | A5 |  |  |  |  |  |  |
| 20 | 1930 ≤ FC < 1950 | ≤ 4.86 MHz/12/SCS | > 4.86 MHz/12/SCS | A9 | > 4.86 MHz/12/SCS ≤ 7.20 MHz/12/SCS | > 9.0 MHz/12/SCS | A7 | ≥ 13.68 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
|  |  | ≤ 2.7 MHz/12/SCS | ≤ 4.86 MHz/12/SCS | A1 |  |  |  |  |  |  |
|  |  | > 2.7 MHz/12/SCS ≤ 4.86 MHz/12/SCS | ≤ 4.86 MHz/12/SCS | A10 |  |  |  |  |  |  |
| 20 | 1950 ≤ FC < 1960 |  | > 9.0 MHz/12/SCS | A6 |  |  |  |  |  |  |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.4-2, 6.2.3.4-3 and 6.2.3.4-10. NOTE 2: Void |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4-2: A-MPR for NS_05 and NS_05U (Power Class 3)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 10 | ≤ 5 | ≤ 4 | N/A |
|  | QPSK | ≤ 10 | ≤ 5 | ≤ 4.5 |  |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 6 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 6 |  |
|  | 256 QAM | ≤ 13 | ≤ 5 | ≤ 7 |  |
| CP-OFDM | QPSK | ≤ 10 | ≤ 5 | ≤ 7.5 |  |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 8 |  |
|  | 256 QAM | ≤ 13 |  | ≤ 10 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |

Table 6.2.3.4-3: A-MPR for NS_05 (Power Class 3)

| Modulation/Waveform |  |  | A4 (dB) |  |  | A5 (dB) |  | A6 (dB) |  |  | A7 (dB) |  | A9 (dB) |  | A10 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 |  | N/A | ≤ 1 |  | N/A |  | ≤ 1 | N/A |  | ≤ 6 |  | ≤ 12 |  | ≤ 7 |  |
|  | QPSK |  |  | ≤ 1.5 |  | ≤ 1.5 |  | ≤ 6 | ≤ 12 | ≤ 7 |
|  | 16 QAM |  |  |  |  |  |  | ≤ 6 | ≤ 12 | ≤ 7 |
|  | 64 QAM |  |  |  |  |  |  | ≤ 6 | ≤ 13 | ≤ 8 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 | ≤ 15 | ≤ 10 |
| CP-OFDM | QPSK | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 | ≤ 12 | ≤ 7 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 | ≤ 12 | ≤ 7 |
|  | 64 QAM |  |  |  |  |  |  | ≤ 6 | ≤ 13 | ≤ 8 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 | ≤ 15 | ≤ 10 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4-4 - Table 6.2.3.4-9: Void

Table 6.2.3.4-10: A-MPR for modulation and waveform type for NS_05U (Power Class 3)

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 2 | N/A | ≤ 2 | N/A | ≤ 6 |
|  | QPSK | ≤ 2 |  | ≤ 2 |  | ≤ 2 |  | ≤ 6 |
|  | 16 QAM | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 6 |
|  | 64 QAM | ≤ 3 |  | ≤ 3 |  | ≤ 3 |  | ≤ 6 |
|  | 256 QAM | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 6 |
| CP-OFDM | QPSK | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 16 QAM | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 64 QAM | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 256 QAM | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |

Table 6.2.3.4-11: A-MPR regions for NS_05 and NS_05U (Power Class 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 | 1922.5 ≤ FC < 1927.5 | < 1.98 MHz/12/SCS | > 1.44 MHz/12/SCS | A3 | < 0.72 MHz/12/SCS | ≤ 1.44 MHz/12/SCS | A4 |  |  |  |
| 10 | 1925 ≤ FC < 1935 | < 1.98 MHz/12/SCS | > 3.6 MHz/12/SCS | A9 | ≥ 1.98 MHz/12/SCS | > max(0, RBstart-1.08 MHz/12/SCS) | A7 | ≥ 7.2 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
|  |  | ≤ 1.08 MHz/12/SCS | ≤ 3.6 MHz/12/SCS | A1 | ≥ 1.98, ≤2.7 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A8 |  |  |  |
|  |  | > 1.08 MHz/12/SCS ≤ 1.98 MHz/12/SCS | ≤ 3.6 MHz/12/SCS | A10 |  |  |  |  |  |  |
| 10 | 1935 ≤ FC < 1945 |  | > 3.96 MHz/12/SCS | A4 |  |  |  |  |  |  |
| 15 | 1927.5 ≤ FC < 1932.5 | < 3.6 MHz/12/SCS | > 4.14 MHz/12/SCS | A9 | ≥ 3.6 MHz/12/SCS | > max(0, RBstart-1.8 MHz/12/SCS) | A7 | ≥ 10.08 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
|  |  | ≤ 2.0 MHz/12/SCS | ≤ 4.14 MHz/12/SCS | A1 | ≥ 3.6, ≤4.68 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A8 |  |  |  |
|  |  | > 2.0 MHz/12/SCS ≤ 3.6 MHz/12/SCS | ≤ 4.14 MHz/12/SCS | A10 |  |  |  |  |  |  |
| 15 | 1932.5 ≤ FC < 1942.5 | < 1.98 MHz/12/SCS | > 0 | A1 | ≥ 1.98 MHz/12/SCS | > max(0, RBstart+1.08 MHz/12/SCS) | A7 | ≥ 12.24 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
| 15 | 1942.5 ≤ FC < 1947.5 |  | > 5.04 MHz/12/SCS | A5 |  |  |  |  |  |  |
| 20 | 1930 ≤ FC < 1950 | < 5.04 MHz/12/SCS | > 4.68 MHz/12/SCS | A9 | ≥ 5.04 MHz/12/SCS | > max(0, RBstart-3.6 MHz/12/SCS) | A7 | ≥ 13.68 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
|  |  | ≤ 2.8 MHz/12/SCS | ≤ 4.68 MHz/12/SCS | A1 | ≥ 5.04, ≤6.66 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A8 |  |  |  |
|  |  | > 2.8 MHz/12/SCS ≤ 4.86 MHz/12/SCS | ≤ 4.68 MHz/12/SCS | A10 |  |  |  |  |  |  |
| 20 | 1950 ≤ FC < 1960 |  | > 9.0 MHz/12/SCS | A6 |  |  |  |  |  |  |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.4-12 and 6.2.3.4-13. NOTE 2: Void |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4-12: A-MPR for NS_05 and NS_05U (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 13 | ≤ 8 | ≤ 6.5 | ≤ 2 |
|  | QPSK | ≤ 13 | ≤ 8 | ≤ 7 | ≤ 2 |
|  | 16 QAM | ≤ 13 | ≤ 8 | ≤ 8.5 | ≤ 2 |
|  | 64 QAM | ≤ 14 | ≤ 8 | ≤ 9 | ≤ 2 |
|  | 256 QAM | ≤ 15 | ≤ 8 | ≤ 9.5 |  |
| CP-OFDM | QPSK | ≤ 13 | ≤ 8 | ≤ 10 | ≤ 4 |
|  | 16 QAM | ≤ 13 | ≤ 8 | ≤ 10 | ≤ 4 |
|  | 64 QAM | ≤ 14 | ≤ 8 | ≤ 10 | ≤ 4 |
|  | 256 QAM | ≤ 16 | ≤ 8 | ≤ 10 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |

Table 6.2.3.4-13: A-MPR for NS_05 and NS_05U (Power Class 2)

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) | A8 (dB) | A9 (dB) | A10 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer |  | Inner | Outer |  | Inner | Outer |  | Inner | Outer/Inner |  | Outer/Inner |  | Outer/Inner |  | Outer/Inner |  |
| DFT-s-OFDM |  | Pi/2 BPSK | ≤ 3 |  | N/A | ≤ 2 |  |  | ≤ 2 |  | N/A | ≤ 8 |  | ≤ 3.5 |  | ≤ 15 |  | ≤ 10 |  |
|  |  | QPSK | ≤ 3 |  |  | ≤ 2 |  |  | ≤ 2 |  |  | ≤ 8 |  | ≤ 3.5 |  | ≤ 15 |  | ≤ 10 |  |
|  |  | 16 QAM | ≤ 3.5 |  |  | ≤ 2.5 |  |  | ≤ 2 |  |  | ≤ 8 |  | ≤ 3.5 |  | ≤ 15 |  | ≤ 10 |  |
|  |  | 64 QAM | ≤ 3.5 |  |  | ≤ 2.5 |  |  |  |  |  | ≤ 8 |  | ≤ 3.5 |  | ≤ 16 |  | ≤ 11 |  |
|  |  | 256 QAM |  |  |  |  |  |  |  |  |  | ≤ 8 |  |  |  | ≤ 17 |  | ≤ 12 |  |
| CP-OFDM |  | QPSK | ≤ 4.5 |  |  | ≤ 4.5 |  |  | ≤ 4 |  |  | ≤ 8.5 |  | ≤ 3.5 |  | ≤ 15 |  | ≤ 10 |  |
|  |  | 16 QAM | ≤ 4.5 |  |  | ≤ 4.5 |  |  | ≤ 4 |  |  | ≤ 8.5 |  | ≤ 3.5 |  | ≤ 15 |  | ≤ 10 |  |
|  |  | 64 QAM | ≤ 5 |  |  | ≤ 5 |  | ≤ 5 | ≤ 4 |  |  | ≤ 8.5 |  |  |  | ≤ 16 |  | ≤ 11 |  |
|  |  | 256 QAM |  |  |  |  |  |  |  |  |  | ≤ 8.5 |  |  |  | ≤ 17 |  | ≤ 13 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<<Next changes>>>

# L.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPR-Behavior indicated per supported NR band in the IE RF-Parameters [7] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination.

NOTE 1:	In the present release, the modifiedMPR-Behavior is indicated [7] by an 8-bit bitmap per supported NR band.

Table L.1-1: Definitions of the bits in the field modifiedMPR-Behavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n30 | 0 (leftmost bit) | Requirements for network signalling value NS_21 as defined in Clause 6.5.2.3.y of 38.101-1 v17.6.0 and A-MPR as defined in Clause 6.2.3.14 of 38.101-1 v17.6.0. | This bit shall be set to 1 by a UE supporting the Rel-17 version of the specification. If the bit is not set, then requirements for NS_21 as defined in Clause 6.5.2.3.3 of 38.101-1 v16.11.0 and A-MPR as defined in Clause 6.2.3.14 of 38.101-1 v16.11.0 apply. |
| n41 | 0 (leftmost bit) | - EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
|  | 1 | - EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | - EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |
|  | 3 | PC 1.5 MPR as defined in Table 6.2D.2-3 of 38.101-1 v17.3.0 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, MPR in Table 6.2D.2-2 of 38.101-1 v17.3.0 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| n71 | 0 (leftmost bit) | - EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)71AA UE EN-DC |
| n77 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 of 38.101-1 v17.3.0 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-3 of 38.101-1 v17.3.0 of 38.101-1 v17.3.0 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| n78 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 of 38.101-1 v17.3.0 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, MPR in Table 6.2D.2-2 of 38.101-1 v17.3.0 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| n79 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 of 38.101-1 v17.3.0 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, MPR in Table 6.2D.2-2 of 38.101-1 v17.3.0 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| n1 | 0 (leftmost bit) | - The A-MPR associated with NS_05 for Band n1 in clause 6.2.3.4 of 38.101-1 v17.7.0 | - This bit shall be set to 1 by a UE of any release supporting A-MPR associated to NS_05 for band n1. |
| n65 | 0 (leftmost bit) | - The A-MPR associated with NS_05 for Band n65 in clause 6.2.3.4 of 38.101-1 v17.7.0 | - This bit shall be set to 1 by a UE of any release supporting A-MPR associated to NS_05 for band n65. |
| n84 | 0 (leftmost bit) | - The A-MPR associated with NS_05 for Band n84 in clause 6.2.3.4 of 38.101-1 v17.7.0 | - This bit shall be set to 1 by a UE of any release supporting A-MPR associated to NS_05 for band n84. |

<<<End of changes>>>