3GPP TSG-RAN WG4 Meeting #104-e	R4- 2212452

Electronic Meeting, 15 – 26 August 2022

Source: 	Huawei, HiSilicon

Title:	Discussions on the feasibility of improving MSD

Agenda Item:	11.6.4.1

Document for:	Agreement

# 1	Introduction

A new WID [1] for Rel-18 FR1 UE RF enhancement was established in RAN#95. One of the tasks for RAN4 is to investigate the feasibility of lower MSD for inter-band CA/EN-DC/DC combinations. As per the agreement on the WID objectives, we will in the sequel select a few example band combinations to cover different types of MSDs (harmonics, harmonic mixing, IMD and cross band isolation), and discuss the feasibility of MSD improvement for different MSD sources.

# 2	Discussion

As proposed by several companies in previous discussions, the combination of band n3 and n78/n77 (or the equivalent E-UTRA bands) is a good example for studying MSD. As specified in TS 38.101-1[2], CA_n3-n78 suffers from UL harmonic interference and IMD interference due to 2UL CA. The allowed reference sensitivity exceptions are duplicated below in Table 1 and 2.

Table 1: MSD due to harmonic interference for CA_n3-n78 [2]

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |

Table 2: MSD due to 2UL IMD for CA_n3-n78[2]

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  |  |  |  |  |  | 28.75 |  |  |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  |  |  |  |  |  | 10.75 |  |  |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 2.2 | FDD | IMD7 |
|  | n78 | 3305 3780 | 10 10 | 1 (RBstart=3) 1 (RBstart=0) | 3305 3780 | N/A | TDD | N/A |

Following the commonly used methodology for estimating MSD, we evaluate the effect of improving PCB isolation as well as antenna isolation. The MSD results produced by the analytical model are depicted in Figure 1, whose source is the IMD2 of the dual UL.

Figure 1: MSD caused by IMD2 for CA_n3-n78 varies with different PCB isolation and antenna isolation. The 40dB antenna isolation is impractical for small form-factor UEs but serves as an performance indicator for conductive measurements.

It can be seen that:

Observation 1: Improving the PCB isolation can reduce the MSD, but the effect will soon bottom out once the PCB isolation reaches certain value （e.g. >70 dB）.

Observation 2: The antenna isolation between the primary branch and the diversity branch is an important factor. The MSD improvement can be almost dB-for-dB once the PCB isolation is large enough (e.g. >70dB).

It’s worth noting that the effect of antenna isolation may not be observed in conductive tests. This probably explains why small or little MSD was shown in some reported measurements, whereas large MSDs are specified by 3GPP.

In the above analysis, the Tx power is assumed to be at its maximum, i.e. 20 dBm for both band n3 and n78. This may not always be the case due to changing path loss, or the limitation of MPR/A-MPR. Figure 2 shows the variation in MSD due to the changes in Tx power levels.

Figure 2: MSD caused by IMD2 for CA_n3-n78 varies with different Tx power levels.

It can be seen that:

Observation 3: The MSD caused by IMD2 decreases with the Tx power level at a ratio of 2:1. For MSDs caused by higher order IMD, the ratio of reduction may be more.

It is worth noting that the MSD specified by 3GPP always assume the maximum Tx power of the nominal power class. In practical networks, even the cell edge UEs may not be able to transmit at the nominal maximum power due to the limitations of MPR or A-MPR. For UEs that are away from the cell edge, the required Tx power could be even less due to reduced path loss.

Observation 4: The actual MSD experienced by the UE depends on the Tx power levels.

Similar trends have been observed for MSD caused by other sources. For example, CA_n3-n78 also suffers from the interference caused by the 2nd harmonic of the UL in band n3. The MSD analysis was carried out in [3] for a similar LTE band combination CA_3A-42A, and the result is duplicated in Figure 3.

Figure 3: MSD caused by H2 of B3 for CA_3A-42A [3]

Apart from improving the antenna isolation and PCB isolation, other options can also be considered, such as adding extra harmonic filters or performing interference cancellation at the receiver baseband. The drawbacks of such schemes are obvious, including higher insertion loss and component cost, increased complexity and power consumption, etc.

It is also worth noting that the MSDs from different sources may not happen at the same time. In order for the self-interference to fall within the own receiver, the aggressors’ carrier frequencies and the victims’ carrier frequencies need to satisfy certain mathematical constraints, subject to the order of the IMD or harmonics. Figure 4 indicates the occurrences of IM interferences for CA_n3-n78.

Figure 4: Illustration of the relationship between IMD and the carrier frequencies. The red color indicates that the IM product may fall into the victim Rx channel, assuming CBW: 10MHz for n3, 20MHz for n78.

It can be seen that for the majority carrier frequency configurations of CA_n3-n78, there’s no self-interference from IMD2 or IMD4. Furthermore, when MSD from IMD2 happens for a given deployment, MSD from IMD4 does not, or vice visa. Since n78 is a TDD band, the MSD caused by H2 of n3 will not happen at the same time as the MSD caused by IMD2 or IMD4 of the dual UL.

Observation 5: MSD from different sources may not happen simultaneously, depending on the configuration of carrier frequencies, duplex mode of the component bands, the order of IMD or harmonics and etc.

Since 3GPP only specifies MSD requirements for the worst cases, it’s important for the network scheduler to evaluate the MSD situation for the current deployment and decide which MSD really matters.

In addition to CA_n3-n78, we propose the following example band combinations for further study:

Proposal 1: In addition to CA_n3-n78, CA_n1-n40 may be used as an example band combination to further study the MSD caused by cross-band isolation. And CA_n1-n3-n78 may be used as an example to study the MSD caused by IMD falling into a 3rd band.

Finally, if 3GPP is to specify improved MSD requirements or low MSD capability, any performance gain claimed by the UE should be verifiable. As discussed before, the antenna isolation affects the MSD significantly but is usually not shown in conductive tests. Hence, we propose:

Proposal 2: Any declared MSD improvement should be testable. FFS whether conductive tests would be sufficient.

# 3	Conclusion

By using the example band combination of CA_n3-n78, we conduct the initial study of the feasibility of MSD improvement. The following observations and proposal have been presented:

Observation 1: Improving the PCB isolation can reduce the MSD, but the effect will soon bottom out once the PCB isolation reaches certain value （e.g. >70 dB）.

Observation 2: The antenna isolation between the primary branch and the diversity branch is an important factor. The MSD improvement can be almost dB-for-dB once the PCB isolation is large enough (e.g. >70dB).

Observation 3: The MSD caused by IMD2 decreases with the Tx power level at a ratio of 2:1. For MSDs caused by higher order IMD, the ratio of reduction may be more.

Observation 4: The actual MSD experienced by the UE depends on the Tx power levels.

Observation 5: MSD from different sources may not happen simultaneously, depending on the configuration of carrier frequencies, duplex mode of component bands, the order of IMD or harmonics and etc.

Proposal 1: In addition to CA_n3-n78, CA_n1-n40 may be used as an example band combination to further study the MSD caused by cross-band isolation. And CA_n1-n3-n78 may be used as an example to study the MSD caused by IMD falling into a 3rd band.

Proposal 2: Any declared MSD improvement should be testable. FFS whether conductive tests would be sufficient.

# References

- RP-220987 New WID Further RF requirements enhancement for NR frequency range 1 (FR1), Huawei, HiSilicon, RAN#95

- 3GPP TS 38.101-1 v17.6.0

- R4-2112381 Views on defining low MSD for CA and DC, Apple, RAN4#100