3GPP TSG-RAN WG4 Meeting #104-e	R4- 2212454

Electronic Meeting, 15 – 26 August 2022

Source: 	Huawei, HiSilicon

Title:	Discussions on IoT NTN UE RF requirements

Agenda Item:	12.5.4

Document for:	Agreement

# 1	Introduction

- Specification of RF requirements for Satellite Access Node (SAN) and UE including the following [RAN4]:

- Specification of a 200 kHz channel raster in bands where this is feasible.

- In bands where it is not feasible to define a 200 kHz channel raster, the specification of a 100 kHz channel raster to be used in conjunction with signalling of the “part-of EARFCN” indication on MIB, with multiple EARFCN hypotheses.

- Verification of co-existence of IoT NTN with TN, re-using or extrapolating from existing coexistence results (from NR NTN or other) where appropriate, and considering additional simulations as necessary.

- Leveraging the studies and requirements (where applicable) of NTN NR bands n256 and n255 (and any relevant E-UTRA bands), specify the following new FDD frequency bands for NB-IoT/eMTC NTN operation:

  - S-band (1980-2010 MHz in UL, and 2170-2200 MHz in DL)

  - L band (1626.5 MHz – 1660.5 MHz in UL, and 1525 MHz – 1559 MHz in DL)

The specification work of this WI shall:

- Be limited to FDD requirements applicable for NB1/NB2 and Cat-M1 UE categories (HD-FDD and FD-FDD for Cat-M1).

- Re-use the framework and requirements from NB-IoT/eMTC, as well as NR NTN requirements, where applicable.

NOTE: Rel-17 IoT NTN specifications do not cover non-NTN NB-IoT/eMTC functionality defined later than Rel-16.

A new WID [1] on NB-IoT/eMTC core & performance requirements for NTN was established in RAN#96. The objectives on core RF requirements are duplicated below:

In the following, we discuss some selected issues related to the UE RF requirements.

# 2	Discussion

In order to take advantage of the existing NB-IoT/eMTC ecosystem, it would be beneficial to reuse the corresponding TN UE RF requirements as much as possible. Some requirements may be even relaxed, considering the NTN characteristics.

## 2.1 Operating bands and channel arrangement

- Operating bands

The definition could follow the example of NR-NTN.

Table 1: NTN satellite bands

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| 256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| 255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from 256. |  |  |  |

- UE channel bandwidth

For Cat-M1, the CBW to be supported is 1.4MHz; for Cat-NB1/NB2, the CBW is 200kHz.

- Channel raster

As per the WID [1], a 200 kHz channel raster is the preferred choice, and 100kHz is the fallback option if the former is deemed infeasible.  So far, no blocking issue has been identified for the 200kHz channel raster.

- Carrier frequency and Absolute Radio Frequency Channel Number (ARFCN)

Two options of ARFCN mapping for IoT NTN are shared below, i.e. follow the convention of NR-NTN or follow the convention of E-UTRA.

Option 1: Follow NR-NTN and define the ARFCN for IoT NTN

Table 2: Applicable NR-ARFCN per operating band

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| 256 | 200 | 396001 – <40> – 401999 | 434001 – <40> – 439999 |
| 255 | 200 | 325301 – <40> – 332099 | 305001 – <40> – 311799 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |

Note that the ARFCN corresponding to band edge frequencies are excluded such as 2170MHz and 2200MHz for band 256 DL, since they cannot be used as carrier frequencies.

Option 2: Follow E-UTRA and define the ARFCN for IoT NTN

The first step is to define new E-UTRA bands and the corresponding EARFCN. Since the EARFCN uses 18 bits, it’s proposed to start from 262143 (=2^18-1) for the last EARFCN for band 256, and then continue in descending order to assign the EARFCN for the uplink of band 256 and then band 255, etc. For the downlink, the assignment of EARFCN may start from 229375, which is equal to 2^18-2^15-1. The details of the new EARFCN are shown in Table 3.

Table 3: E-UTRA channel numbers

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 256 | 2170 | 229076 | 229076 – 229375 | 1980 | 261844 | 261844 – 262143 |
| 255 | 1525 | 228736 | 228736 – 229075 | 1626.5 | 261504 | 261504 – 261843 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |  |  |  |

The second step is to select the applicable EARFCN for IoT NTN bands as shown in Table 4.

Table 4: Applicable EARFCN per operating band

| NTN satellite operating band | ΔFRaster (kHz) | Downlink Range of NDL (First – <Step size> – Last) | Uplink Range of NUL (First – <Step size> – Last) |
| --- | --- | --- | --- |
| 256 | 200 | 229077 – <2> – 229375 | 261845 – <2> – 262143 |
| 255 | 200 | 228737 – <2> – 229075 | 261505 – <2> – 261843 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |

## 2.2 Transmitter characteristics

- UE maximum output power

Both PC5 and PC3 are to be supported, the feasibility of which has been confirmed in [2].

- ACLR & SEM

It’s expected that the same requirements as the TN UEs would be sufficient for NTN UEs. For Cat NB1/NB2, it’s not expected that such NTN UEs would coexist with GSM UEs on the same band. Hence, the GSM_ACLR requirement for Cat NB1/NB2 may be removed. Actually, the OBW requirement implies a more stringent ACLR (23 dB) than the GSM_ACLR (20 dB).

- Spurious emissions for UE co-existence

The NTN band 255 has the same frequency range as E-UTRA band 24, and the NTN band 256 is a sub-block of E-UTRA band 65. It’d be reasonable to reuse the spurious emission requirements for protected bands of the corresponding E-UTRA bands.

## 2.3 Receiver characteristics

- Maximum input level

The same requirement for NR-NTN UE may be reused, i.e. -40 dBm.

- ACS

It’s expected that the same requirements as the TN UEs would be sufficient for NTN UEs. For Cat NB1/NB2, it’s not expected that such NTN UEs would coexist with GSM UEs on the same band. Hence, the GSM ACS requirement for Cat NB1/NB2 may be removed.

# 3	Conclusion

In this paper, we have shared our initial views on the IoT NTN UE RF requirements. Particularly, two options are proposed for the definition of NTN ARFCN based on a 200 kHz channel raster.

# References

- RP-221556 WID on NB-IoT/eMTC core & performance requirements for Non-Terrestrial Networks, MediaTek, RAN#96

- TR 36.763 Study on NB-IoT/eMTC support for NTN