3GPP TSG-RAN Working Group 4 (Radio) Meeting #104-E	R4-2212458

# Electronic Meeting,  15th - 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.803 | CR | 0018 | rev | - | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.803: Addition of sub-array parameters for AAS BS relevant for the frequency range 24250 to 52600 MHz in subclause 5.2.3.2.4 and Annex C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_newRAT |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At last RAN4 meeting (RAN4#102-E) an extension to the antenna array model and corresponding model parameters relevant for the frequnecy range 1710 to 4990 MHz was included in TR 38.803. The antenna model extension was required to better reflect deployed AAS base stations operating within the freqeuncy range 1710 to 4990 MHz. This information have been communicated to ITU-R WP 5D in an LS (R4-2108080). Also for base stations operating within the frequency range 24250 to 52600 MHz (FR2-1) RAN4 have previously provided antenna parameters to ITU-R. Unfortunately, these parameters does not reflect the majority of deployed products in networks. With this CR additional parameter sets are included in technical report TR 38.803 as background information. The intention with the information is to better describe how FR2 base stations typically is implemented with respect to the antenna array configuration. It is clear that sub-arrays are not only used for the frequency range 1710 to 4990 MHz. Sub-array antenna geometrires are also used for other frequencies such as 24250 to 52600 MHz. Relevant and correct parameters are essential for sharing with other services (work typically done in ITU-R and other groups). Even though sharing activites is typically done outside RAN4 it is essential that RAN4 capture relevant parameters for typical BS implementations as background information in technical reports. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclause 5.2.3.2.4 model parameter sets for urban macro and dense urban deployments within the frequency range 24250 to 52600 MHz is added. - In annex C, technical background information related to how to mitigate spatial interference is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the technical background information in technical reports relevant for NR would not capture relevant information relevant for modelling AAS base stations correctly. Also, relevant TRs will not capture antenna modelling parameters shared with other external groups. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Subclause 5.2.3.2.4, Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Subclause 5.2.3.2.4

##### 5.2.3.2.4	Array antenna model extension

To model an AAS BS equipped with a sub-array antenna geometry an extended antenna model is required. A sub-array antenna geometry is created by combining vertical elements to sub-arrays as indicated in Figure 5.2.3.2.4-1. The antenna model extension was created to model AAS base station operating within the frequency range 1710 to 4990 MHz required for sharing studies in ITU-R.

Figure 5.2.3.2.4-1: Sub-array structure

In Table 5.2.3.2.4-1, the parameters used by the parameterized array antenna model supporting sub-array geometries are described.

Table 5.2.3.2.4-1: Extended parameter definitions

| Level | Parameter | Symbol | Unit |
| --- | --- | --- | --- |
| Element | Front to back ratio | Am | dB |
|  | Side lobe suppression | SLAv | dB |
|  | Horizontal half power beamwidth | j3dB | Degrees |
|  | Vertical half power beamwidth | q3dB | Degrees |
|  | Array element peak gain | GE,max | dBi |
| Sub-array | Number of element rows in sub-array | Msub | Integer |
|  | Vertical element separation | dv,sub | m |
|  | Electrical pre-set sub-array down-tilt angle | qsubtilt | Degrees |
| Array | Number of elements/sub-array rows | M | Integer |
|  | Number of elements columns | N | Integer |
|  | Horizontal element separation | dh | m |
|  | Vertical element/sub-array separation | dv | m |
|  | Electrical down-tilt angle | qetilt | Degrees |
|  | Electrical scan angle | jescan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 5.2.3.2.4-2.

Table 5.2.3.2.4-2: Extended AAS model

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ |

In Table 5.2.3.2.4-3, representable parameter sets relevant for an AAS base station operating within 1710 to 4990 MHz are provided.

Table 5.2.3.2.4-3: Antenna array parameters

| Parameter | Macro Rural | Macro suburban | Macro urban |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º |
| Antenna sub-array configuration (Row × Column) (Note 4) | 4 × 8 elements | 4 × 8 elements | 4 × 8 elements |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V |
| Number of element rows in sub-array | 3 | 3 | 3 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | 3 | 3 | 3 |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 28 | 28 | 28 |
| Base station horizontal coverage range (degrees) | +/-60 | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | 3 | 6 | 6 |
| Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180°. Note 2: The element gain includes the loss and is per polarization. Note 3: The conducted power per sub-array assumes 4x8x2 sub-arrays (i.e., power per H/V polarized element). Note 4: 4 × 8 means there are 4 vertical and 8 horizontal radiating sub-arrays. Note 5: For the case of 3 elements per sub array, dv will be 2.1 wavelengths. |  |  |  |

In Table 5.2.3.2.4-4, representable parameter sets relevant for an AAS base station operating within 24250 to 52600 MHz are provided.

Table 5.2.3.2.4-4: Antenna array parameters

| Parameter | Urban macro | Dense urban | Inddor |
| --- | --- | --- | --- |
| Element gain | 5.5 dBi | 5.5 dBi | 5.5 dBi |
| Horizontal/vertical 3 dB beam width of single element | 90 º for H 90 º for V | 90 º for H 90 º for V | 90 º for H 90 º for V |
| Horizontal/vertical front to back ratio | 30 dB for both H/V | 30 dB for both H/V | 30 dB for both H/V |
| Antenna polarization | Linear H/V | Linear H/V | Linear H/V |
| Antenna sub-array configuration (Row × Column) (Note 4) | 8 x 24 | 4 x 24 | 4 x 8 |
| Horizontal/Vertical radiating sub-array spacing (Note 5) | 0.5l for H, 1.2l for V | 0.5l for H, 1.2l for V | 0.5l for H, 1.2l for V |
| Number of element rows in sub-array | 2 | 2 | 2 |
| Vertical element separation in sub-array (dv,sub) | 0.6l | 0.6l | 0.6l |
| Pre-set sub-array down-tilt (Note 6) | 0 º | 0 º | 0 º |
| Array Ohmic loss (Note 2) | 2.0 dB | 2.0 dB | 2.0 dB |
| Conducted power (before Ohmic loss) per sub-array (Note 3) | 7 dBm | 7 dBm | 7 dBm |
| Base station horizontal coverage range | +/-60 º | +/-60 º | N/A (Note 8) |
| Base station vertical coverage range (Note 1) | 90-105 º | 90-105 º | N/A (Note 8) |
| Mechanical down-tilt (Note 7) | 15 º | 10 º | 90 º |
| Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180°. Note 2: The element gain includes the loss and is per polarization. Note 3: The conducted power per sub-array assumes 4x8x2 sub-arrays (i.e., power per H/V polarized element). Note 4: 8 × 24 means there are 8 vertical and 24 horizontal radiating sub-arrays. Note 5: For the case of 2 elements per sub array, dv will be 1.2 wavelengths. Note 6: Pre-set sub-array down-tilt its used to optimize coverage at ground. Note 7: The mechanical tilt depends on deployment scenario. Note 8: The base station is mounted in the ceiling. |  |  |  |

Annex C:

Annex C: Array antenna theory

In general, an array antenna design consists of several parts; radiating elements, a finite ground plane and mechanical structure components. The elements are typically placed in a lattice on the ground plane. The individual location of each element together with the radiation characteristics of each element determines the composite array antenna characteristics.

The element characteristics in terms of radiation properties (aka. element factor) depends on the area for which the array antenna is intended to provide coverage within.

In general, for a single polarized element, the individual embedded element pattern for an array antenna with N elements is described as:

, where n=1..N.

An array antenna consists of many radiating elements located close to each other under a weatherproof encapsulation. The locations of the n-th element in an antenna array can be described by; .

For the transceiver direction, each element in the array antenna is feed with a signal s(t) from a transmitter. The array antenna beam is determined by the weighting factor wn(t).

For a single polarized array antenna, the transmitted signal is created from super positioning in the far-field region. The composite field strength can be expressed as:

, where is the wave vector defined as:

The wave vector refers to a vector that describes the phase variation of a plane wave, in 3 orthogonal directions.

Since elements are located close to each other the radiation characteristics for individual elements will not be the same in the whole array. This phenomenon is referred to as mutual coupling. That means that the element factor R, cannot be seen independent of the element separation. The element separation also relates to radiation characteristics by means of spatial sampling resolution. Typically, for an array implementation where the element separation is larger than 0.5l, gating lobe performance will be affected.

Generally, the maximum steering without grating lobes can be expressed as:

, where d is the element separation and jmax is the maximal steering angle (along one dimension).

If the spatial sampling criterion is not fulfilled folding effect will occur creating a grating lobe response. Therefore, the element separation often is set close to 0.5l for system using large steering angles. However, when the elements are close the interaction between them is more severe, which results in ripple in radiation pattern per element level. Therefore, the element beam-width will be different. The element directivity is dependent on the element aperture which is of course affected by the element spacing, i.e. a 0.7l element cannot be spaced at 0.5l. Hence the interaction between element separation and element radiation characteristics is a delicate challenge to resolve. Typical element separation is in the range of 0.5l to 0.7l. The consequence of grating lobes is that energy will be spread in unintended direction, this may or may not be harmful for the system from an interference perspective. Nevertheless, the power in the intended direction drops because of large grating lobes. From a system design perspective, grating lobes, and the fact that the effective antenna area is reduced due to projection will reduce the directivity. This phenomenon is referred to as scan-loss. The scan-loss will impact EIRP in the intended direction. This effect needs considerations from a system perspective.

This means that the radiated power is a complex function of deployment parameters such as steering angle and design parameters such as antenna mutual coupling. A common phenomenon is referred to as scan-blindness, where EIRP drops unexpectedly due to interactions between coupling characteristics and excitation of the array. Note that EIRP can be affected by both scan-loss and scan-blindness at large steering angles, which means that the radiated power will drop considerably.

Another aspect is that radiation the individual radiation patterns or embedded radiation pattern will suffer from mutual coupling. The embedded pattern is distorted with a ripple, where characteristics such as beam pointing direction and beam-width may be impacted.

A simple and ideal, but still quite realistic, model for the element pattern should satisfy the earlier explained relation:

Written as power, or gain, patterns, the array antenna gain can be expressed as:

If the element distances in the array are small enough to not produce any grating lobes, and the array is a planar array, the maximum array gain for any scan angle should equate the gain of a planar aperture, i.e.

, where A is the total area of the antenna array and f is the angle off the normal direction.

Here we also neglect all losses related to reflections and mutual coupling. The factor cos(f) comes from the projection of the array area as seen in the direction of observation. Maximum array gain is achieved when all elements have the same amplitude, and are co-phased in the scan direction, i.e. the array factor.

This implies that the element factor should be:

, where dx and dy are the element distances in x and y direction. The 3 dB beamwidth thus becomes degrees and the maximum element gain for an array with 0.5l element spacing is p, or ~5 dBi.

In practice, however, the 3 dB beam width is lower than 120°, typically around 90° to 100°. This discrepancy to the theoretical 120° 3 dB beam width derived above is partly due to power loss from mutual coupling effects, which typically gets higher for larger scan angles. The element pattern beam width should also be related to the element gain, since element gain is only a measure on how focused the energy is in the far-field. An approximate formula for planar arrays [x], relating the gain and beam widths is

, where HPBW1 and HPBW1 are the 3dB beam widths in two orthogonal planes. Combining this way of calculating the maximum gain with the above expression relating the maximum element gain with the antenna area yields

[degrees] and  [degrees]
A practical element pattern model could be a Gaussian pattern with peak gain equal to

and beam widths related to the element distance as described above. This model would thus to some degree account for reflection and mutual coupling losses for large scan angles and losses due to grating lobes. It also scales the antenna element patterns gain and beam widths with the element spacing of the array in such a way that the superposition of all the elements in the array gives a total antenna gain equal to the array area.

## C.1	Interference mitigation techniques

In this section it will be illustrated that the impact of different mitigation techniques to suppress spatial interference caused by side-lobes.

Given an antenna array with M multiplied by N identical elements/sub-arrays, the radiation pattern of the array antenna can be described according to the pattern multiplication theorem as:

Rθ,φ=REθ,φRAθ,φ 	(Eq. C.1-1)

where RE is the radiation pattern for the array elements (for single element structures just one element or a sub-array for sub-array structures) and RA is the radiation pattern associated to the array factor. The element patten, RE(,, is based on a parameterized Gaussian shaped element or a sub-array constituted of multiple Gaussian shaped elements, with floors to model for side-lobe levels and front-to-back ratio. The element peak gain is directivity normalized, hence the peak element gain GE,max, element loss LE and half power beam widths 3dB and 3dB should be selected carefully to maintain correct antenna gain.

It can be noticed that both the element factor and the array factor can be used to shape the composite radiation pattern. The element pattern can be used to suppress side-lobe characteristics. For a limited steering range, a sub-array element can be used to suppress side-lobes better than a single element configuration. Typically for an AAS base station implementation the element radiation pattern and the array factor are customized to optimize the coverage within a specific coverage range for a given deployment scenario.

For a general array antenna, the array factor radiation pattern for transmitting array antenna with MN elements/subarrays per polarization can be expressed as:

RAθ,φ=n=1MNwnejkr2 	(Eq. C.1-2)

, where

-	wn is the complex array excitation,

-	k is the wave vector of the transmitted wave, and

-	r is the element location matrix.

From Eq. C.1-1 it can be noticed that both the element factor and the array factor can be used to shape the composite radiation pattern, which will be further described later. The element pattern can be used to suppress side-lobe characteristics. For a limited steering range, a sub-array element configuration can be used to suppress side-lobes better than a single element configuration. Typically for an AAS base station implementation the element radiation pattern and the array factor are customized to optimize the coverage within a specific coverage range and coexistence with other services for a given deployment scenario.

For an array antenna with single element structure and element separation of 0.5l or less the first side-lobe is the strongest. For uniform amplitude excitation the first side lobe power level is approximately 13 dB below the main beam power level. For array antenna with element separation larger than 0.5l folding effects will create grating lobes in the side lobe region when the main beam is steered. A common scenario with element separation larger than 0.5l is when sub-array is used in the vertical domain. The angular location of the grating lobe is determined by the element separation and steering angle and the power level of the grating lobes is determined by the element factor.

As an example, in Figure C.1-1, the impact of element separation is visualised for an 8x8 element Uniform Rectangular Array (URA) with single element structure where the vertical beam steering directions is set to 130 degrees.

Figure C.1-1: Impact of element separation on vertical radiation pattern

The array factor described by Eq. C.1-2 will produce maximum directivity for the wanted carrier for which the system is calibrated minimizing the excitation error. For unwanted emission outside the carrier the directivity will roll-off gradually to a point where the array factor directivity is lost.

The relation between array excitation correlation and directivity can be modelled as described in TR 37.840, subclause 5.4.4.1.4. The average radiation pattern for different correlation values is plotted in Figure C.1-2. The maximum directivity is achieved for correlation equal to 1 and the minimum directivity equal to the element directivity is achieved for correlation equal to 0.

Figure C.1-2: Relation between excitation correlation and directivity

To be able to conduct accurate coexistence analysis the unwanted emission directivity roll-off characteristics is required. The decorrelation effect in an array antenna will together with traditional filters provide suppression of unwanted emission EIRP levels required to guarantee coexistence with other adjacent services. The directivty roll-off charactersitcs depends on out-of-band array correlation which may depend on the carrier configuration.

There is no single general solution available to mitigate all possible interference situations. Depending on situation one or more mitigation techniques described in following section can be used to guarantee coexistence between AAS base stations and other services operating in adjacent spectrum.

## C.1.1	Electrical and mechanical down-tilt

To avoid high-EIRP level to be transmitted above the horizon electrical and/or mechanical down-tilt will be used. There is a conceptual difference between mechanical and electrical down-tilt as illustrated in Figure C.1.1-1.

| a) without tilt | b) electrical tilt | c) mechanical tilt |
| --- | --- | --- |

Figure C.1.1-1: Illustration of different kind of tilt options

It can be noticed that mechanical down-tilt can be used to reduce the vertical steering angle, which minimize the impact due to side lobes excited by large steering angles. In Figure C.1.1-2, vertical radiation pattern visualises the impact of mechanical and electrical tilt, assuming antenna parameter values in Table 5.2.3.2.4-4.

| a) Electrical tilt | b) Electrical tilt combined with mechanical tilt of 6 degrees |
| --- | --- |

Figure C.1.1-2: Normalized EIRP pattern after being tilted electrically or mechanically

## C.1.2	Individual beam power control

Selective reduction of power for specific beams will ensure an equivalent drop in EIRP for both the main lobes and the grating lobes. This achieves a reduction in grating lobes without loss of performance. This is illustrated in Figure C.1.2-1. In this case, the EIRP of the lowest pointing beams are reduced by increasing values (From 0.5 to 6 dB).  Note that also the grating lobes are reduced by the same number of dB. Figure C.1.2-1 further shows the combined effect of mechanical tilt and individual beam power control.

Figure C.1.2-1: Normalized array EIRP patterns when using individual beam power control and the combination of individual beam power control and mechanical tilt.

## C.1.3	Beam widening

Another mitigation technique is to reduce the directivity of those lobes that generate grating lobes, thereby reducing the EIRP. This is achieved through widening of those beams; the concept is shown in Figure C.1.3-1.

Figure C.1.3-1: Normalized array EIRP patterns when using beam widening

## C.1.4	Power back-off

The previous mitigation techniques can be further supplemented by additional power back-off schemes for reduction of side lobes and grating lobes for out-of-band and spurious domain interference scenarios. These schemes would apply to only those carrier combinations that contribute most to the disturbance out-of-band/spurious and are within acceptable limits for product performance.

Figure C.1.4-1: Carrier power allocation

Figure C.1.4-1 provides an example of this. Carrier C3, C4 and C5 is configured with equal power, while for carrier C1 and C2 the power is backed off to suppress emission in directions related to grating lobes at frequencies allocated to adjacent services.

## C.1.5	Pre-set sub-array down-tilt

Sub-array tilt is decided already in the BS design and will shift the peak location as well as alter the sidelobes of the subarray, and thus also alter the positions of the grating lobes for the array, see Figure C.1.5-1 below. The first plot shows the antenna patterns for different electrical tilts for 0 degrees sub-array tilt, and the second for 10 degrees sub-array down-tilt.

Figure C.1.5-1: Carrier power allocation

## C.1.6	Amplitude tapering

AAS offer a wide range of opportunities on optimizing the directivity patterns through amplitude and phase control. High directivity antenna array also has side lobes which are often undesirable since they may cause intra-cell or inter-cell interference. Side lobe levels can be reduced via tailoring the amplitude across the antenna array which is often referred as amplitude weighting technology. Whilst amplitude weighting/tapering reduces the side lobes it also makes the main lobe wider and hence reduces gain. It needs trade-off between antenna gain and side lobe suppression. Amplitude tapering does not impact the grating lobes.

Applying tapering electrically for the complete array will results in a main beam peak EIRP drop due to the amplitude window itself. In addition, the directivity will also reduce as an effect of a wider beam as the effective array antenna aperture size decrease.

For sub-array structures larger than 2 element sub-arrays tapering within the sub-array can be used. If tapering is applied at sub-array level the power is redistributed between elements in the sub-array, hence the impact additional power loss can be minimized.