TSG-RAN Working Group 4 (Radio) meeting #104-E	R4-2212462

Electronic Meeting, 15th – 26th August 2022

Source:	Ericsson

Title:	On receiver test requirement level derivation for FR2-2

Agenda item:	9.14.5.3

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#103-E) the work to extend the conformance test specification TR 38.141-2 up to 71 GHz started. A way-forward with guidance for this meeting was agreed in [1].

Traditionally, when operating bands at higher frequencies is added in specifications test system uncertainties are widened due to performance of test equipment and capabilities of test procedures. For FR2 the measurement uncertainty for receiver requirements is now above +/- 5 dB for frequencies below FR2-2. For systems complying to 3GPP specifications, conformance testing is an essential part to guarantee function and performance of cellular networks. When the test requirements for FR2-2 is developed, careful considerations are required when acceptable test system measurement uncertainty values are defined. For FR2-2 the uplink will be limited by the UE power capability, which means that the uplink will limit the coverage. Hence, BS receiver requirement needs to be tested with good accuracy. There are some additions in term of additional intermediate calibration steps to be introduced and additional test equipment in the test set up which would reduce the test system measurement uncertainty for FR2-2. Details are discussed in a companion contribution [2].

In this contribution an overview of the test system measurement uncertainty for FR2-1 is presented together with a proposal on how to proceed the work with respect to base station receiver conformance testing requirements in TS 38.141-2 for FR2-2. A CR to TS 38.141-2 with essential changes to support FR2-2 is provided in a companion contribution [3]. Unresolved issues are marked TBD or FFS. The intention with the CR is to set the structural additions to the specification.

# Discussion

In Table 2-1, the test system uncertainty values relevant for FR2-1 is summarized together with proposals for test system uncertainty values relevant for FR2-2. Test system uncertainty values are applicable for normal condition unless otherwise stated.

Table 2-1: Receiver test system uncertainty overview

| Subclause | Maximum OTA Test System uncertainty | Action |
| --- | --- | --- |
| 7.3 OTA reference sensitivity level | ±2.4 dB, 24.25 to 29.5 GHz ±2.4 dB, 37.0 to 43.5 GHz +[3.5] dB, 43.5 to 48.2 GHz ±TBD dB, 57.0 to 71.0 GHz | Value for FR2-2 needs to be determined. |
| 7.5.1 OTA adjacent channel selectivity | ±3.4 dB, 24.25 to 29.5 GHz ±3.4 dB, 37.0 to 43.5 GHz +[5.1] dB, 43.5 to 48.2 GHz ±TBD dB, 57.0 to 71.0 GHz | Value for FR2-2 needs to be determined. |
| 7.5.2 In-band blocking (General) | ±3.4 dB, 24.25 to 29.5 GHz ±3.4 dB, 37 to 43.5 GHz +[5.1] dB, 43.5 to 48.2 GHz ±TBD dB, 57.0 to 71.0 GHz | Value for FR2-2 needs to be determined. For FR2 the same value as for OTA adjacent channel selectivity have been used. |
| 7.6 OTA out-of-band blocking | ±3.6 dB, 24.25 to 29.5 GHz ±3.6 dB, 37.0 to 43.5 GHz +[4.5] dB, 43.5 to 48.2 GHz ±TBD dB, 57.0 to 71.0 GHz | Value for FR2-2 needs to be determined. |
| 7.7 OTA receiver spurious emissions | ±2.5 dB, 0.03 to 6 GHz ±2.7 dB, 6.0 to 40.0 GHz ±5.0 dB, 40.0 to 60.0 GHz ±TBD dB, 60.0 to 142.0 GHz | Value for the frequency range from 60 GHz to upper frequency limit needs to be determined. |
| 7.8 OTA receiver intermodulation | ±3.9 dB, 24.25 to 29.5 GHz ±3.9 dB, 37.0 to 43.5 GHz +[5.4] dB, 43.5 to 48.2 GHz ±TBD dB, 57.0 to 71.0 GHz | Value for FR2-2 needs to be determined. |
| 7.9 OTA in-channel selectivity | ±3.4 dB, 24.25 to 29.5 GHz ±3.4 dB, 37 to 43.5 GHz +[5.1] dB, 43.5 to 48.2 GHz ±TBD dB, 57.0 to 71.0 GHz | Value for FR2-2 needs to be determined. For FR2 the same value as for OTA adjacent channel selectivity have been used. |

It can be noticed that OTA test system uncertainty currently defined for FR2-1 receiver requirements is larger than what is set for FR2-1 transmitter requirements. It can also be noticed that the test system uncertainty for the frequency range 43.5 to 48.2 GHz is significantly larger than for the frequency range 37.0 to 43.5 GHz. The values applicable for the frequency range 43.5 to 48.2 GHz is still within [], which indicates that the discussion is not settled.

Observations:

- The receiver test system uncertainty is larger than transmitter test system uncertainty.

- The test system uncertainty for the frequency range 43.5 to 48.2 GHz is significantly larger than for lower parts of FR2.

A reason for this may be that the signal generator capability to generate RF power maintaining acceptable spectral performance is a limiting factor at high frequencies. For up-link requirements testing the base station receiver performance typically multiple signal generators are required to produce a wanted signal and an interferer signal simultaneously.

For FR2-2 test procedures and test equipment need to be adopted to keep the acceptable test system uncertainty values within a range so that conformance test requirements are relevant. E.g., having test system uncertainties larger than +/- 6 dB would have significant impact cell coverage and allow for a very large deviation from the minimum requirement limit in RF core specification.

Instead of increasing the test system uncertainty as function of frequency it is now really important to find new test methods and test procedures which allows for test system measurement uncertainties well below +/- 6 dB.

Potential candidates are to introduce mixers in the test setups, allowing for re-use of traditional test equipment with lower measurement uncertainty.

The signal generator absolute measurement uncertainty can be calibrated towards a power meter with much better absolute power measurement uncertainty. This is a new intermediate calibration procedure to be included to current calibration procedure.

Proposal: For receiver requirement test system measurement uncertainty, extend measurement procedures to break trend where measurement uncertainty grows rapidly as function of frequency.

# Conclusion

In this contribution we presented an overview of test system measurement uncertainties for receiver requirements applicable for FR2-1. Some observations and trends were identified. For FR2-2 it is not reasonable to just let the test system measurement uncertainty to grow as before. Instead, new additions to current test procedures and test methods must be considered.

Proposal: For receiver test system measurement uncertainty, extend measurement procedures to enable for test setups to break the trend where measurement uncertainty grows rapidly as function of frequency.

# References

[1]	R4-2210638, “WF on BS RF conformance testing for FR2-2”, ZTE

[2] 	R4-2212465, “On general aspects relevant for conformance testing up to 71 GHz”, Ericsson

[3]	R4-2212464, “Draft CR to 38.141-2: Addition of FR2-2 receiver support in clause 7”, Ericsson