3GPP TSG-RAN4 Meeting # 104-e	R4-2212506

Electronic meeting, August 15 – August 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | - | rev | - | Current version: | 15.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.141-1: Additional BS conformance to other standards |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the final draft of EN 301 908-24, additional BS conformance to other standards was defined. The conformance with the requirements in ETSI EN 301 908-24 can also equally be demonstrated through MSR standards such as ETSI EN 301 908-18 for BS type 1-C, ETSI EN 301 908-23 for BS type 1-H and 1-O. But in current 38.141-1 and 38.141-2 such equal demonstration is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new sub-clause to include the additional BS conformance. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The additional equal conformance is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.104: "NR Base Station (BS) radio transmission and reception".

[3]	3GPP TS 38.141-2: "NR, Base Station (BS) conformance testing, Part 2: Radiated conformance testing".

[4]	ITU-R Recommendation M.1545, "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[5]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[6]	IEC 60 721-3-3: "Classification of environmental conditions - Part 3-3: Classification of groups of environmental parameters and their severities - Stationary use at weather protected locations".

[7]	IEC 60 721-3-4: "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Section 4: Stationary use at non-weather protected locations".

[8]	IEC 60 721: "Classification of environmental conditions".

[9]	IEC 60 068-2-1 (2007): "Environmental testing - Part 2: Tests. Tests A: Cold".

[10]	IEC 60 068-2-2: (2007): "Environmental testing - Part 2: Tests. Tests B: Dry heat".

[11]	IEC 60 068-2-6: (2007): "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)".

[12]	ITU-R Recommendation SM.328: "Spectra and bandwidth of emissions".

[13]	Federal Communications Commission: "Title 47 of the Code of Federal Regulations (CFR)".

[14]	ECC/DEC/(17)06: "The harmonised use of the frequency bands 1427-1452 MHz and 1492-1518 MHz for Mobile/Fixed Communications Networks Supplemental Downlink (MFCN SDL)".

[15]	3GPP TR 25.942: "RF system scenarios".

[16]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[17]	3GPP TS 38.211: "NR; Physical channels and modulation".

[18]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[19]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[20]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

[21]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[22]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[23]	ITU-T Recommendation O.150, "Equipment for the measurement of digital and analogue/digital parameters".

[24]	3GPP TS 37.141: "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing".

[25]	3GPP TS 37.145-1: "Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: Conducted conformance testing".

<End of Change 1>

<Start of Change 2>

4.8	Applicability of requirements

4.8.1	General

4.8.2	Requirement set applicability

In table 4.8.2-1, the requirement applicability for each requirement set is defined. For each requirement, the applicable requirement clause in the specification is identified.

Table 4.8.2-1: Requirement set applicability

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | 1-C | 1-H |
| BS output power | 6.2 | 6.2 |
| Output power dynamics | 6.3 | 6.3 |
| Transmit ON/OFF power | 6.4 | 6.4 |
| Transmitted signal quality | 6.5 | 6.5 |
| Occupied bandwidth | 6.6.2 | 6.6.2 |
| ACLR | 6.6.3.5.3 | 6.6.3.5.4 |
| Operating band unwanted emissions | 6.6.4.5.7 | 6.6.4.5.8 |
| Transmitter spurious emissions | 6.6.5.5.3 | 6.6.5.5.4 |
| Transmitter intermodulation | 6.7.5.1 | 6.7.5.2 |
| Reference sensitivity level | 7.2 | 7.2 |
| Dynamic range | 7.3 | 7.3 |
| In-band selectivity and blocking | 7.4 | 7.4 |
| Out-of-band blocking | 7.5 | 7.5 |
| Receiver spurious emissions | 7.6.5.2 | 7.6.5.3 |
| Receiver intermodulation | 7.7 | 7.7 |
| In-channel selectivity | 7.8 | 7.8 |
| Performance requirements | 8 | 8 |

4.8.3	Applicability of test configurations for single-band operation

The applicable test configurations are specified in the tables below for each the supported RF configuration, which shall be declared according to clause 4.6. The generation and power allocation for each test configuration is defined in clause 4.7. This clause contains the test configurations for a BS capable of single carrier, multi-carrier and/or CA operation in both contiguous and non-contiguous spectrum in single band.

For a BS declared to be capable of single carrier operation only (D.16), a single carrier (SC) shall be used for testing.

For a BS declared to support multi-carrier and/or CA operation in contiguous spectrum within a single band (D.15-D.16), the test configurations in the second column of table 4.8.3-1 shall be used for testing.

For a BS declared to support multi-carrier and/or CA operation in contiguous and non-contiguous spectrum within a single band (D.15-D.16) and where the parameters in the manufacture's declaration according to clause 4.6 are identical for contiguous (C) and non-contiguous (NC) spectrum operation (D.9), the test configurations in the third column of table 4.8.3-1 shall be used for testing.

For a BS declared to support multi-carrier and/or CA in operation contiguous and non-contiguous spectrum within a single band (D.15-D.16) and where the parameters in the manufacture's declaration according to clause 4.6 are not identical for contiguous (C) and non-contiguous (NC) spectrum operation (D.9), the test configurations in the fourth column of table 4.8.3-1 shall be used for testing.

Unless otherwise stated, single carrier configuration (SC) tests shall be performed using signal with narrowest supported channel bandwidth and the smallest supported sub-carrier spacing.

Table 4.8.3-1: Test configurations for a BS capable of multi-carrier and/or CA in a single band

| BS test case | Contiguous spectrum capable BS | C and NC capable BS with identical parameters | C and NC capable BS with different parameters |
| --- | --- | --- | --- |
| Base station output power | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| RE Power control dynamic range | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Total power dynamic range | SC | SC | SC |
| Transmit ON/OFF power (only applied for NR TDD BS) | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| Frequency error | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Error Vector Magnitude | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| Time alignment error | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| Occupied bandwidth | SC, NRTC2 (NOTE 1) | SC, NRTC2 (NOTE 1) | SC, NRTC2 (NOTE 1) |
| Adjacent Channel Leakage power Ratio (ACLR) | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| Cumulative ACLR requirement in non-contiguous spectrum | - | NRTC3 | NRTC3 |
| Operating band unwanted emissions | NRTC1, SC (NOTE 2) | NRTC1, NRTC3, SC (NOTE 2) | NRTC1, NRTC3, SC (NOTE 2) |
| Transmitter spurious emissions | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| Transmitter intermodulation | NRTC1 | NRTC1, NRTC3 | NRTC1, NRTC3 |
| Reference sensitivity level | SC | SC | SC |
| Dynamic range | SC | SC | SC |
| Adjacent Channel Selectivity (ACS) | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| In-band blocking | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| Out-of-band blocking | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| Receiver spurious emissions | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| Receiver intermodulation | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| In-channel selectivity | SC | SC | SC |
| NOTE 1: NRTC2 is only applicable when contiguous CA is supported. NOTE 2: OBUE SC shall be tested using the widest supported channel bandwidth and the highest supported sub-carrier spacing. |  |  |  |

4.8.4	Applicability of test configurations for multi-band operation

For a BS declared to be capable of multi-band operation, the test configuration in table 4.8.4-1 and/or table 4.8.3-1 shall be used for testing. In the case where multiple bands are mapped on common multi-band connector, the test configuration in the second column of table 4.8.4-1 shall be used. In the case where multiple bands are mapped on common single-band connector, the test configuration in table 4.8.3-1 shall be used. In the case where multiple bands are mapped on separate single-band connector or multi-band connector, the test configuration in the third column of table 4.8.4-1 shall be used.

Unless otherwise stated, single carrier configuration (SC) tests shall be performed using signal with narrowest supported channel bandwidth and the smallest supported sub-carrier spacing.

Table 4.8.4-1: Test configuration for a BS capable of multi-band operation

| BS test case | Test configuration |  |
| --- | --- | --- |
|  | Common connector | Separate connectors |
| Base station output power | NRTC1/3 (NOTE 1), NRTC4 | NRTC1/3 (NOTE 1), NRTC4 |
| RE Power control dynamic range | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Total power dynamic range | SC | SC |
| Transmit ON/OFF power (only applied for NR TDD BS) | NRTC4 | NRTC4 |
| Frequency error | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Error Vector Magnitude | NRTC1/3 (NOTE 1), NRTC4 | NRTC1/3 (NOTE 1), NRTC4 |
| Time alignment error | NRTC1/3 (NOTE 1), NRTC5 (NOTE 2) | NRTC1/3 (NOTE 1), NRTC5 (NOTE 2) |
| Occupied bandwidth | SC, NRTC2 (NOTE 3) | SC, NRTC2 (NOTE 3) |
| Adjacent Channel Leakage power Ratio (ACLR) | NRTC1/3 (NOTE 1), NRTC5 (NOTE 4) | NRTC1/3 (NOTE 1, 5), NRTC5 (NOTE 4, 5) |
| Cumulative ACLR requirement in non-contiguous spectrum | NRTC3 (NOTE 1), NRTC5 (NOTE 4) | NRTC3 (NOTE 1, 5) |
| Operating band unwanted emissions | NRTC1/3 (NOTE 1), NRTC5, SC (NOTE 7) | NRTC1/3 (NOTE 1, 5), NRTC5 (NOTE 5), SC(NOTE 7) |
| Transmitter spurious emissions | NRTC1/3 (NOTE 1), NRTC5 | NRTC1/3 (NOTE 1, 5), NRTC5 (NOTE 5) |
| Transmitter intermodulation | NRTC1/3 (NOTE 1) | NRTC1/3 (NOTE 1, 5) |
| Reference sensitivity level | SC | SC |
| Dynamic range | SC | SC |
| Adjacent Channel Selectivity(ACS) | NRTC5 | NRTC1/3 (NOTE 1), NRTC5 (NOTE 6) |
| In-band blocking | NRTC5 | NRTC1/3 (NOTE 1), NRTC5 (NOTE 6) |
| Out-of-band blocking | NRTC5 | NRTC1/3 (NOTE 1), NRTC5 (NOTE 6) |
| Receiver spurious emissions | NRTC1/3 (NOTE 1), NRTC5 | NRTC1/3 (NOTE 1, 5), NRTC5 (NOTE 5) |
| Receiver intermodulation | NRTC5 | NRTC1/3 (NOTE 1), NRTC5 (NOTE 6) |
| In-channel selectivity | SC | SC |
| NOTE 1: NRTC1 and/or NRTC3 shall be applied in each supported operating band. NOTE 2: NRTC5 is only applicable when inter-band CA is supported. NOTE 3: NRTC2 is only applicable when contiguous CA is supported. NOTE 4: NRTC5 may be applied for Inter RF Bandwidth gap only. NOTE 5: For single-band operation test, other antenna connector(s) is (are) terminated. NOTE 6: NRTC5 is only applicable for multi-band receiver. NOTE 7: OBUE SC shall be tested using the widest supported channel bandwidth and the highest supported sub-carrier spacing. |  |  |

4.8.5	Additional conformance

For BS type 1-C that is NR (single-RAT) capable only, the requirements in the present document are applicable and additional conformance to TS 37.141 [24] is optional. For a BS additionally conforming to TS 37.141 [24], conformance to some of the RF requirements in the present document can be demonstrated through the corresponding requirements in TS 37.141 [24] as listed in Table 4.8.5-1.

Table 4.8.5-1: Alternative RF test requirements for a BS additionally conforming to TS 37.141 [24]

| RF requirement | Clause in the present document | Alternative clause in TS 37.141 [24] |
| --- | --- | --- |
| Base station output power | 6.2.5 | 6.2.1.5 |
| Transmit ON/OFF power | 6.4 | 6.4 |
| Unwanted emissions |  |  |
| Transmitter spurious emissions | 6.6.5.5(except for 6.6.5.5.4) | 6.6.1.5 (except for 6.6.1.5.3) |
| Operating band unwanted emissions | 6.6.4.5(except for 6.6.4.5.8) | 6.6.2.5 (except for 6.6.2.5.3 and 6.6.2.5.4) |
| Transmitter intermodulation | 6.7.5.1 | 6.7.5.1 |
| Narrowband blocking | 7.4.2.5 | 7.4.5.2 |
| Blocking | 7.4.2.5 | 7.4.5.1 |
| Out-of-band blocking | 7.5.5.1 | 7.5.5.1 |
| Co-location with other base stations | 7.5.5.2 | 7.5.5.2 |
| Receiver spurious emissions | 7.6.5(except for 7.6.5.3) | 7.6.5.1 |
| Intermodulation | 7.7.5 | 7.7.5.1 |
| Narrowband intermodulation | 7.7.5 | 7.7.5.2 |

For BS type 1-H that is NR (single-RAT) capable only, the requirements in the present document are applicable and additional conformance to TS 37.145-1 [25] is optional. For a BS additionally conforming to TS 37.145-1 [25], conformance to some of the RF requirements in the present document can be demonstrated through the corresponding requirements in TS 37.145-1 [25] as listed in Table 4.8.5-2.

Table 4.8.5-2: Alternative RF test requirements for a BS additionally conforming to TS 37.145-1 [25]

| RF requirement | Clause in the present document | Alternative clause in TS 37.145-1 [25] |
| --- | --- | --- |
| Base station output power | 6.2.5 | 6.2.2.5 |
| Transmit ON/OFF power | 6.4 | 6.4 |
| Unwanted emissions |  |  |
| Transmitter spurious emissions | 6.6.5.5(except for 6.6.5.5.3) | 6.6.6.5 |
| Operating band unwanted emissions | 6.6.4.5(except for 6.6.4.5.7) | 6.6.5.5 (except for 6.6.5.5.5) |
| Transmitter intermodulation | 6.7.5.2 | 6.7.5.1 |
| Narrowband blocking | 7.4.2.5 | 7.4.5.1.2 |
| Blocking | 7.4.2.5 | 7.4.5.1.1 |
| Out-of-band blocking | 7.5.5.1 | 7.5.5.1.1 |
| Co-location with other base stations | 7.5.5.2 | 7.5.5.1.2 |
| Receiver spurious emissions | 7.6.5(except for 7.6.5.2) | 7.6.5.2.1 |
| Intermodulation | 7.7.5 | 7.7.5.1.1 |
| Narrowband intermodulation | 7.7.5 | 7.7.5.1.2 |

<End of Change 2>