3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212528

Electronic Meeting, August 15 – August 26, 2022

Agenda Item: 9.17.2.2

Source: MediaTek Inc.

Title: Discussion on LS on SSB measurement for L1-RSRP on inter-cell beam management

Document for: Discussion

- Introduction

In the last meeting, one LS R1-2205640 regarding SSB based L1-RSRP measurement for inter-cell BM is set from RAN1 to RAN4. Our view on this LS is provided in this paper.

- Discussion

The content of LS is provided below for reference.

Overall Description: In Rel-17 FeMIMO, for inter-cell beam management, RAN1 has agreed to support PDCCH/PDSCH rate matching around the SSBs indicated by ssb-PositionsInBurst-r17 for the same PCI as that associated with TCI state of the PDSCH/PDCCH as working assumption. Working assumption On inter-cell beam management, the PDCCH /PDSCH should be rate matched around the SSBs indicated by ssb-PositionsInBurst-r17 for the same PCI as that associated with TCI state of the PDSCH /PDCCH - Send LS to RAN4 on whether there is requirements in RAN4 that assumes UE to measure SSB for L1-RSRP measurement and receiving PDSCH /PDCCH on the same RE in FR1. Revisit this issue after there is RAN4 feedback. RAN1 would kindly like to ask RAN4 to provide feedback on whether there are any requirements that are related to UE measurements of L1-RSRP and reception of PDSCH/PDCCH on the same RE in FR1 for inter-cell beam management. RAN1 would revisit this issue when there is RAN4 feedback.

For the question from RAN1 “whether there is requirements in RAN4 that assumes UE to measure SSB for L1-RSRP measurement and receiving PDSCH /PDCCH on the same RE in FR1”. To our understanding, there is no such requirement in TS 38.133. As far as we know, in RRM and Demod session, we assume SSB and PDSCH shall not be transmitted in the same RE. Because SSB is an important signals and it can be used for channel estimation, timing tracking and AGC tunning, … , etc. So, if PDSCH/PDCCH is transmitted in the same RE as SSB, the performance will have hugely impact.

Proposal 1: No UE requirement is applied when UE measures SSB for L1-RSRP measurement and receives PDSCH /PDCCH on the same RE in FR1.

- Summary

In this paper, the discussion of this LS is provided. We have the following proposal:

Proposal 1: No UE requirement is applied when UE measures SSB for L1-RSRP measurement and receives PDSCH /PDCCH on the same RE in FR1.

- Reference

- R1-2205640, LS to RAN4 on SSB measurement for L1-RSRP on inter-cell beam management, RAN1, RAN1 #109-e