3GPP TSG-RAN WG4 Meeting#104-e	R4-2212544

Online, 15th Aug  – 26th Aug, 2022

Source: 	Qualcomm Incorporated

Title: 	Views on m-TRP CSI Performance Requirements

Agenda item:	9.17.4.2.1

Document for:	Discussion

- Introduction

It was agreed to introduce PMI reporting requirement for multi-TRP CSI, as captured in the WF from RAN4#103-e [1]. Furthermore, RAN4 also agreed on several simulation assumptions for the multi-TRP PMI reporting test setup. In this paper, we provide our views on the remaining issues for the PMI reporting requirement.

- Requirements for m-TRP CSI Tests

Issue 1-2-1: Test setup for CSI reporting enhancement

It was agreed to assume cross-polarized (XP) high correlation for the MIMO transmission in the m-TRP PMI reporting test setup. However, there were no discussions about the beam-steering model in the context of m-TRP transmission. Due to the assumption of high correlation for MIMO transmission as well as low Doppler for channel (i.e., TDLA 10Hz), the beam selection may often be stuck and may not be representative of the practical deployment scenarios. Furthermore, existing single TRP PMI reporting tests have beam steering so that it could be tested whether UE is able to properly report PMI as the channel direction changes over time. Without considering beam-steering model into the m-TRP test setup, we will not be able to test whether UE can adapt to changing channel directions, hence the performance requirement would be incomplete.

Agreement:

- Only PMI requirement introduced for CSI reporting enhancement for m-TRP transmission with Single-DCI based scheme.

Agreement:

- MIMO correlation: XP high

- 8 ports per TRP as baseline

- Test Metric: TP ratio between following PMI and Random PMI

- Test point: 90%

- Encourage interesting companies to provide simulation results for both 4 and 8 ports in RAN4#104-e meeting, and make the final decision in RAN4#104-e meeting

Proposal 1: Consider beam-steering model into the multi-TRP PMI reporting test set-up.

- Conclusions

This paper provides our views on the remaining open issues in defining the requirements for the m-TRP CSI tests. The following has been proposed.

Proposal 1: Consider beam-steering model into the multi-TRP PMI reporting test set-up.

# References

[1] R4-2210669, “WF on CSI requirement for Rel-17 FeMIMO,” 3GPP TSG RAN WG4 Meeting #103-e, Online, May 9 – May 20, 2022.