3GPP TSG-RAN WG4 Meeting#104-e	R4-2212546

Online, 15th Aug – 26th Aug, 2022

Source: 	Qualcomm Incorporated

Title: 	Views on Performance Requirements for Further Enhanced TypeII Port Selection Codebook

Agenda item:	9.17.4.2.2

Document for:	Discussion

- Introduction

The Further Enhanced Type II port selection codebook (FeTypeII PS CB) is designed to exploit partial UL/DL reciprocity in FDD scenarios, where the BS can beamform on the CSI-RS signals based on measurements of the UL SRS signals. According to the WF from RAN4#103-e (as copied below), it is still unclear how to design a feasible and practical test case for FeTypeII PS CB. In this paper, we provide our views whether RAN4 should introduce such a requirement.

- Requirements for FeTypeII PS CB

Issue 2-1: Whether to define PMI requirement for Rel-17 FeTye II PS codebook

Agreement:

- Further discuss test case design especially for BF modelling in BS side, RAN4 will not introduce requirements for Rel-17 FeType II PS codebook if RAN4 can’t identify proper test case set-up by end of Aug RAN4 meeting.

The FeTypeII PS CB requires BS beamforming on the CSI-RS signal based on the estimation of SRS signal. However, the implementation of the BS beamforming is not standardized for FeTypeII PS CB. Any restriction on the BS beamforming will not guarantee optimal performance for UE. Hence, the UE performance cannot be guaranteed with such a CB. Therefore, introducing such a requirement will become more of a functional test. Furthermore, absence of a baseline performance from earlier releases, e.g., performance from Rel-15 PS CB, we think that the performance requirement for FeTypeII PS CB should not be defined in Rel-17.

Proposal 1: Do not define performance requirement for Rel-17 FeTye II PS codebook

- Conclusions

This paper provides our views on the open issue of whether performance requirement should be introduced for FeTypeII PS CB. The following has been proposed:

Proposal 1: Do not define performance requirement for Rel-17 FeTye II PS codebook

# 4. References

[1] R4-2210669, “WF on CSI requirement for Rel-17 FeMIMO,” 3GPP TSG RAN WG4 Meeting #103-e, Online, May 9 – May 20, 2022.