3GPP TSG-RAN WG4 Meeting#104-e	R4-2212569

Online, 15th Aug – 26th Aug, 2022

Source: 	Qualcomm Incorporated

Title: 	Simulation Results on NTN UE PDSCH Requirements

Agenda item:	9.11.7.3.1

Document for:	Discussion

- Introduction

For UE PDSCH requirements, most of the simulation assumptions were finalized during RAN4#103-e [1]. In this paper, we share our simulation results for defining PDSCH demodulation requirements.

- Requirements for NTN UE PDSCH Tests

Based on the agreed simulation assumptions, we choose alignment SNR requirements at 70% peak throughput across several MCS with various Doppler assumptions due to UE mobility. As agreed during RAN4#103-e, we assume that Doppler shift due to satellite mobility is fully compensated by UE. Below we share our simulation results for both NTN TDLA and NTN TDLC channels.

Table 1: SNR requirements for NTN TDL-A channel

| MCS/Doppler | 100Hz | 200Hz | 300Hz |
| --- | --- | --- | --- |
| MCS13 | 5.50 | 5.76 | 5.89 |
| MCS10 | 2.87 | 3.11 | 3.22 |
| MCS4 | -2.66 | -2.52 | -2.57 |

Table 2: SNR requirements for NTN TDL-C channel with AoA of 50 degrees for the LOS path

| MCS/Doppler | 100Hz | 200Hz | 300Hz |
| --- | --- | --- | --- |
| MCS13 | 4.23 | 4.3 | 4.58 |
| MCS10 | 1.76 | 1.80 | 2.01 |
| MCS4 | -3.46 | -3.53 | -3.37 |

Table 3: SNR requirements for NTN TDL-C channel with AoA of 60 degrees for the LOS path

| MCS/Doppler | 100Hz | 200Hz | 300Hz |
| --- | --- | --- | --- |
| MCS13 | 4.34 | 4.28 | 4.56 |
| MCS10 | 1.82 | 1.77 | 2.02 |
| MCS4 | -3.36 | -3.51 | -3.31 |

| MCS/Doppler | 100Hz | 200Hz | 300Hz |
| --- | --- | --- | --- |
| MCS13 | 4.33 | 4.4 | 4.49 |
| MCS10 | 1.83 | 1.85 | 1.95 |
| MCS4 | -3.39 | -3.40 | -3.36 |

Table 4: SNR requirements for NTN TDL-C channel with AoA of 70 degrees for the LOS path

- Conclusions

In this paper, we share our simulation results for PDSCH requirements based on the agreed simulation assumptions.

# References

[1] R4-2210661, “WF on NTN demodulation requirements – general and PDSCH,” 3GPP TSG RAN WG4 Meeting #103-e, Online, May 09 – May 20, 2022.