3GPP TSG-RAN WG4 Meeting #104-e	R4-2212572

Electronic Meeting, August 15 – August 26, 2022

Agenda Item:	9.14.8.2.2

Source:	Ericsson

Title:	On the PDCCH and PBCH requirements in 52.6 – 71 GHz band

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on PDCCH and PBCH demodulation requirements for FR2-2. Furthermore, in RAN4 meeting #103-e, the channel model and the corresponding delay spread have been agreed in [1], such as

Issue 1-2-1/1-2-2: TDL Channel Model and RMS Delay Spread - RAN4 will specify demodulation requirements covering TDL-A10 (baseline) and TDL-D10 (for specific test cases) - FFS whether TDL-A20 needed or not Issue 1-2-3: Max Doppler Frequency - TDL-D with 200Hz Max doppler - TDL-A channel: - Option 1: 200Hz - Option 2: 650Hz - Option 3: both 200Hz, and 650Hz

However, several remaining issues related to PDCCH and PBCH demodulation are still needed to be discussed.

# 2	Remaining issues for PDCCH requirements

We propose to define PDCCH with all the possible aggregation levels 2, 4, 8, and 16 by reusing the existing test case specified in TS38.101-4 Section 7.3.2.2 for SCS 120 KHz in FR2-2 (see Appendix A1).

Observation 1: TDD UL-DL pattern should be defined for tests.

Consequently, we propose to define PDCCH requirements in FR2-2 for both SCS 120 KHz and SCS 480 KHz.

Observation 2: While RAN1 defined multi-slot PDCCH monitoring for 480 KHz SCS, we do not believe that it is necessary here since it could be implicitly tested with PDSCH.

Proposal 1: Define PDCCH demodulation requirements for UE in FR2-2 with the following test setup.

FR2-2 TDD, SCS 120 KHz/CBW 100 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 1 | 2 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 1x2 ULA Low | 1_0 | 40 | New |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 1x2 ULA Low | 1_1 | 56 | New |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 2x2 ULA Low | 1_1 | 56 | New |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 2x2 ULA Low | 1_0 | 40 | New |

FR2-2 TDD, SCS 120 KHz/CBW 400 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 2-1 | 3 | 2 | 240 | 1 | 2 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 1x2 ULA Low | 1_0 | 40 | New |
| Test 2-2 | 2 | 6 | 240 | 1 | 4 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 1x2 ULA Low | 1_1 | 56 | New |
| Test 2-3 | 3 | 2 | 240 | 1 | 8 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 2x2 ULA Low | 1_1 | 56 | New |
| Test 2-4 | 3 | 2 | 240 | 1 | 16 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 2x2 ULA Low | 1_1 | 56 | New |

FR2-2 TDD, SCS 480 KHz/CBW 400 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 3-1 | 3 | 2 | 60 | 1 | 2 | TDLA10-200 TDLA20-200 TDLD10-200 | 1x2 ULA Low | 1_0 | 40 | New |
| Test 3-2 | 2 | 6 | 60 | 1 | 4 | TDLA10-200 TDLA20-200 TDLD10-200 | 1x2 ULA Low | 1_1 | 56 | New |
| Test 3-3 | 3 | 2 | 60 | 1 | 8 | TDLA10-200 TDLA20-200 TDLD10-200 | 2x2 ULA Low | 1_1 | 56 | New |
| Test 3-4 | 3 | 2 | 60 | 2 | 16 | TDLA10-200 TDLA20-200 TDLD10-200 | 2x2 ULA Low | 1_0 | 40 | New |

# 3	Remaining issues for PBCH requirements

It has been stated in TR 38.808 that it is recommended to strive for maximum commonality for the system design for licensed and unlicensed operation for FR2-2 band and maximize the re-use of the existing NR design.

Issue 3-2-2: SSB index assumption - Option 1: Only with not known SSB index - Option 2: Both known and not known SSB index - Option 3: Only known SSB index for 120kHz and 480kHz SCS

RAN4 defined the PBCH demodulation requirements (i.e., MIB decoding performance) in Rel-15 and we propose to extend the configurations in FR2-2. Since we have new propagation environments, we should then define new requirements for PBCH considering whether the SS/PBCH block index is known or unknown.

Proposal 2: Define PBCH demodulation requirements with the following test setup

FR2-2 TDD, SCS 120 KHz/CBW 100 MHz

| Test number | SSB SCS | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120 KHz / 100 MHz | 1x2 Low | TDLA10-200 | New | unknown |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLA20-200 | New |  |
|  |  |  | TDLD10-200 | New |  |
| 1-2 |  |  | TDLA10-200 | New | known |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLA20-200 | New |  |
|  |  |  | TDLD10-200 | New |  |

FR2-2 TDD, SCS 480 KHz/CBW 400 MHz

| Test number | SSB SCS | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 480 KHz / 400 MHz | 1x2 Low | TDLA10-200 | New | unknown |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLA20-200 | New |  |
|  |  |  | TDLD10-200 | New |  |
| 1-2 |  |  | TDLA10-200 | New | known |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLA20-200 | New |  |
|  |  |  | TDLD10-200 | New |  |

# 4	Specification structure

Table 1	Specification structure for UE Demodulation performance requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 7.3 | PDCCH demodulation requirements | Updates in Table 7.3-1 |
| 7.3.1 | 1RX requirements | Void |
| 7.3.2 | 2RX requirements |  |
| 7.3.2.2 | TDD |  |
| 7.3.2.2.1 | 1 Tx Antenna performances | Updates on FR2-2 |
| 7.3.2.2.2 | 2 Tx Antenna performances | Updates on FR2-2 |
| 7.4 | PBCH demodulation requirements |  |
| 7.4.1 | 1RX requirements |  |
| 7.4.2 | 2RX requirements |  |
| 7.4.2.2 | TDD | Updates on FR2-2 UEs |

# 5	Summary

In this paper, we tried to contribute with the following proposals for further discussions:

Observation 1: TDD UL-DL pattern should be defined for tests.

Observation 2: While RAN1 defined multi-slot PDCCH monitoring for 480 KHz SCS, we do not believe that it is necessary here since it could be implicitly tested with PDSCH.

Proposal 1: Define PDCCH demodulation requirements for UE in FR2-2 with the following test setup.

FR2-2 TDD, SCS 120 KHz/CBW 100 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 1 | 2 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 1x2 ULA Low | 1_0 | 40 | New |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 1x2 ULA Low | 1_1 | 56 | New |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 2x2 ULA Low | 1_1 | 56 | New |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 2x2 ULA Low | 1_0 | 40 | New |

FR2-2 TDD, SCS 120 KHz/CBW 400 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 2-1 | 3 | 2 | 240 | 1 | 2 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 1x2 ULA Low | 1_0 | 40 | New |
| Test 2-2 | 2 | 6 | 240 | 1 | 4 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 1x2 ULA Low | 1_1 | 56 | New |
| Test 2-3 | 3 | 2 | 240 | 1 | 8 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 2x2 ULA Low | 1_1 | 56 | New |
| Test 2-4 | 3 | 2 | 240 | 1 | 16 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 2x2 ULA Low | 1_1 | 56 | New |

FR2-2 TDD, SCS 480 KHz/CBW 400 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 3-1 | 3 | 2 | 60 | 1 | 2 | TDLA10-200 TDLA20-200 TDLD10-200 | 1x2 ULA Low | 1_0 | 40 | New |
| Test 3-2 | 2 | 6 | 60 | 1 | 4 | TDLA10-200 TDLA20-200 TDLD10-200 | 1x2 ULA Low | 1_1 | 56 | New |
| Test 3-3 | 3 | 2 | 60 | 1 | 8 | TDLA10-200 TDLA20-200 TDLD10-200 | 2x2 ULA Low | 1_1 | 56 | New |
| Test 3-4 | 3 | 2 | 60 | 2 | 16 | TDLA10-200 TDLA20-200 TDLD10-200 | 2x2 ULA Low | 1_0 | 40 | New |

Proposal 2: Define PBCH demodulation requirements with the following test setup

FR2-2 TDD, SCS 120 KHz/CBW 100 MHz

| Test number | SSB SCS | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120 KHz / 100 MHz | 1x2 Low | TDLA10-200 | New | unknown |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLA20-200 | New |  |
|  |  |  | TDLD10-200 | New |  |
| 1-2 |  |  | TDLA10-200 | New | known |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLA20-200 | New |  |
|  |  |  | TDLD10-200 | New |  |

FR2-2 TDD, SCS 480 KHz/CBW 400 MHz

| Test number | SSB SCS | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 480 KHz / 400 MHz | 1x2 Low | TDLA10-200 | New | unknown |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLA20-200 | New |  |
|  |  |  | TDLD10-200 | New |  |
| 1-2 |  |  | TDLA10-200 | New | known |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLA20-200 | New |  |
|  |  |  | TDLD10-200 | New |  |

# References

- R4-2210665, “WF on UE demodulation performance requirements definition for 52.6 - 71 GHz”, Qualcomm.

# Appendix	Simulation assumption

## A.1	PDCCH

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | TBD |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 2 for test 1-1 6 for test 1-2 | 2 |
| Interleaver size |  | 3 for test 1-1 2 for test 1-2 | 3 |
| Shift index |  | 0 |  |

## A.2	PBCH

| Parameter | Unit | Single antenna port |
| --- | --- | --- |
| Physical Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index Note1 |  | 0 |
| SS/PBCH block periodicity | ms | 20 |
| TDD UL-DL pattern |  | TBD |
| Note 1: as specified in clause 4.1 of TS 38.213 Note 2: as specified in clause 11.1 of TS 38.213 |  |  |

See 38.101-4 Table 7.4.2.2-1