3GPP TSG-RAN WG4 Meeting #104-e	R4-2212574

Electronic Meeting, August 15 – August 26, 2022

Agenda Item:	9.14.8.2.3

Source:	Ericsson

Title:	SDR requirements in 52.6 – 71 GHz band

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on the SDR requirements for FR2-2, where some related aspects have been gathered in the WF [1] and still need to be discussed. It is worth reminding that the purpose of the SDR test is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by the UE capabilities.

Issue 4-1-1: Rank - Define SDR requirements for Rank1. - Decide based on simulation results for Rank2. Issue 4-1-2: SCS - Option 1: Only 120kHz - Option 2: 120kHz and 480kHz Issue 4-1-3: PN Compensation - Option 1: Same as PDSCH test cases based on outcome of Issue 2-1-12 - Option 2: CPE compensation only Issue 4-1-4: Changes to Maximum MCS for FR2-2 64QAM, Rank1 Decide based on simulation results in the next meeting - Option 1: MCS24 for Scaling factor 1. - Option 2: MCS27 for Scaling factor 1. - Other Options not precluded. Issue 4-1-5: Changes to Maximum MCS for FR2-2 64QAM, Rank2 (if defined) Decide based on simulation results in the next meeting - Option 1: MCS22 for Scaling factors 1 and 0.8. - Other Options not precluded. Issue 4-1-6: Minimum MCS for FR2-2 MCS to SNR mapping Decide based on simulation results in the next meeting - Option 1: MCS 6 - Other Options not precluded. Issue 4-1-7: Whether to define separate MCS to SNR mapping tables for different CBWs - Option 1: Yes - Option 2: No - Option 3: Decide based on simulation results. Issue 4-1-8: SDR procedure for FR2-2 - For scenario of FR2-2 DC or CA with FR1, use procedure specified in clause 9.4A.1 of TS 38.101-4. For scenario of SA FR2-2, use procedure specified in 7.5.1 of TS 38.101-4. Issue 4-1-9: Common Parameters for FR2-2 - Reuse common parameters for FR2-1 120kHz. Issue 4-1-10: Scenarios - Define SDR requirements for following scenarios: - SA FR2-2 - FR1 (CA or single CC) CA with FR2-2 (CA or single CC) - Further discuss which cases can be covered considering limited number of HARQ processes. For example, one such case could be FR1 FDD + FR2-2 CA. - FR1 (CA or single CC) DC with FR2-2 (CA or single CC) Issue 4-1-11: Schedule and feedback patterns for FR1+FR2-2 CA scenario FFS

# 2	SDR Requirements

The purpose of the SDR requirements is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. According to TS38.101-4, the existing MCS index table has already covered the configuration applicable for UEs such as rank 1/2 and QPSK/16QAM/64QAM. RAN4 only need to evaluate the applicability rule for FR2-2 where rank 1 is supported as shown in Table 3.6-1.

Table 3.6-1	MCS indexes for indicated UE capabilities applicable for FR2-2.

| Received antenna | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 2Rx UE | 1 | 6 | 0.75 | 22 |
|  | 1 | 6 | 0.4 | 14 |
|  | 1 | 4 | 1 | 16 |
|  | 1 | 4 | 0.8 | 16 |
|  | 1 | 4 | 0.75 | 16 |
|  | 1 | 4 | 0.4 | 10 |
|  | 1 | 2 | 1 | 9 |
|  | 1 | 2 | 0.8 | 9 |
|  | 1 | 2 | 0.75 | 9 |
|  | 1 | 2 | 0.4 | 4 |

Proposal 1: Define SDR test for UE in FR2-2 considering 2Rx UE considering the following parameters

| Received antenna | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS | PN compensation |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PRB (66) | PRB (124 or 264) |
| 2Rx UE | 1 | 6 | 1 | 27 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.8 | 23 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.75 | 22 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.4 | 14 | CPE only | CPE+ICI (u=2) |
|  |  | 4 | 1 | 16 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.4 | 10 | CPE only | CPE only |
|  |  | 2 | 1 | 9 | CPE only | CPE only |
|  |  |  | 0.4 | 4 | CPE only | CPE only |

where the following numerologies have been considered (under PN effects):

120 KHz/100 MHz (66 RBs), 120 KHz/400 MHz (264 RBs), 480 KHz/400 MHz (66 RBs), and 480 KHz/800 MHz (124 RBs).

Proposal 2: Define SDR requirements for following scenarios:

  - SA FR2-2

  - FR1 (CA or single CC) CA with FR2-2 (CA or single CC)

  - FR1 (CA or single CC) DC with FR2-2 (CA or single CC)

# 3	Specification structure

Table 1	Specification structure for UE Demodulation performance requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 7.5 | Sustained downlink data rate provided by lower layers |  |
| 7.5.1 | FR2 single carrier requirements | Updates on FR2-2 |

# 4	Summary

In this paper we tried to contribute with the following proposal for further discussions:

Proposal 1: Define SDR test for UE in FR2-2 considering 2Rx UE considering the following parameters

| Received antenna | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS | PN compensation |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PRB (66) | PRB (124 or 264) |
| 2Rx UE | 1 | 6 | 1 | 27 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.8 | 23 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.75 | 22 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.4 | 14 | CPE only | CPE+ICI (u=2) |
|  |  | 4 | 1 | 16 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.4 | 10 | CPE only | CPE only |
|  |  | 2 | 1 | 9 | CPE only | CPE only |
|  |  |  | 0.4 | 4 | CPE only | CPE only |

where the following numerologies have been considered (under PN effects):

120 KHz/100 MHz (66 RBs), 120 KHz/400 MHz (264 RBs), 480 KHz/400 MHz (66 RBs), and 480 KHz/800 MHz (124 RBs).

Proposal 2: Define SDR requirements for following scenarios:

  - SA FR2-2

  - FR1 (CA or single CC) CA with FR2-2 (CA or single CC)

  - FR1 (CA or single CC) DC with FR2-2 (CA or single CC)

# References

- R4-2210665, “WF on UE demodulation performance requirements definition for 52.6 - 71 GHz”, Qualcomm.

# Appendix	Simulation assumption

## A.1	MCS Table from TS 38.214

Table 5.1.3.1-1: MCS index table 1 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |