3GPP TSG-RAN WG4 Meeting #104-e	R4-2212576

Electronic Meeting, August 15 – August 26, 2022

Agenda Item:	9.14.8.2.4

Source:	Ericsson

Title:	CSI reporting requirements in 52.6 – 71 GHz band

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on the CSI reporting requirements in 52.5 GHz – 71 GHz band, where the related aspects have been gathered in the WF [1].

It is worth reminding that only CSI reporting under static conditions will be considered, where some remaining issues should be discussed

Issue 5-1-1: CBW/SCS Combination - Option 1: Only 100MHz/120kHz - Option 2: 100MHz/120kHz and 400MHz/480kHz Issue 5-1-2: CSI-RS Periodicity/Offset for 480kHz SCS (if agreed in Issue 5-1-1) - Periodicity/offset = 32/1 slot and follow TDD UL-DL pattern FR2.120-2 as: FR2-2 TDD 480 kHz: 11D S 4U, S: 2D:12G:0U Issue 5-1-3: CQI Delay for 480kHz SCS (if agreed in Issue 5-1-1) - Option 1: Same as 120kHz, i.e., 8.375ms - Option 2: Different value based on TDD pattern 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U. Issue 5-1-4: SNR value - Option 1: Not consider SNR related to 64QAM with following reasons: - The lowest SNR with CQI reporting related to 64QAM is 12dB which is rather higher than maximum achievable SNRBB specified by RF part. - Phase noise has big impact on the BLER performance when SNR is equal or larger than 14dB which CQI reporting index is larger than 12. - Option 2: Decide based on simulation results.

# 2	CSI Reporting Requirements

RAN4 has defined CQI, PMI and RI reporting requirements in Rel-15. For Rel-17 and FR2-2 band, we will follow RAN1 where PDSCH MIMO rank has been agreed to be equal to 1. Consequently, we propose to define channel quality indicator (CQI) reporting requirements only, since we do not need to verify the rank indicator (RI). Furthermore, we need to define the CQI reporting requirements using 64QAM for 2Rx UE.

The reporting accuracy of the CQI under static conditions is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI median.

Proposal 1: Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM), see Appendix A.1, by reusing the existing test setup and metrics for SCS 120 KHz.

- Consider 120 KHz/100 MHz (66 RBs) and use PN with CPE compensation only.

Proposal 2: For SCS 480 KHz in static conditions, define CSI reporting by considering

- Consider 480 KHz/400 MHz (66 RBs) and use PN with CPE compensation only.

- Periodicity/offset = 32/1 slot and follow TDD UL-DL pattern FR2.120-2 as:

FR2-2 TDD 480 KHz: 11D S 4U, S: 2D:12G:0U

- Keep same CQI delay as 120 KHz SCS, i.e., 8.375 ms.

# 3	Specification structure

Table 1	Specification structure for UE CSI reporting requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 8.2 | Reporting of Channel Quality Indicator (CQI) |  |
| 8.2.2 | 2RX requirements |  |
| 8.2.2.2 | TDD |  |
| 8.2.2.2.1 | CQI reporting definition under AWGN | Updates on FR2-2 |
| 8.2.2.2.2 | CQI reporting under fading conditions | Updates on FR2-2 |

# 4	Summary

In this paper, we tried to contribute with the following proposals for further discussions:

Proposal 1: Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM), see Appendix A.1, by reusing the existing test setup and metrics for SCS 120 KHz.

- Consider 120 KHz/100 MHz (66 RBs) and use PN with CPE compensation only.

Proposal 2: For SCS 480 KHz in static conditions, define CSI reporting by considering

- Consider 480 KHz/400 MHz (66 RBs) and use PN with CPE compensation only.

- Periodicity/offset = 32/1 slot and follow TDD UL-DL pattern FR2.120-2 as:

FR2-2 TDD 480 KHz: 11D S 4U, S: 2D:12G:0U

- Keep same CQI delay as 120 KHz SCS, i.e., 8.375 ms.

# References

- R4-2210665, “WF on UE demodulation performance requirements definition for 52.6 - 71 GHz”, Qualcomm.

# Appendix

## A.1	CQI Tables

We consider Table A.4-1 (TS 38.101-4) mapping CQI index to the information bit payload, given by

Table A.4-1: Mapping of CQI Index to Information Bit payload (CQI table 1)

| TBS Scheme |  |  |  | TBS.1-1 | TBS.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 64QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 66 | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 |  |  |  |  |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 |  |  |  |  |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 |  |  |  |  |
| Overhead for TBS determination |  |  |  | 6 | 6 |  |  |  |  |
| Available RE-s |  |  |  | 7590 | 7590 |  |  |  |  |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A |  |  |  |  |
| 1 | 0.2344 | 0 | QPSK | 1800 | 3624 |  |  |  |  |
| 2 | 0.2344 | 0 |  | 1800 | 3624 |  |  |  |  |
| 3 | 0.3770 | 2 |  | 2856 | 5640 |  |  |  |  |
| 4 | 0.6016 | 4 |  | 4480 | 8968 |  |  |  |  |
| 5 | 0.8770 | 6 |  | 6528 | 13064 |  |  |  |  |
| 6 | 1.1758 | 8 |  | 8712 | 17928 |  |  |  |  |
| 7 | 1.4766 | 11 | 16QAM | 11016 | 22032 |  |  |  |  |
| 8 | 1.9141 | 13 |  | 14343 | 28680 |  |  |  |  |
| 9 | 2.4063 | 15 |  | 17928 | 35856 |  |  |  |  |
| 10 | 2.7305 | 18 | 64QAM | 20496 | 40976 |  |  |  |  |
| 11 | 3.3223 | 20 |  | 25104 | 50184 |  |  |  |  |
| 12 | 3.9023 | 22 |  | 29192 | 58384 |  |  |  |  |
| 13 | 4.5234 | 24 |  | 33816 | 67584 |  |  |  |  |
| 14 | 5.1152 | 26 |  | 38936 | 77896 |  |  |  |  |
| 15 | 5.5547 | 28 |  | 42016 | 83976 |  |  |  |  |

where the CQI indexes can be mapped to the corresponding MCS as given by Table 5.1.3.1-1 (TS 38.214)

| CQI Index | MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- | --- |
| 1 - 2 | 0 | 2 | 120 | 0.2344 |
| 3 | 2 | 2 | 193 | 0.3770 |
| 4 | 4 | 2 | 308 | 0.6016 |
| 5 | 6 | 2 | 449 | 0.8770 |
| 6 | 8 | 2 | 602 | 1.1758 |
| 7 | 11 | 4 | 378 | 1.4766 |
| 8 | 13 | 4 | 490 | 1.9141 |
| 9 | 15 | 4 | 616 | 2.4063 |
| 10 | 18 | 6 | 466 | 2.7305 |
| 11 | 20 | 6 | 567 | 3.3223 |
| 12 | 22 | 6 | 666 | 3.9023 |
| 13 | 24 | 6 | 772 | 4.5234 |
| 14 | 26 | 6 | 873 | 5.1152 |
| 15 | 28 | 6 | 948 | 5.5547 |

## A.2	CQI reporting in static condition

### A.2.1	Test metric

- The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

- If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

| For FR2-2 Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 | 400 |
| Subcarrier spacing |  |  | kHz | 120 | 480 |
| Duplex Mode |  |  |  | TDD |  |
| TDD Slot Configuration |  |  |  | FR2.120-2 Annex A.1.3 |  |
| SNRBB |  |  | dB | TBD |  |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 2x2 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | FD-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |
|  | CSI-RS periodicity and offset |  | Slot | 8/1 32/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |
|  | CDM Type |  |  | fd-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 8/1 32/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 1 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 8/1 |  |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 1 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 8 |  |
| csi-ReportingBand |  |  |  | 111111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 8/3 |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type | typeI-SinglePanel | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |
|  |  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 8.375 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | TBD |  |