3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212587

# Online meeting, 15-26 Aug, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR |  | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Rel-17 38.101-2 to correct the configured transmitted power for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | the explanation of PPowerclass in configured transmitted power for CA is unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the explanation of PPowerclass. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | if not, it will cause understanding confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

6.2A.4.2	Configured transmitted power for inter-band UL CA

A UE can configure its maximum output power for each uplink band when it is configured for inter-band UL carrier aggregation with two NR bands each with a single UL CC. For each uplink band n, the configured UE maximum output power PCMAX,f,c,n for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11].

The configured UE maximum output power PCMAX,f,c,n for carrier f of a serving cell c in band n shall be set such that the corresponding measured peak EIRP PUMAX,f,c,n is within the following bounds

PPowerclass + PIBE – MAX(MAX(MPRf,c,n, A- MPRf,c,n) + ΔTIB,P,n, P-MPRf,c,n) – MAX{T(MAX(MPRf,c,n, A- MPRf,c,n,)), T(P-MPRf,c,n)} ≤ PUMAX,f,c,n ≤ EIRPmax,n

while the corresponding measured total radiated power in uplink band n, PTMAX,f,c,n , is bounded by

PTMAX,f,c,n ≤ TRPmax,n

with PPowerclass the UE minimum peak EIRP as specified in sub-clause 6.2A.1, EIRPmax,n the applicable maximum EIRP as specified in sub-clause 6.2A.1 for uplink band n and TRPmax,n the applicable maximum TRP as specified in sub-clause 6.2A.1 for uplink band n. MPRf,c,n as specified in sub-clause 6.2A.2 for uplink band n, A-MPRf,c,n as specified in sub-clause 6.2A.3 for uplink band n, ΔTIB,P,n the peak EIRP relaxation as specified in clause 6.2A.1. The requirement is verified in beam peak direction.

PIBE, mpr-PowerBoost-FR2-r16 and maxUplinkDutyCycle-FR2 are described in clause 6.2.4.

P-MPRf,c,n is the power management maximum output power reduction P-MPRf,c in band n. P-MPRf,c is defined in clause 6.2.4.

The tolerance T(∆P) for applicable values of ∆P (values in dB) in each band is specified in Table 6.2.4-1.

< end of changes >