3GPP TSG-RAN WG4 Meeting # 104-e                                 R4-2212611

Electronic Meeting, August 15 – August 26, 2022

Title: 	Discussion on system parameters for APT 600MHz

Source: 	CATT

Agenda item:	10.19.2

Document for:	Approval

- Introduction

A new WI [1] on APT 600MHz NR band was approved in RAN#96 meeting and RAN4 is planned to initiate the work from this meeting. Based on the request from APT, option B1 with a single duplexer is targeted for 3GPP specification work. This contribution will provide our view on system parameters, including operating band, UE channel bandwidth and channel arrangement for APT 600MHz.

- Discussion

2.1 Operating band and UE channel bandwidth

Based on the request from APT and the approved WID, the operating band for APT 600MHz should be defined as Table 1. Given the existing bands defined in TS 38.101-1, the band number for APT 600MHz can be n105 on a first-come first-served basis.

Table 1: NR operating band

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [n105] | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Based on the request from APT and the approved WID, the UE channel bandwidth and SCS parameters should be defined as Table 2.

Table 2: Channel bandwidth and SCS parameters

| SCS (kHz) | channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 |  |
| 30 |  | 10 | 15 | 20 | 25 | 30 | 35 |  |

Proposal 1: The band number for APT 600MHz can be defined as n105 on a first-come first served basis.

2.2 Channel arrangement

Channel raster

Considering the APT band is not a LTE refarming band, it is not required to define 100kHz channel raster like other sub-1GHz bands specified in 38.101-1. 15 kHz/30kHz channel raster would enable more flexibility and be more appropriate for the new NR band n105. The applicable NR-ARFCN is calculated as Table 3.

Table 3: Applicable NR-ARFCN

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n105] | 15 | 132600 – <3> – 140600 | 122400 – <3> – 130400 |
|  | 30 | 132600 – <6> – 140600 | 122400 – <6> – 130400 |

Proposal 2: It is proposed to define 15kHz/30kHz channel raster for the new NR band n105. The applicable NR-ARFCN is calculated as Table 3.

Sync raster

Based on the sync raster definition, the applicable SS raster entries for APT 600MHz can be defined as Table 4.

Table 4: Applicable SS raster entries

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n105] | 15 kHz | Case A | 1535 – <1> – 1624 |

Proposal 3: The applicable SS raster entries can be defined as Table 4.

- Conclusion

This contribution mainly presents the analysis on system parameters for APT 600MHz. The following proposals are provided:

Proposal 1: The band number for APT 600MHz can be defined as n105 on a first-come first served basis.

Proposal 2: It is proposed to define 15kHz/30kHz channel raster for the new NR band n105. The applicable NR-ARFCN is calculated as Table 3.

Proposal 3: The applicable SS raster entries can be defined as Table 4.

- Reference

- RP-221778, New WID on APT 600 MHz NR band, Spark NZ, Qualcomm, RAN#96-e

- TS 38.101-1 v17.6.0, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone