3GPP TSG-RAN WG4 Meeting # 104-e                                 R4-2212612

Electronic Meeting, August 15 – August 26, 2022

Title: 	Discussion on UE RF requirements for APT 600MHz

Source: 	CATT

Agenda item:	10.19.3

Document for:	Approval

- Introduction

A new WI [1] on APT 600MHz NR band was approved in RAN#96 meeting and RAN4 is planned to initiate the work from this meeting. Based on the request from APT, option B1 with a single duplexer is targeted for 3GPP specification work. This contribution will provide our view on UE RF requirements for APT 600MHz.

- Discussion

The following normative regulatory information is provide in the reply LS [2]:

a) Frequency plan for allowed operation in the band AWG has considered the options B1 and B2 and would like to make the following recommendations to 3GPP: 1. That B1 is the preferred option for APT and be referred to as the ‘APT 600 MHz’ band. 2. That AWG invites 3GPP to immediately start work on the technical specifications to support Option B1. 3. That B2 may be considered as an option for later standardization should it be required (in a 35 MHz + 35 MHz configuration). b) Out of band emission limits APT/AWG is not considering any band specific emission limit other than the general emission limits already specified 3GPP TS 38.101-1 and TS 38.104. However, it is not precluded additional emission limit may be introduced by each administration to protect the other services. We suggest that out of band emission limits for both categories A and B should be defined. c) Emission limits outside of the allocated channel, or emission mask, as applicable APT/AWG is not considering any band specific emission limit other than the general emission limits already specified in 3GPP TS 38.101-1 and TS 38.104. From the network side, NS_35 is intended for US specific requirements and may not apply in all APT countries. From the UE side, the device should ideally support both the APT band plan as well as Band n71 for roaming purposes. As such, the UE should support NS_35 for operation in Band n71 networks. The choice between categories A and B for specifying base station unwanted emissions is for each Administration. d) Protected services and associated emission limits The AWG would like to maximise spectrum efficiency and minimise frequency separations (guard bands) between DTT and Mobile. APT/AWG observes that 3GPP already studied them and summarized in TR 38.860 in the following. • It was concluded that due to various DTV channel arrangements across national regulations, the frequency separation is considered to be decided by administrators. APT countries use 6, 7 and 8 MHz TV channels. We have undertaken simulations to determine guard band requirements for DTV interference to UE blocking. • The AWG has undertaken a study on a 4 MHz frequency separation (guard band). The results of the study found that 4 MHz of frequency separation (guard band) is adequate for mobile user equipment to operate in the presence of DTT transmitters without interference (see details in the attached document). This means that option B1 would not impact any additional DTT channels on the 8 MHz and 6 MHz DTT channel spacing / raster compared with option B2. • For all the frequency arrangements considered in this study item, it was agreed that no BS spurious emissions limits be specified for protection of RAS services. Any possible protections measured required for the RAS services protection are considered to be under the responsibility of each administration considering aspects such as distance separation site engineering solutions, etc. The AWG invite 3GPP to consider the attached study when it is undertaking its work on the APT 600 MHz band plan. e) Blocking requirements, if applicable APT/AWG is not considering any band specific blocking requirement other than the general blocking requirement already specified 3GPP TS 38.101-1 and TS 38.104. f) Maximum transmit power APT/AWG is not considering any specific requirement other than the already available UE power classes in 3GPP TS 38.101-1 and base station classes in TS 38.104 for other NR bands.

Based on the approved WID and the request from APT, an overview of UE RF requirement impact is provided in Table 1.

Table 1: Overview on RF requirement impact for APT 600MHz

| Transmitter |  |
| --- | --- |
| UE maximum output power | These requirements are defined as band agnostic except for MOP and A-MPR. It is not expected to have any impact on the RF requirements except for MOP and A-MPR for the introduction of APT 600MHz. |
| UE maximum output power reduction |  |
| UE additional maximum output power reduction |  |
| Configured transmitted power |  |
| Output power dynamics |  |
| Transmit signal quality |  |
| Occupied bandwidth |  |
| Out of band emission |  |
| Spurious emission |  |
| Transmitter intermodulation |  |
| Receiver |  |
| Reference sensitivity | The REFSENS for band n71 can be reused for APT 600MHz. |
| Maximum input level | These requirements are defined as band agnostic. It is not expected to have any impact on these requirements for the introduction of APT 600MHz. |
| Adjacent channel selectivity |  |
| Blocking | The in-band and out-of-band blocking requirements for band n71 can be reused for APT 600MHz. |
| Spurious response | These requirements are defined as band agnostic. It is not expected to have any impact on these requirements for the introduction of APT 600MHz. |
| Intermodulation |  |
| Spurious emission |  |

- Conclusion

This contribution provides an overview of UE RF requirement impact for APT 600MHz in Table 1.

- Reference

- RP-221778, New WID on APT 600 MHz NR band, Spark NZ, Qualcomm, RAN#96-e

- RP-221045, FREQUENCY ARRANGEMENTS FOR IMT IN THE BAND 470 – 703 MHZ, APT, RAN#96-e