3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212634

Electronic Meeting, August 15 – August 26, 2022

Source:	ZTE Corporation

Title:          Discussion on timing issues for simultaneous operation of IAB

Agenda Item:	9.15.1

Document for:	Approval

- Introduction

In this contribution, the analysis on the issue of timing for simultaneous operation between IAB-DU and IAB-MT of the same IAB-Node is provided.

- Discussion

In RAN4 #102-e meeting, TEA requirement for simultaneous operation was agreed[1] as following:

Issue 1-2-1: Timing error between intra-node MT TX and DU TX for case#6 Candidate options: - Option 1: To specify TAE between IAB-MT and IAB-DU in timing case #6 (Nokia, Samsung, ZTE) - The requirement value is min [3us , 4.69 / (SCS/15 kHz) µs]. - Option 2: No TAE between IAB-MT and IAB-DU (Huawei, Ericsson) Agreement: Introducing this requirement into core specification - Whether IAB need to pass corresponding conformance test cases if introduced pending on IAB declaration - Further discuss the details of conformance test cases if introduced in conformance phase - It’s FFS whether conformance test cases needed or not considering test feasibility - Note: Cell phase sync performance maybe relaxed by TAE. If relaxtion not acceptable, RAN4 can further revisit the agreement.

The value of min [3us , 4.69 / (SCS/15 kHz) µs] was not confirmed in 103-e meeting, the main concern for not being able to reach a conclusion is that even a smaller value is determined the UL Rx timing of the parent node can not be guaranteed as the DU Tx timing between parent node and IAB node may be up to 3us.

Figure 1 shows the relation of UL Rx timing and the TAEs when child IAB node operates in case 6 timing mode, the parent node will assume the MT UL Tx timing of the child IAB node is aligned with its collocated DU DL Tx timing thus the DL Tx timing difference between parent DU and child node DU will put impact on the parent DU UL Rx timing.

Figure 1 Relation between TAEs and UL Rx timing

From figure 1 we can find that 2 Tes will put impacts on UL Rx timing uncertainty. Te1 in the figure is the timing uncertainty between MT Tx timing and the co-located DU Tx timing, Te2 is the timing uncertainty between parent DU Tx timing and child DU Tx timing.

With regard to Te2, the IAB node can use OTA timing scheme to ensure DL Tx timing alignment[2].

If an IAB-node is provided an index Tdelta in a Timing Delta MAC CE [11, TS 38.321] from a serving cell, the IAB-node may assume that TTA/2+Ndelta+Tdelta⋅Gstep−NTA,Offset/2⋅Tc is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-MT when TTA/2+Ndelta+Tdelta⋅Gstep−NTA,Offset/2⋅Tc>0, where - TTA is the difference between the IAB-MT reception time and the IAB-MT transmission time for IAB-MT transmission timing mode 'Case6', and is defined in clause 4.3.1 of [4, TS 38.211] for IAB-MT transmission timing mode 'Case1' and parent node reception mode 'Case7' - Ndelta and Gstep are determined as - Ndelta=−70528 and Gstep=64, if the serving cell providing the Timing Delta MAC CE operates in FR1 - Ndelta=−17664 and Gstep=32, if the serving cell providing the Timing Delta MAC CE operates in FR2

We think the DL Tx timing between parent node and child node can be adjusted with the accuracy of Tdelta which is much smaller than the CP duration as shown in table 1.

Table 1, Comparison between Tdelta accuracy and CP duartion

| Tdelta accuracy |  | CP duration(normal CP) |  |
| --- | --- | --- | --- |
| FR1 | FR2 | @30kHz FR1 | @120kHz FR2 |
| 32.5ns | 16.3ns | 2343ns | 586ns |

Observation 1: If OTA timing scheme is performed between parent IAB node and child IAB node, the timing difference between parent node and child node can be very smaller than the CP duration of FR1 and FR2 respectively.

If OTA timing mechanism is adopted between parent nodes, a reasonable assumption is that the TAE between parent node and child node will be in a range that will much smaller that CP duration and the previously reached timing difference between IAB-MT and co-located IAB-DU of the same IAB-Node can be confirmed.

Moreover, multi-hop is supported in IAB network which can be found in TR38874 that TA-based OTA synchronization can support a multi-hop IAB network (up to 5 hops) for FR2. If the timing difference between the child node and the parent node is 3us, the first-order node and the last-order node in the 5-hop IAB network cannot guarantee the cell synchronization requirement. Therefore, we can not assume the TAE between parent IAB node DU and child IAB node DU is 3us, a value much smaller than 3us can support multi-hop IAB network (up to 5 hops) at least for FR2.

Observation 2: For multi-hop IAB networks, a timing difference of less than 3us between a parent node and a child node is a reasonable assumption to meet cell synchronization requirement.

Proposal 1: To confirm the TAE requirement between IAB-DU and IAB-MT as min [3us , 4.69 / (SCS/15 kHz) µs].

- Conclusion

Based on the analysis, the following observation and proposal are given:

Observation 1: If OTA timing scheme is performed between parent IAB node and child IAB node, the timing difference between parent node and child node can be very smaller than the CP duration of FR1 and FR2 respectively.

Observation 2: For multi-hop IAB networks, a timing difference of less than 3us between a parent node and a child node is a reasonable assumption to meet cell synchronization requirement.

Proposal 1: To confirm the TAE requirement between IAB-DU and IAB-MT as min [3us , 4.69 / (SCS/15 kHz) µs].

- Reference

RAN4_102-e_BSRF_Demod_Test_ Session_report_EOMv2_AfterPostMeetingApproval.docx

38213-h10.docx