3GPP TSG-RAN4 Meeting #104-e	R4-2212652

# Electronic meeting, August 15-26, 2022

Source:			Ericsson

Title:			NTN: satellite UE  out of band blocking requirement

Agenda item:		9.11.4.2

Document for:		Approval

# Introduction

The NTN WI ([1]) has been approved in RAN#88e meeting to specify requirements for the support of NTN. It has been revised in last RAN#94-e meeting [2].

In last RAN4#103-e meeting, the first version of the NTN satellite UE specification TS 38.101-5 was agreed. Nevertheless, the out of band blocking requirement was specified with [] for n256. This contribution is so further discussing this requirement.

# Discussion

## Background

The NTN satellite UE out of band blocking requirement was specified in TS 38.101-5 according the following:

As it could be noticed, the [] have been kept for band n256 requirement (notes 1, 2, 3 and 4), this to leave more time for companies to consider how to best specify this requirement for band n256.

## Discussion and proposal

In last RAN4#103-e meeting, RAN4 made the following agreement ([3]):

As both type of duplexer has to be considered for n256, this should be taken into account when specifying out of band blocking requirement: the -30dBm interferer limit for Range 2 can’t be supported with a 90MHz duplexer, out of band blocking relaxation would be needed.

Observation 1: To enable reusing n65 duplexer, n256 out of band requirement needs to be relaxed.

Two approaches have been proposed in last RAN4#103-e meeting, which are somehow captured in the out of band blocking table 7.6.3-2 notes as well:

- Keeping the same ranges but specifying new interferer power levels for ranges 2 and 3 for n256, as suggested by notes 3 and 4.

- Keeping the same interferer power levels but specifying new ranges 2 and 3 for n256, as suggested by notes 1 and 2.

At least, it was common understanding to not modify both the interferer power level and the frequency offset ranges.

If each of both approaches above has a good rationale. Still, we would propose to update the frequency offset ranges 2 and 3 for n256, keeping the interferer power level unchanged to preserve the same protection level at least on some part of the ranges.

Nevertheless, in the note 1, the range 2 is 45 MHz wide, while “usual” TN range 2 is 25 MHz wide. As the assumption here would be to reuse TN n65 duplexer, there is no reason to extend the range 2 width, it should be kept to 25 MHz width.

Also, there is a need to extend range 1 down to 100MHz below the band to align with the new frequency offset range 2 which would start at 100MHz below band n256.

Proposal: Specify NTN satellite UE out of band requirement for band n256 according to Table 1 (changes are yellow highlighted):

Table 1: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | [-30] | -15 |
| n255, n256 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: For band n256 in Range 1 requirement, the applicable lower frequency range should be modified as -100 < f – FDL_low ≤ -15. For band n256 in Range 2 requirement, the applicable lower frequency range should be modified as -125 < f – FDL_low ≤ -100. NOTE 2: For band n256 in Range 3 requirement, the applicable lower frequency range should be modified as 1 ≤ f ≤ FDL_low –125 NOTE 3: For band n256 in Range 2 requirement, the Pinterferer should be modified as [-30] NOTE 4: For band n256 in Range 3 requirement, the Pinterferer should be modified as [-15] |  |  |  |  |  |

Note that our companion CR ([4]) is reflecting this update.

# Conclusion

In this contribution, we discussed the issue with the current specification of the NTN satellite UE out of band blocking requirement and made the following observation and proposal:

Observation: To enable reusing n65 duplexer, n256 out of band requirement needs to be relaxed.

Proposal: Specify NTN satellite UE out of band requirement for band n256 according to Table 1 (changes are yellow highlighted):

Table 1: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | [-30] | -15 |
| n255, n256 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: For band n256 in Range 1 requirement, the applicable lower frequency range should be modified as -100 < f – FDL_low ≤ -15. For band n256 in Range 2 requirement, the applicable lower frequency range should be modified as -125 < f – FDL_low ≤ -100. NOTE 2: For band n256 in Range 3 requirement, the applicable lower frequency range should be modified as 1 ≤ f ≤ FDL_low –125 NOTE 3: For band n256 in Range 2 requirement, the Pinterferer should be modified as [-30] NOTE 4: For band n256 in Range 3 requirement, the Pinterferer should be modified as [-15] |  |  |  |  |  |

# References

- RP-201256, Solutions for NR to support non-terrestrial networks (NTN), Thales

- RP-213691, Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2210635, WF on remaining issue of NTN UE RF, ZTE

- R4-2212645, CR to TS 38.108 - Tx requirements issues fixes, Ericsson