3GPP TSG-RAN WG4 Meeting # 104-e                	R4-2212674

Electronic Meeting, August 15 – 26, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on PUSCH demodulation requirements for the extension to 71 GHz

Agenda item: 	9.14.8.3.1

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on general aspects of demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

Discussion

Number of PRBs

On the last RAN4 meeting we reached agreement on the combinations of CBW and number of PRBs as follow [2]:

General PUSCH test setup Consider the following simulation assumptions at starting point for PUSCH performance requirements: - MCS 4, 16, 20 - FFS MCS 4, 16, 20 for 1 Tx and MCS 4, 16 for 2 Tx - DM-RS/PT-RS configuration Rel-15 assumptions - 1+1 DMRS configuration - PTRS Tx on - Temporary PRB number (for simulations): - (66)(264)(66)([124])([248])(33)([62])([124])(148) for SCS (kHz CWB (MHz)) = (120 100)(120 400)(480 400) (480 800)(480 1600)(960 400)(960 800)(960 1600)(960 2000) - The numbers are modified according to RF agreements in this meeting

This number of PRBs is in line with what was agreed during the UE RF discussion [4][5]. However, this number of PRBs cannot be applied also when transform precoding is enabled. For DFT-s-OFDM the number of PRBs must be such that it can be approximated by the formula:

MRBPUSCH=2α23α33α5

Where α2, α3, and α5 is a set of non-negative integers. Considering this limitation on the number of PRBs, the maximum number of PRBs that can be used with transform precoding for FR2-2 would be

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 100 | 64 | 256 | N/A | N/A | N/A |
| 480 | N/A | 64 | 120 | 240 | N/A |
| 960 | N/A | 32 | 60 | 120 | 144 |

Latest RAN4 agreements on number of PRBs cannot be used for transform-precoding test cases for which the number of PRBs have to be calculated considering powers of 2, 3, and 5 only.

Adopt the number of PRBs for each SCS and CBW combination when transform precoding is enabled as:

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 100 | 64 | 256 | N/A | N/A | N/A |
| 480 | N/A | 64 | 120 | 240 | N/A |
| 960 | N/A | 32 | 60 | 120 | 144 |

Channel bandwidth for requirements definition

During the last meeting the following was agreed on maximum BW [1]:

Issue 2-1-2: CBW for UL requirements definition Consider minimum CBW for each SCS for UL requirements definition. Consider maximum CBW for each SCS for UL requirements definition if no testability aspects are identified - Perform link budget analysis to study max achievable SNR during the test

A concern was raised regarding testability of wider CBW, and it was proposed to perform link budget analysis to evaluate the maximum achievable SNR during the test. A similar discussion had taken place for the definition of the 47 GHz demodulation requirements.  In the case of 47 GHz requirements, the potential problems of unrealistically large demodulator input power and limited TE output power were solved by allowing choice of the AWGN_offset during the test as documented in the WF [3]:

- Test feasibility - RAN4 assumes OTA link budget as sufficient if AWGN_offset can be chosen. - Add the following note in the Tables ‘AWGN power level at the BS input’ for BS type 2-O in TS38.141-2 at least from Rel-17 specification in NR_47GHz_band WI: - Note X: The AWGN power level contains an AWGN offset of 15dB. If needed for test purposes, the AWGN level can be reduced by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level.

We also expect that the same AWGN_offset reduction opportunities from 47GHz are maintained for 71GHz, hence we propose:

RAN4 to define demodulation requirements for FR2-2 including opportunity for AWGN_offset reduction as specified for FR2-1.

RAN4 to define demodulation requirements for 400 MHz CBW, 1600 MHz for 480 kHz SCS,  and 2000 MHz for 960 kHz SCS.

Conclusion

This paper has presented Nokia’s views on PUSCH demodulation requirements for the extension to 71 GHz. From this discussion we have derived the following observations and proposals:

- Latest RAN4 agreements on number of PRBs cannot be used for transform-precoding test cases for which the number of PRBs have to be calculated considering powers of 2, 3, and 5 only.

- Adopt the number of PRBs for each SCS and CBW combination when transform precoding is enabled as:

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 100 | 64 | 256 | N/A | N/A | N/A |
| 480 | N/A | 64 | 120 | 240 | N/A |
| 960 | N/A | 32 | 60 | 120 | 144 |

RAN4 to define demodulation requirements for FR2-2 including opportunity for AWGN_offset reduction as specified for FR2-1.

RAN4 to define demodulation requirements for 400 MHz CBW, 1600 MHz for 480 kHz SCS,  and 2000 MHz for 960 kHz SCS.

References

- R4-2207223, WF on demodulation performance requirements definition for 52.6 - 71 GHz, Intel Corporation

- R4-2210664, WF on general and BS aspects for FR2-2 demodulation requirements, Intel Corporation

- R4-2106781, On 47GHz OTA BS demodulation testing, Nokia, Nokia Shanghai Bell

- R4-2210577, WF on system parameters for NR extension to 71GHz, Intel Corporation

- TS 38.101-2, NR, User equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone