3GPP TSG-RAN WG4 Meeting # 104-e                	R4-2212676

Electronic Meeting, August 15th to 26th, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on PUCCH demodulation requirements for the extension to 71 GHz

Agenda item: 	9.14.8.3.2

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on PUCCH demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

Discussion

General

On the last RAN4 meeting, general agreements were made for each PUCCH format including number of PRBs and OFDM symbols, as follow [2]:

Updated Issue 2-3-1: How many PRBs and symbols are considered for each of the PUCCH format? Introduce below test cases - For Format 0: - 1RB /1 symbol - FFS for [10/16] RB/1 symbol - For format 1: - 1 RB/14 symbols - [10/16] RB/14 symbols - For format 2: Reusing FR2-1 - For format 3: Reusing FR2-1 - For format 4: - 1 RB/14 symbols - [10/16] RB/14 symbols Agreed Issue 2-3-4: Parameter list for PUCCH performance requirements (Agreements in the last meeting) Introduce below test cases - For Format 0: - Intra-slot frequency hopping: enabled if number of OFDM symbols higher than 1 - Test metric: ACK missed detection rate < 1% - For format 1: - Intra-slot frequency hopping: enabled - Test metric: ACK missed detection rate < 1% and NACK to ACK probability < 0.1% - For format 2: - Intra-slot frequency hopping: enabled - Test metric: ACK missed detection rate < 1%, UCI BLER < 1% - For format 3: - Intra-slot frequency hopping: enabled - Test metric: UCI BLER < 1% - For format 4: - Intra-slot frequency hopping: enabled - Test metric: UCI BLER < 1%

The general channel model related agreements are:

Issue 1-3-2: RMS delay spread TDL channel model: - Option 1: TDL-A - Option 2: TDL-D RMS Delay spread: - Option 1: 5ns - Option 2: 10ns - Option 3: 20ns Note: Definition of requirements with different channel models is not precluded Issue 1-3-3: Max Doppler frequency Consider 3 km/h UE speed ( 200 Hz ). FFS on higher UE speed. - Option 1: 10 km/h (650 Hz) - Option 2: 30 km/h (2000 Hz)

Additionally, no specific channel model and delay spread agreement was reached for PUCCH formats. As part of the general discussion on channel model, there was a tendency to prefer 10 ns delay spread, since with 5 ns delay spread the required tap resolution would be small in comparison to the feasible delay resolution that were suggested by TE vendors. Additionally, FR2-2 is expected to operate with low mobility, therefore we believe 650 Hz doppler frequency is representative of typical scenario for the operation in FR2-2.

RAN4 to define PUCCH demodulation requirements using TDL-A 10-650 Low channel model.

The agreements from the last RAN4 meeting included use of 1 PRB and either 10 or 16 PRBs for the enhanced PUCCH formats 0, 1, and 4. In our view only either 10 or 16 PRBs are needed for the performance requirements. Our companion paper with simulation results shows that there is a difference of about 1.6 to 2.3 dB for PF0, 1.5 to 1.9 dB for PF1, and 1.5 to 2 dB for PF4 in demodulation performance when comparing 10 PRBs and 16 PRBs. IN both cases the results are much more robust than the 1 PRBs results, with a improvement abode 9 dBs when comparing single PRB and 10 PRBs.

Enhanced PUCCH formats provide SNR gains in the range of 9 to 10 dBs in comparison to single PRB PUCCH formats 0, 1, and 4

RAN4 to define requirements for enhanced PUCCH formats 0, 1, and 4 with 10 PRBs and 1 OFDM symbol.

PUCCH format 0

RAN4 to define enhanced PUCCH format 0 requirements with 10 PRBs, and 1 OFDM symbol.

RAN4 to adopt the following parameters for enhanced PUCCH format 0 simulations:

| Parameter | Value |
| --- | --- |
| TR/RX | 1T2R |
| Number of UCI information bits | 1 |
| Number of PRBs and Number of OFDM symbols: | Number of PRBs: 1 and 10 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol |
| Propagation condition | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | Test metric: ACK missed detection rate < 1% |

PUCCH format 1

RAN4 to define enhanced PUCCH format 1 requirements with 10 PRBs, and 2 UCI information bits.

RAN4 to adopt the following table of parameters for PUCCH format 1:

| Parameter | Value |
| --- | --- |
| TR/RX | 1T2R |
| Number of UCI information bits | 2 |
| Number of PRBs and Number of OFDM symbols: | Number of PRBs: 1 and 10 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Propagation condition | TDL channel model: TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | ACK missed detection rate < 1% NACK to ACK (0.1 %) |

PUCCH format 2

In the previous meeting we agreed to reuse FR2-1 configuration as baseline for PUCCH format 2 requirements. As part of these, it is the use of 1 configuration with 4 PORBs and another with 9 PRBs.

RAN4 to define PUCCH format 2 requirements with the following 2 cases
      -Case 1 with 4 PRBs, 1 OFDM symbol and 4 UCI bits, 
      -Case 2 with 9 PRBs, 2 OFDM symbols and 22 UCI bits

RAN4 to adopt the following table of parameters for PUCCH format 2:

| Parameter | Case 1 | Case 2 |
| --- | --- | --- |
| TR/RX | 1T2R | 1T2R |
| Modulation order | QPSK | QPSK |
| Number of UCI information bits | 4 | 22 |
| Number of PRBs | 4 | 9 |
| OFDM symbols | 1 | 2 |
| First PRB prior to frequency hopping | 0 | 0 |
| Intra-slot frequency hopping | Disabled | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither | Neither |
| Hopping ID | 0 | 0 |
| Initial cyclic shift | 0 | 0 |
| First symbol | 13 | 12 |
| Propagation condition | TDL-A 10-650 | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | ACK missed detection rate < 1% | UCI BLER < 1% |

PUCCH format 3

In the previous meeting we agreed to reuse FR2-1 configuration as baseline for PUCCH format 3 requirements. As part of these, it is the use of 1 configuration with 4 PORBs and another with 9 PRBs.

RAN4 to define PUCCH format 3 requirements with the following 2 cases 
      -Case 1 with 1 PRBs, 14 OFDM symbols and 16 UCI bits, 
      -Case 2 with 3 PRBs, 4 OFDM symbols and 16 UCI bits

RAN4 to define PUCCH format 3 requirements with and without additional DMRs for test case with 14 OFDM symbols.

RAN4 to adopt the following table of parameters for PUCCH format 3:

| Parameter | value |
| --- | --- |
| Number of UCI information bits | 16 bits |
| Number of PRBs Number of OFDM symbols: | 1 for 14 OFDM symbols |
|  | and |
|  | 3 for 4 OFDM symbols |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| First symbol | 13 |
| Modulation order | QPSK |
| DM-RS sequence generation | NID0=0 |
| Additional DMRS | With and without for 14 OFDM symbols |
|  | Without for 4 OFDM symbols |
| Propagation condition | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz and 1600 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | UCI BLER < 1% |

PUCCH format 4

RAN4 to define enhanced PUCCH format 4 requirements with 10 PRBs, and 22 UCI information bits.

RAN4 to adopt the following table of parameters for PUCCH format 4:

| Parameter | Value |
| --- | --- |
| Number of UCI information bits | 22 bits |
| Number of PRBs | 1 and 10 (Enhanced) |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| First symbol | 0 |
| Number of OFDM symbols: | 14 |
| Additional DMRS | With and without |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Propagation condition | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz and 1600 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | UCI BLER < 1% |

Conclusion

This paper has presented Nokia’s views on PUSCH demodulation requirements for the extension to 71 GHz. From this discussion we have derived the following observations and proposals:

- RAN4 to define PUCCH demodulation requirements using TDL-A 10-650 Low channel model.

Enhanced PUCCH formats provide SNR gains in the range of 9 to 10 dBs in comparison to single PRB PUCCH formats 0, 1, and 4

RAN4 to define requirements for enhanced PUCCH formats 0, 1, and 4 with 10 PRBs and 1 OFDM symbol.

RAN4 to define enhanced PUCCH format 0 requirements with 10 PRBs, and 1 OFDM symbol.

RAN4 to adopt the following parameters for enhanced PUCCH format 0 simulations:

| Parameter | Value |
| --- | --- |
| TR/RX | 1T2R |
| Number of UCI information bits | 1 |
| Number of PRBs and Number of OFDM symbols: | Number of PRBs: 1 and 10 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol |
| Propagation condition | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | Test metric: ACK missed detection rate < 1% |

RAN4 to define enhanced PUCCH format 1 requirements with 10 PRBs, and 2 UCI information bits.

RAN4 to adopt the following table of parameters for PUCCH format 1:

| Parameter | Value |
| --- | --- |
| TR/RX | 1T2R |
| Number of UCI information bits | 2 |
| Number of PRBs and Number of OFDM symbols: | Number of PRBs: 1 and 10 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Propagation condition | TDL channel model: TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | ACK missed detection rate < 1% NACK to ACK (0.1 %) |

RAN4 to define PUCCH format 2 requirements with the following 2 cases
      -Case 1 with 4 PRBs, 1 OFDM symbol and 4 UCI bits, 
      -Case 2 with 9 PRBs, 2 OFDM symbols and 22 UCI bits

RAN4 to adopt the following table of parameters for PUCCH format 2:

| Parameter | Case 1 | Case 2 |
| --- | --- | --- |
| TR/RX | 1T2R | 1T2R |
| Modulation order | QPSK | QPSK |
| Number of UCI information bits | 4 | 22 |
| Number of PRBs | 4 | 9 |
| OFDM symbols | 1 | 2 |
| First PRB prior to frequency hopping | 0 | 0 |
| Intra-slot frequency hopping | Disabled | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither | Neither |
| Hopping ID | 0 | 0 |
| Initial cyclic shift | 0 | 0 |
| First symbol | 13 | 12 |
| Propagation condition | TDL-A 10-650 | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | ACK missed detection rate < 1% | UCI BLER < 1% |

RAN4 to define PUCCH format 3 requirements with the following 2 cases 
      -Case 1 with 1 PRBs, 14 OFDM symbols and 16 UCI bits, 
      -Case 2 with 3 PRBs, 4 OFDM symbols and 16 UCI bits

RAN4 to define PUCCH format 3 requirements with and without additional DMRs for test case with 14 OFDM symbols.

RAN4 to adopt the following table of parameters for PUCCH format 3:

| Parameter | value |
| --- | --- |
| Number of UCI information bits | 16 bits |
| Number of PRBs Number of OFDM symbols: | 1 for 14 OFDM symbols |
|  | and |
|  | 3 for 4 OFDM symbols |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| First symbol | 13 |
| Modulation order | QPSK |
| DM-RS sequence generation | NID0=0 |
| Additional DMRS | With and without for 14 OFDM symbols |
|  | Without for 4 OFDM symbols |
| Propagation condition | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz and 1600 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | UCI BLER < 1% |

RAN4 to define enhanced PUCCH format 4 requirements with 10 PRBs, and 22 UCI information bits.

RAN4 to adopt the following table of parameters for PUCCH format 4:

| Parameter | Value |
| --- | --- |
| Number of UCI information bits | 22 bits |
| Number of PRBs | 1 and 10 (Enhanced) |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| First symbol | 0 |
| Number of OFDM symbols: | 14 |
| Additional DMRS | With and without |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Propagation condition | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz and 1600 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | UCI BLER < 1% |

References

- R4-2207223, WF on demodulation performance requirements definition for 52.6 - 71 GHz, Intel Corporation

- R4-2210664, WF on general and BS aspects for FR2-2 demodulation requirements, Intel Corporation