3GPP TSG-RAN WG4 Meeting # 104-e                	R4-2212677

Electronic Meeting, August 15th to 26th, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PUCCH simulation results for the extension to 71 GHz

Agenda item: 	9.14.8.3.2

Document for: 	Discussion

Introduction

This paper presents PUCCH simulation results for the definition of demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

Simulation assumptions

Simulation assumptions are shown below and are derived considering the agreements on the WF [1].

PUCCH format 0

Table 1 Simulation assumption for PUCCH format 0

| Parameter | Value |
| --- | --- |
| TR/RX | 1T2R |
| Number of UCI information bits | 1 |
| Number of PRBs and Number of OFDM symbols: | Number of PRBs: 1, 10 and 16 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol |
| Propagation condition | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | Test metric: ACK missed detection rate < 1% |
| Phase noise model | TR 38.808 set 2 |

Table 4 Results for PUCCH format 0: SNR at 1% ACK missed detection rate

| PUCCH format | SCS | CBW | Number symbols | Number RBs | Info Bits | Test metric | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 120 | 100 | 1 | 1 | 1 | ACK missed detection rate < 1% | 7.85 |
| 0 | 120 | 100 | 1 | 10 | 1 | ACK missed detection rate < 1% | -2 |
| 0 | 120 | 100 | 1 | 16 | 1 | ACK missed detection rate < 1% | -4.3 |
| 0 | 480 | 400 | 1 | 1 | 1 | ACK missed detection rate < 1% | 7.89 |
| 0 | 480 | 400 | 1 | 10 | 1 | ACK missed detection rate < 1% | -3.75 |
| 0 | 480 | 400 | 1 | 16 | 1 | ACK missed detection rate < 1% | -5.34 |
| 0 | 960 | 400 | 1 | 1 | 1 | ACK missed detection rate < 1% | 8.21 |
| 0 | 960 | 400 | 1 | 10 | 1 | ACK missed detection rate < 1% | -3.73 |
| 0 | 960 | 400 | 1 | 16 | 1 | ACK missed detection rate < 1% | -5.98 |

PUCCH format 1

Table 5 Simulation assumption for PUCCH format 1

| Parameter | Value |
| --- | --- |
| TR/RX | 1T2R |
| Number of UCI information bits | 2 |
| Number of PRBs | 1, 10 and 16 |
| Number of OFDM symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Propagation condition | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | ACK missed detection rate < 1% NACK to ACK (0.1 %) |
| Phase noise model | TR 38.808 set 2 |

Table 6 Results for PUCCH format 1

| PUCCH format | SCS | CBW | Number symbols | Number RBs | Info Bits | Test metric | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 120 | 100 | 14 | 1 | 2 | ACK missed detection rate < 1% | -7.4 |
|  |  |  |  |  |  | NACK to ACK (0.1 %) | -5.1 |
|  |  |  |  | 10 | 2 | ACK missed detection rate < 1% | -7.3 |
|  |  |  |  |  |  | NACK to ACK (0.1 %) | -5.4 |
|  |  |  |  | 16 | 2 | ACK missed detection rate < 1% | -7.1 |
|  |  |  |  |  |  | NACK to ACK (0.1 %) | -5.1 |
| 1 | 480 | 400 | 14 | 1 | 2 | ACK missed detection rate < 1% | -14.8 |
|  |  |  |  |  |  | NACK to ACK (0.1 %) | -13.6 |
|  |  |  |  | 10 | 2 | ACK missed detection rate < 1% | -15.5 |
|  |  |  |  |  |  | NACK to ACK (0.1 %) | -14.4 |
|  |  |  |  | 16 | 2 | ACK missed detection rate < 1% | -15.5 |
|  |  |  |  |  |  | NACK to ACK (0.1 %) | -14.5 |
| 1 | 960 | 400 | 14 | 1 | 2 | ACK missed detection rate < 1% | -16.5 |
|  |  |  |  |  |  | NACK to ACK (0.1 %) | -15.5 |
|  |  |  |  | 10 | 2 | ACK missed detection rate < 1% | -17 |
|  |  |  |  |  |  | NACK to ACK (0.1 %) | -16.1 |
|  |  |  |  | 16 | 2 | ACK missed detection rate < 1% | -16.9 |
|  |  |  |  |  |  | NACK to ACK (0.1 %) | -15.9 |

PUCCH format 2

Table 7 Simulation assumption for PUCCH format 2

| Parameter | Value |
| --- | --- |
| TR/RX | 1T2R |
| Modulation order | QPSK |
| Number of UCI information bits | 4 |
| Number of PRBs | 9 and 4 |
| OFDM symbols | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Propagation condition | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | ACK missed detection rate < 1% UCI BLER < 1% |
| Phase noise model | TR 38.808 set 2 |

Table 8 Simulation results for PUCCH format 2

| PUCCH format | SCS | CBW | Number symbols | Number RBs | Info Bits | Test metric | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 120 | 100 | 1 | 4 | 4 | ACK missed detection rate < 1% | 6.2 |
| 2 | 120 | 100 | 2 | 9 | 22 | UCI BLER < 1% | 1 |
| 2 | 480 | 400 | 1 | 4 | 4 | ACK missed detection rate < 1% | 4 |
| 2 | 480 | 400 | 2 | 9 | 22 | UCI BLER < 1% | -0.7 |
| 2 | 960 | 400 | 1 | 4 | 4 | ACK missed detection rate < 1% | 3.7 |
| 2 | 960 | 400 | 2 | 9 | 22 | UCI BLER < 1% | 0.1 |

PUCCH format 3

Table 9 Simulation assumptions for PUCCH format 3

| Parameter | value |
| --- | --- |
| Number of UCI information bits | 16 bits |
| Number of PRBs Number of OFDM symbols: | 1 for 14 OFDM symbols |
|  | and |
|  | 3 for 4 OFDM symbols |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| First symbol | 13 |
| Modulation order | QPSK |
| DM-RS sequence generation | NID0=0 |
| Additional DMRS | With and without for 14 14 OFDM symbols |
|  | Without for 4 OFDM symbols |
| Propagation condition | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz and 1600 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | UCI BLER < 1% |
| Phase noise model | TR 38.808 set 2 |

Table 10 Results for PUCCH format 3: SNR at 1% BLER

| PUCCH format | SCS | CBW | Number symbols | Number RBs | Info Bits | Additional DMRS | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 120 | 100 | 14 | 1 | 16 | No | -1.6 |
|  |  |  |  |  |  | Yes | -1.1 |
|  |  |  | 4 | 3 | 16 | No | -0.8 |
| 3 | 480 | 400 | 14 | 1 | 16 | No | -1.9 |
|  |  |  |  |  |  | Yes | -1.4 |
|  |  |  | 4 | 3 | 16 | No | -1.3 |
| 3 | 960 | 400 | 14 | 1 | 16 | No | -1.9 |
|  |  |  |  |  |  | Yes | -1.2 |
|  |  |  | 4 | 3 | 16 | No | -1.4 |

PUCCH format 4

| Parameter | Value |
| --- | --- |
| Number of UCI information bits | 22 bits |
| Number of PRBs | 1 10 and 16 (Enhanced) |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| First symbol | 0 |
| Number of OFDM symbols: | 14 |
| Additional DMRS | With and without |  |
| Length of the orthogonal cover code | n2 |  |
| Index of the orthogonal cover code | n0 |  |
| Propagation condition | TDL-A 10-650 |  |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz and 1600 MHz SCS = 960 kHz; CBW = 400 MHz |  |
| Testing metric | UCI BLER < 1% |  |
| Phase noise model | TR 38.808 set 2 |  |

Table 8 Results for PUCCH format 4: SNR at 1% BLER

| PUCCH format | SCS | CBW | Number symbols | Number RBs | Info Bits | Additional DMRS | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 120 | 100 | 14 | 1 | 22 | No | 0.2 |
|  |  |  |  |  |  | Yes | 0.7 |
|  |  |  |  | 10 | 22 | No | -0.1 |
|  |  |  |  |  |  | Yes | 0.5 |
|  |  |  |  | 16 | 22 | No | 0.1 |
|  |  |  |  |  |  | Yes | 0.6 |
| 4 | 480 | 400 | 14 | 1 | 22 | No | -9.6 |
|  |  |  |  |  |  | Yes | -9.3 |
|  |  |  |  | 10 | 22 | No | -10.5 |
|  |  |  |  |  |  | Yes | -10.1 |
|  |  |  |  | 16 | 22 | No | -10.3 |
|  |  |  |  |  |  | Yes | -9.9 |
| 4 | 960 | 400 | 14 | 1 | 22 | No | -11.6 |
|  |  |  |  |  |  | Yes | -11.2 |
|  |  |  |  | 10 | 22 | No | -12.1 |
|  |  |  |  |  |  | Yes | -11.7 |
|  |  |  |  | 16 | 22 | No | -11.8 |
|  |  |  |  |  |  | Yes | -11.4 |

References

- R4-2210664, WF on general and BS aspects for FR2-2 demodulation requirements, Intel Corporation

- R4-2207223, WF on demodulation performance requirements definition for 52.6 - 71 GHz, Intel Corporation