3GPP TSG-RAN WG4 Meeting # 104-e                	R4-2212678

Electronic Meeting, August 15 – 26, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on PRACH demodulation requirements for the extension to 71 GHz

Agenda item: 	9.14.8.3.3

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on PRACH demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

Discussion

Channel model

The general channel model related agreements  reached in the previous RAN4 meetings are 5:

Issue 1-3-2: RMS delay spread TDL channel model: - Option 1: TDL-A - Option 2: TDL-D RMS Delay spread: - Option 1: 5ns - Option 2: 10ns - Option 3: 20ns Note: Definition of requirements with different channel models is not precluded Issue 1-3-3: Max Doppler frequency Consider 3 km/h UE speed ( 200 Hz ). FFS on higher UE speed. - Option 1: 10 km/h (650 Hz) - Option 2: 30 km/h (2000 Hz)

No specific channel model and delay spread agreement was reached for PRACH formats. As part of the general discussion on channel model, there was a tendency to prefer 10 ns delay spread, since with 5 ns delay spread the required tap resolution would be small in comparison to the feasible delay resolution that were suggested by TE vendors. Additionally, FR2-2 is expected to operate with low mobility, therefore we believe 650 Hz doppler frequency is representative of typical scenario for the operation in FR2-2.

RAN4 to define PRACH demodulation requirements using AWGN and TDL-A 10-650 channel model.

Ncs value

During the last meeting, the following options for NCS were discussed  with the tentative agreement as in [1]:

Issue 2-5-8: (Ncs, logical sequence index, v) - Consider the following PRACH parameters as starting point: Ncs = 69, logical sequence index = 0, v =0 as starting point - Other options are not precluded

From our point of view, it is reasonable to assume that the NCS is chosen such considering the equivalent cell size. For this reason, we calculate the maximum detectable timing offset based on NCS. Considering that:

- for each root Zadoff-Chu sequence the number of cyclically shifted sequences that can be obtained is

NSEQ=LRANCS

- that the sequence length for the short preamble format C0 is

Nu=2048∙κ∙2-μ

- the maximum detectable time offset can be calculated as

TMAX=NuNSEQTc

Additionally, the values of Ncs can be chosen from Table 6.3.3.1-7 of 38.211 [3]:

Table 6.3.3.1-7: for preamble formats with LRA∈139, 571, 1151. zeroCorrelationZoneConfig, msgA-ZeroCorrelationZoneConfig; NCS value. LRA=139; LRA=571; LRA=1151. 0; 0; 0; 0. 1; 2; 8; 17. 2; 4; 10; 21. 3; 6; 12; 25. 4; 8; 15; 30. 5; 10; 17; 35. 6; 12; 21; 44. 7; 13; 25; 52. 8; 15; 31; 63. 9; 17; 40; 82. 10; 19; 51; 104. 11; 23; 63; 127. 12; 27; 81; 164. 13; 34; 114; 230. 14; 46; 190; 383. 15; 69; 285; 575.

Considering the formulas above, the maximum detectable timing offset can be calculated as shown in Table 1 for the existing Rel 15 and Rel 16 requirements. In this table, the values of Ncs are chosen in accordance to 38.104. The results from the table show similar cell size for all SCS and LRA options of FR1. For FR2 the Tmax decreases with the SCS since all SCSs already use the Ncs value that provides larger Tmax, i.e. Ncs = 69.

Therefore, we understand that we can chose the same approach as the one used in FR2-1, where Ncs is chosen to provide good coverage are using zeroCorrelationZoneConfig = 15. The calculated TMAX in that case is as shown in Table 1 for FR1 in Rel15 + NRU and FR2-1 and in Table 2 with the proposed Ncs value.

Table 1 Calculated maximum timing offset for existing PRACH requirements

|  | FR1 (Rel 15 + Rel 16 NR-U) |  |  |  | FR2-1 (Rel 15) |  |
| --- | --- | --- | --- | --- | --- | --- |
| LRA | 139 | 139 | 1151 | 571 | 139 | 139 |
| NCS | 23 | 46 | 164 | 190 | 69 | 69 |
| SCS | 15 | 30 | 15 | 30 | 60 | 120 |
| μ | 0 | 1 | 0 | 1 | 2 | 3 |
| κ | 64 | 64 | 64 | 64 | 64 | 64 |
| Tc | 0.509 | 0.509 | 0.509 | 0.509 | 0.509 | 0.509 |
| Nu | 131072 | 65536 | 131072 | 65536 | 32768 | 16384 |
| NSEQ | 6 | 3 | 7 | 3 | 2 | 2 |
| TMAX (us) | 11.1 | 11.1 | 9.5 | 11.1 | 8.3 | 4.2 |

Table 2 Calculated maximum timing offset for the proposed Ncs PRACH configuration

|  | FR2-2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| LRA | 139 | 571 | 1151 | 139 | 571 | 139 |
| NCS | 69 | 285 | 575 | 69 | 285 | 69 |
| SCS | 120 | 120 | 120 | 480 | 480 | 960 |
| μ | 3 | 3 | 3 | 5 | 5 | 6 |
| κ | 64 | 64 | 64 | 64 | 64 | 64 |
| Tc | 0.509 | 0.509 | 0.509 | 0.509 | 0.509 | 0.509 |
| Nu | 16384 | 16384 | 16384 | 4096 | 4096 | 2048 |
| NSEQ | 2 | 2 | 2 | 2 | 2 | 2 |
| TMAX (us) | 4.2 | 4.2 | 4.2 | 1.0 | 1.0 | 0.5 |

The NCS parameters used for FR2 requirements (Ncs=69) are the ones that provide the maximum coverage area for LRA=139.

The LRA=139 and Ncs=69 provide a similar coverage area to LRA=571 and Ncs=285 and LRA=1151 and Ncs=575.

RAN4 to define PRACH demodulation performance requirements using the following Ncs parameters:
      -SCS=120, LRA=139, NCS=69
      -SCS=120, LRA=571, NCS=285
      -SCS=120, LRA=1151, NCS=575
      -SCS=480, LRA=139, NCS=69
      -SCS=480, LRA=571, NCS=285
      -SCS=960, LRA=139, NCS=69

PRACH maximum detectable offset

As a part of the PRACH conformance testing, a timing offset scheme is used as shown in Figure 1. This scheme is used for all existing PRACH requirements, including SCS up to 120 kHz. This scheme uses a maximum offset of 0.8 us [4], which is fine for the Rel 15 SCSs, since for 120 kHz the maximum detectable offset is TMAX=4.2 us as indicated in Table 1. However, 0.8 us is very close to the achievable limit for 480 kHz, TMAX=1.0 us and above the limit for 960 kHz SCS, TMAX=1.0 us.

Figure 1 Timing offset scheme for PRACH preamble format A1 A2, A3, B4, C0 and C2 [3]

The achievable maximum detectable timing offset for 480 kHz and 960 kHz SCS is 1.0 us and 0.5 us respectively.

The Rel 15 requirements for PRACH include timing offset scheme with offset up to 0.8 us, which exceeds the maximum detectable offset for 960 kHz SCS.

RAN4 to specify PRACH performance with timing offset up to 0.4 us for 480 kHz and 960 kHz SCS.

Time estimation tolerance

As part of the PRACH performance requirements the time error tolerance is specified. The time error tolerance is determined based on the format SCS, the sequence length, as well as the channel model.

The existing requirements for Rel. 15 PRACH are shown in Table 3. The requirements for FR1 and FR2-1 are built based on the requirements for AWGN with an additional term that corresponds to the second last tap of the channel in Table G.2.1.1-4 of 38.141-2 [4]. The first term for the calculation is the PRACH minimum time resolution, which is obtained in accordance to the numerology μ and LRA as:

∆RA=2048∙κ∙2-μLRATc

For Rel. 15 short PRACH sequences and LRA=139 this formulas results in a value slightly smaller than TAC resolution. As the nal requirement, this time is approximated to multiples of the TA command resolution. The timing advance step can be derived from the description in the clause 4.2 of 38.213 as

∆TA=16∙64∙2-μTc

Some examples are shown in Table 3 for ∆RA and ∆TA. Next, the time for the second last tap of the channel model is summed to the first result. For TDLC300 that time is 1.51 us. Both values combined result in the current Rel. 15 NR PRACH sequences as shown in Table 3.

Table 3 Analysis of PRACH time error tolerance for Rel 15 and Rel 16 requirements

| SCS | Channel model | LRA | Min PRACH resolution (ns) ∆RA | TAC resolution (ns) ∆TA | 11th tap (ns) | TAC resol + 11th tap | Time error tolerance (us) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | AWGN | 139 | 479.6 | 520.8 | 0 | 520.8 | 0.52 |
|  | TDLC300-100 | 139 | 479.6 | 520.8 | 1510 | 2030.8 | 2.03 |
|  | TDLA30-10 | 1151 | 57.9 | 520.8 | 150 | 670.8 | 0.67 |
| 30 | AWGN | 139 | 239.8 | 260.4 | 0 | 260.4 | 0.26 |
|  | TDLC300-100 | 139 | 239.8 | 260.4 | 1510 | 1770.4 | 1.77 |
|  | TDLA30-10 | 571 | 58.4 | 260.4 | 150 | 410.4 | 0.41 |
| 60 | AWGN | 139 | 119.9 | 130.2 | 0 | 130.2 | 0.13 |
|  | TDLA30-300 | 139 | 119.9 | 130.2 | 150 | 280.2 | 0.28 |
| 120 | AWGN | 139 | 60.0 | 65.1 | 0 | 65.1 | 0.07 |
|  | TDLA30-300 | 139 | 60.0 | 65.1 | 150 | 215.1 | 0.22 |

A similar approach can be used for the channel models and SCS used in FR2-2. Some proposed values are shown in

Table 4 Analysis of PRACH time error tolerance for Rel 17 FR2-2 requirements

| SCS | Channel model | LRA | Min PRACH resolution (ns) ∆RA | TAC resolution (ns) ∆TA | 11th tap (ns) | TAC resol + 11th tap | Time error tolerance (ns) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | AWGN | 139 | 60.0 | 65.1 | 0 | 65.1 | 70 |
|  | AWGN | 571 | 14.6 | 65.1 | 0 | 65.1 | 70 |
|  | AWGN | 1151 | 7.2 | 65.1 | 0 | 65.1 | 70 |
|  | TDLA10-650 | 139 | 60.0 | 65.1 | 50 | 115.1 | 120 |
|  | TDLA10-650 | 571 | 14.6 | 65.1 | 50 | 115.1 | 120 |
|  | TDLA10-650 | 1151 | 7.2 | 65.1 | 50 | 115.1 | 120 |
| 480 | AWGN | 139 | 15.0 | 16.3 | 0 | 16.3 | 18 |
|  | AWGN | 571 | 3.6 | 16.3 | 0 | 16.3 | 18 |
|  | TDLA10-650 | 139 | 15.0 | 16.3 | 50 | 66.3 | 68 |
|  | TDLA10-650 | 571 | 3.6 | 16.3 | 50 | 66.3 | 68 |
| 960 | AWGN | 139 | 7.5 | 8.1 | 0 | 8.1 | 9 |
|  | TDLA10-650 | 139 | 7.5 | 8.1 | 50 | 58.1 | 59 |

Additionally, as part of the NR-U demod requirements, it was discussed whether the time error tolerance should also consider the improved time accuracy of sequences with LRA=571 and LRA=1151. As shown in Table 2, the ∆RA is improved with LRA. However there is little benefit to decrease the PRACH time error resolution below the TAC quantization, since this will not translate anyway in a clear benefit for the UE accuracy. Therefore it was decided for NR-U to use the TAC resolution as a baseline for the Time Error tolerance of requirements with LRA=1151 and LRA=571.

Rel 16 NR-U requirements are defined such that PRACH time error tolerance is larger or equal to TAC resolution.

Not to define time error tolerance that is smaller than the minimum possible step for the timing advance command.

RAN4 to define time error tolerance of PRACH timing requirements as:

| SCS | Channel model | Time error tolerance (ns) |
| --- | --- | --- |
| 120 | AWGN | 70 |
|  | TDLA10-650Note 1 | [120]Note 1 |
| 480 | AWGN | 18 |
|  | TDLA10-650 | [68] Note 1 |
| 960 | AWGN | 9 |
|  | TDLA10-650 | [59] Note 1 |
| NOTE 1: To be revised after definition of channel taps used for TDLA 10. |  |  |

Test cases table for PRACH simulations

Considering the discussion above, we propose the following test cases for RAN4.

Use the table below for the test cases and parameters related to PRACH demodulation requirements:

| Format | SCS | Prach sequence length | Ncs | Propagation condition | Frequency Offset (Hz) | Time estimation tolerance |
| --- | --- | --- | --- | --- | --- | --- |
| A2 | 120kHz | 139 | 69 | AWGN | 0 | 70 ns |
| B4 | 120kHz | 139 | 69 | AWGN | 0 | 70 ns |
| C2 | 120kHz | 139 | 69 | AWGN | 0 | 70 ns |
| A2 | 120kHz | 139 | 69 | TDLA10-650 | 7100 | 120 ns |
| B4 | 120kHz | 139 | 69 | TDLA10-650 | 7100 | 120 ns |
| C2 | 120kHz | 139 | 69 | TDLA10-650 | 7100 | 120 ns |
| A2 | 120kHz | 571 | 285 | AWGN | 0 | 70 ns |
| B4 | 120kHz | 571 | 285 | AWGN | 0 | 70 ns |
| C2 | 120kHz | 571 | 285 | AWGN | 0 | 70 ns |
| A2 | 120kHz | 571 | 285 | TDLA10-650 | 7100 | 120 ns |
| B4 | 120kHz | 571 | 285 | TDLA10-650 | 7100 | 120 ns |
| C2 | 120kHz | 571 | 285 | TDLA10-650 | 7100 | 120 ns |
| A2 | 120kHz | 1151 | 575 | AWGN | 0 | 70 ns |
| B4 | 120kHz | 1151 | 575 | AWGN | 0 | 70 ns |
| C2 | 120kHz | 1151 | 575 | AWGN | 0 | 70 ns |
| A2 | 120kHz | 1151 | 575 | TDLA10-650 | 7100 | 120 ns |
| B4 | 120kHz | 1151 | 575 | TDLA10-650 | 7100 | 120 ns |
| C2 | 120kHz | 1151 | 575 | TDLA10-650 | 7100 | 120 ns |
| A2 | 480kHz | 139 | 69 | AWGN | 0 | 18 ns |
| B4 | 480kHz | 139 | 69 | AWGN | 0 | 18 ns |
| C2 | 480kHz | 139 | 69 | AWGN | 0 | 18 ns |
| A2 | 480kHz | 139 | 69 | TDLA10-650 | 7100 | 68 ns |
| B4 | 480kHz | 139 | 69 | TDLA10-650 | 7100 | 68 ns |
| C2 | 480kHz | 139 | 69 | TDLA10-650 | 7100 | 68 ns |
| A2 | 480kHz | 571 | 285 | AWGN | 0 | 18 ns |
| B4 | 480kHz | 571 | 285 | AWGN | 0 | 18 ns |
| C2 | 480kHz | 571 | 285 | AWGN | 0 | 18 ns |
| A2 | 480kHz | 571 | 285 | TDLA10-650 | 7100 | 68 ns |
| B4 | 480kHz | 571 | 285 | TDLA10-650 | 7100 | 68 ns |
| C2 | 480kHz | 571 | 285 | TDLA10-650 | 7100 | 68 ns |
| A2 | 960kHz | 139 | 69 | AWGN | 0 | 9 ns |
| B4 | 960kHz | 139 | 69 | AWGN | 0 | 9 ns |
| C2 | 960kHz | 139 | 69 | AWGN | 0 | 9 ns |
| A2 | 960kHz | 139 | 69 | TDLA10-650 | 7100 | 59 ns |
| B4 | 960kHz | 139 | 69 | TDLA10-650 | 7100 | 59 ns |
| C2 | 960kHz | 139 | 69 | TDLA10-650 | 7100 | 59 ns |

Conclusion

This paper has presented Nokia’s views on PRACH demodulation requirements for the extension to 71 GHz. From this discussion we have derived the following observations and proposals:

- RAN4 to define PRACH demodulation requirements using AWGN and TDL-A 10-650 channel model.

- The NCS parameters used for FR2 requirements (Ncs=69) are the ones that provide the maximum coverage area for LRA=139.

The LRA=139 and Ncs=69 provide a similar coverage area to LRA=571 and Ncs=285 and LRA=1151 and Ncs=575.

RAN4 to define PRACH demodulation performance requirements using the following Ncs parameters:
      -SCS=120, LRA=139, NCS=69
      -SCS=120, LRA=571, NCS=285
      -SCS=120, LRA=1151, NCS=575
      -SCS=480, LRA=139, NCS=69
      -SCS=480, LRA=571, NCS=285
      -SCS=960, LRA=139, NCS=69

The achievable maximum detectable timing offset for 480 kHz and 960 kHz SCS is 1.0 us and 0.5 us respectively.

The Rel 15 requirements for PRACH include timing offset scheme with offset up to 0.8 us, which exceeds the maximum detectable offset for 960 kHz SCS.

RAN4 to specify PRACH performance with timing offset up to 0.4 us for 480 kHz and 960 kHz SCS.

Rel 16 NR-U requirements are defined such that PRACH time error tolerance is larger or equal to TAC resolution.

Not to define time error tolerance that is smaller than the minimum possible step for the timing advance command.

RAN4 to define time error tolerance of PRACH timing requirements as:

| SCS | Channel model | Time error tolerance (ns) |
| --- | --- | --- |
| 120 | AWGN | 70 |
|  | TDLA10-650Note 1 | [120]Note 1 |
| 480 | AWGN | 18 |
|  | TDLA10-650 | [68] Note 1 |
| 960 | AWGN | 9 |
|  | TDLA10-650 | [59] Note 1 |
| NOTE 1: To be revised after definition of channel taps used for TDLA 10. |  |  |

Use the table below for the test cases and parameters related to PRACH demodulation requirements:

| Format | SCS | Prach sequence length | Ncs | Propagation condition | Frequency Offset (Hz) | Time estimation tolerance |
| --- | --- | --- | --- | --- | --- | --- |
| A2 | 120kHz | 139 | 69 | AWGN | 0 | 70 ns |
| B4 | 120kHz | 139 | 69 | AWGN | 0 | 70 ns |
| C2 | 120kHz | 139 | 69 | AWGN | 0 | 70 ns |
| A2 | 120kHz | 139 | 69 | TDLA10-650 | 7100 | 120 ns |
| B4 | 120kHz | 139 | 69 | TDLA10-650 | 7100 | 120 ns |
| C2 | 120kHz | 139 | 69 | TDLA10-650 | 7100 | 120 ns |
| A2 | 120kHz | 571 | 285 | AWGN | 0 | 70 ns |
| B4 | 120kHz | 571 | 285 | AWGN | 0 | 70 ns |
| C2 | 120kHz | 571 | 285 | AWGN | 0 | 70 ns |
| A2 | 120kHz | 571 | 285 | TDLA10-650 | 7100 | 120 ns |
| B4 | 120kHz | 571 | 285 | TDLA10-650 | 7100 | 120 ns |
| C2 | 120kHz | 571 | 285 | TDLA10-650 | 7100 | 120 ns |
| A2 | 120kHz | 1151 | 575 | AWGN | 0 | 70 ns |
| B4 | 120kHz | 1151 | 575 | AWGN | 0 | 70 ns |
| C2 | 120kHz | 1151 | 575 | AWGN | 0 | 70 ns |
| A2 | 120kHz | 1151 | 575 | TDLA10-650 | 7100 | 120 ns |
| B4 | 120kHz | 1151 | 575 | TDLA10-650 | 7100 | 120 ns |
| C2 | 120kHz | 1151 | 575 | TDLA10-650 | 7100 | 120 ns |
| A2 | 480kHz | 139 | 69 | AWGN | 0 | 18 ns |
| B4 | 480kHz | 139 | 69 | AWGN | 0 | 18 ns |
| C2 | 480kHz | 139 | 69 | AWGN | 0 | 18 ns |
| A2 | 480kHz | 139 | 69 | TDLA10-650 | 7100 | 68 ns |
| B4 | 480kHz | 139 | 69 | TDLA10-650 | 7100 | 68 ns |
| C2 | 480kHz | 139 | 69 | TDLA10-650 | 7100 | 68 ns |
| A2 | 480kHz | 571 | 285 | AWGN | 0 | 18 ns |
| B4 | 480kHz | 571 | 285 | AWGN | 0 | 18 ns |
| C2 | 480kHz | 571 | 285 | AWGN | 0 | 18 ns |
| A2 | 480kHz | 571 | 285 | TDLA10-650 | 7100 | 68 ns |
| B4 | 480kHz | 571 | 285 | TDLA10-650 | 7100 | 68 ns |
| C2 | 480kHz | 571 | 285 | TDLA10-650 | 7100 | 68 ns |
| A2 | 960kHz | 139 | 69 | AWGN | 0 | 9 ns |
| B4 | 960kHz | 139 | 69 | AWGN | 0 | 9 ns |
| C2 | 960kHz | 139 | 69 | AWGN | 0 | 9 ns |
| A2 | 960kHz | 139 | 69 | TDLA10-650 | 7100 | 59 ns |
| B4 | 960kHz | 139 | 69 | TDLA10-650 | 7100 | 59 ns |
| C2 | 960kHz | 139 | 69 | TDLA10-650 | 7100 | 59 ns |

References

- R4-2207223, WF on demodulation performance requirements definition for 52.6 - 71 GHz, Intel Corporation

- R4-2210664, WF on general and BS aspects for FR2-2 demodulation requirements, Intel Corporation

- TS 38.211, NR; Physical channels and modulation

- TS 38.141-2, NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing

- RP-213540, Revised WID:  Extending current NR operation to 71 GHz, Qualcomm

- TR 38.808 v17.0.0, Study on supporting NR from 52.6 GHz to 71 GHz

- R1-2007982 "On NR operations in 52.6 to 71 GHz" Ericsson

- R1-2008615 "NR using existing DL-UL NR waveform to support operation between 52p6 GHz and 71 GHz" Qualcomm Incorporated.

- R1-2200780, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e”

- R4-2202364, “Draft CR for TS 38.101-2: Introduction of system parameters for FR2-2”