3GPP TSG-RAN WG4 Meeting # 104-e                	R4-2212684

Electronic Meeting, August 15 – 26, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	On demodulation performance requirements for SDT

Agenda item: 	9.23

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on demodulation performance requirements for SDT.

  - Observations from last way forward

In the WF from RAN4#103-e meeting, [2], RAN4 reached the following agreements on the requirements for the RSRP measurements used for TA validation for CG-SDT:

Issue 1-1: X1 value for FR2 - Proposals - Option 1: max{480ms, 8*SSB periodicity} - Option 1a: max{480ms, N*SSB periodicity} - Option 1b: max{480ms, 8*SMTC periodicity} - Option 2: 400ms - Option 3: 1280ms - Option 4: N1 * 160 ms 2nd round discussion: Two sub-issues are involved: 1) a fixed or varying scaling factor; 2) SSB periodicity or SMTC periodicity SMTC periodicity is agreeable, and and most companies can accept a fixed value of 8 for the scaling factor, i.e., Option 1b is acceptable to most of companies, and one company proposes max{480ms, N*SMTC periodicity}. Since this is an open issue for core requirements, Moderator recommends going for Option 1b. Agreement: Option 1 b agreed Issue 1-2-1: Requirement for the duration between T2 and the actual CG-SDT transmission - Proposals - Option 1: No need to introduce a requirement for the interval between T2 and the actual CG-SDT transmission - Option 2: 640ms Agreement: A common understanding among companies is that the maximum duration between the moment of TA validation and actual CG-SDT transmission is 640ms.

Discussion

TA validation outcome from RRM

Figure 1: Worst case uncertainty introduced by RSRP measurement timing for TA validation.

As can be seen, the maximum uncertainty in the RSRP window considered for TA validation can be expressed as:

FR1: Umax = min(640ms, M1*TDRX)+ min(640ms, M1*TDRX)+Z,

FR2: Umax = X1+X1+Z,

Where Z is the time between T2 and the CG resource time. The value of Z is suggested in issue 1-2-1 above.

Based on the current version of TS 38.133 and the proposals above, issue 1-1 and issue 1-2-1, the worst case value of Umax are:

FR1: Umax = 640ms + 640ms + 640ms = 1920ms

FR2: Umax = 1280ms + 1280ms +640ms =3200ms

At a speed of 10m/s, the above Umax corresponds to moving 19.2m for FR1 and 32m for FR2.

FR1: Potential TA error due to Umax: 64ns

FR2: Potential TA error due to Umax: 107ns

For 15kHz SCS at FR1, the Cyclic Prefix, CP is 2.34us and for 120kHz at FR2, the CP is 0.59us.

This gives:

FR1, 30kHz SCS: TA error as percentage of CP: ~2.8%

FR2, 120kHz SCS: TA error as percentage of CP: ~20%

The timing error for same SCS’s are given as:

FR1, 30kHz SCS: Te = 8*64*Tc=261ns -> ~11% of CP

FR2, 120kHz SCS: Te = 3.5*64*Tc=114ns-> ~20% of CP

For FR1, 30kHz SCS, the additional potential timing error introduced by uncertainty added by TA validation at 10m/s is minor. At higher speeds, the error will grow and not reach a level similar to FR2 until speeds beyond what is reasonable for NR.

For FR2, 120kHz SCS, the additional potential timing error introduced by uncertainty added by TA validation at 10m/s is at the same level as Te itself, meaning the timing error which needs to be handled by BS demod is doubled. A higher speeds, the error may be larger than the BS can handle without special implementation.

Conclusion

This paper has presented Nokia’s views on RRM performance requirements for SDT. As part of this discussion, we have reached the following observations and proposals:

- For FR1, 30kHz SCS, the additional potential timing error introduced by uncertainty added by TA validation at 10m/s is minor. At higher speeds, the error will grow and not reach a level similar to FR2 until speeds beyond what is reasonable for NR.

For FR2, 120kHz SCS, the additional potential timing error introduced by uncertainty added by TA validation at 10m/s is at the same level as Te itself, meaning the timing error which needs to be handled by BS demod is doubled. A higher speeds, the error may be larger than the BS can handle without special implementation.

References

- RP-212594, Updated Work Item on NR small data transmissions in INACTIVE state

- R4-2210622, WF on RRM requirements for NR-SDT_v02, ZTE