3GPP TSG-RAN WG4 Meeting # 104-e                	R4-2212688

Electronic Meeting, August 15 – 25, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on RRM requirements for FR2 multi-Rx chain reception

Agenda item: 	11.8.3

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on RRM aspects related to the multi-RX chain reception in FR2 for Rel-18. This paper presents an overview on the objectives of the work item, and based on that observations and proposals are derived concerning expected scheduling restrictions and RRM measurement enhancements.

Discussion

Introduction

RAN4#104 starts the study of multi-chain RX UE. Rel-17 feMIMO has had discussions on UE with multiple antenna panels. At the end, multiple Rel-18 WIs covering the requirement for NR FR2 multi-Rx chain DL reception has been approved in RAN4.  In Rel-18, the scope of NR_FR2_multiRx_DL is extended to multi-RX chain UE and includes UE capability of multi-Rx chains. The WID objectives are as follow [1]:

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO - Enhanced RF requirements: - Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs - The legacy spherical coverage requirement for reception from a single direction will be kept - PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized - Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier - Enhanced RRM requirements: - The following requirements should be studied and specified if necessary: - L1-RSRP measurement delay - L3 measurement delay (both cell detection delay and measurement period can be considered) - The starting point is the enhancements related to L1-RSRP measurement enhancements - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) NOTE: - The case of single TCI is handled as a second priority. Additional aspects related to single TCI can be further revisited.

The WID specifies UE study on simultaneous DL reception from different directions with different QCL Type-D properties on a single component carrier. The study starts from UE capabilities on the RRM and demod. Since it mentions different QCL Type-D RSs, a basic assumption is that a UE receives DL signals from multiple non-collocated TRPs (or gNBs).

Rel-16 MIMO has studied under intra-cell mTRP deployment, and Rel-17 MIMO has studied under inter-cell mTRP scenarios. Therefore Rel-18 multi-RX chain UE should be able to support both intra-cell and inter-cell scenarios. Also, Rel-18 MIMO assumes non-collocated mTRP scenarios, network scenarios with non-collocated cell deployment should be reasonably considered with common deployment cell size.

Rel-18 multi-RX chain UE should be able to support both intra-cell and inter-cell operation with TRPs located within reasonable intercell distance.

UE capability using multiple RX chains

The scope of the work depends on how the UE capability with multiple RX chains is defined. The UE capability scopes will be related to the impact on requirements for RRM measurements, TCI switching and RX scheduling restriction rules eventually. The WID has defined the  terms ‘multiple RX chains’, that does not simply mean multiple antenna panels. It is possible that multi-RX chains assumes multiple UE implementations options including simultaneous operation on multiple antenna panels, frond-end, demodulation and RRM. There can be various UE implementation options, the WI scope practically include them for requirement study.

The scope of a RX chain architecture includes possible implementations and UE capabilities as below:

- Multiple Antenna panel

- Multiple Antenna panel + AGC

- Multiple Antenna panel + AGC + front-end (time and frequency sync)

- Multiple Antenna panel + AGC + front-end + Demod/RRM

Figure 1 Architectural overview of blocks involved in multiple RX chain capabilities

Beam management with multiple antenna panel

FR2 antenna arrays on UEs are directive, therefore, in order to achieve good coverage UEs have multiple antenna panels embedded. Nevertheless, the number of antenna panels and the number of Rx chains on the UE does not necessarily need to have 1-to-1 mapping. As such, Rel-17 FR2 RAN4 requirements are developed for single-chain UEs, meaning that these UEs can only activate a single FR2 panel at the time. This behavior has a clear drawback on some procedures, e.g. neighbor cell measurements, since the UE has to sweep through its panels one at the time in order to get a full picture of its environment. This is illustrated in Figure 2 where it is shown that a single-chain 4-panel UE requires 4 consecutive bursts for a single sample acquisition. Then additionally, 3-5 samples are assumed needed for each Rx spatial setting for L1 or L3 measurements to ensure the measurement accuracy.

Figure 2 Example of single Rx chain UE operation with 4 panels measuring neighbor cells

RAN4 can consider two options for beam management for UE with multiple RX chains

- Option-(i) Reuse independent beam management concept (IBM) from Inter-band CA study

- Option-(ii) define panel or RX chain specific behaviors with RX panel control signal for DL

A main difference between options is whether the UE RX panel information is transparent to network or not. In option-(i) case, the network may know if the UE is capable of multiple RX chains, but it may not know which panel is activated for the best RX performance. In option-(ii) case, a UE may need to report which panel is used or active for RRM measurement, so the network may indicate a UE with panel-activation assistance information. According to current RAN1 discussions of Rel-18 UL MIMO, the panel specific information may be introduced by RAN1 or RAN2, accordingly RAN4 may need to consider it to be consistent.

One RX chain has independently controllable antenna panel. The minimum study scope using multiple antenna panels is found in UE receiving signals from different QCL-type-D sources. If two or more  TRPs are non-collocated, the channels have different QCL-type-D property, and also average gain, delay spread/shift, and Doppler spread/shift can be different. Other QCL types A,B,C also are possibly considered. The study scope of UE behaviors includes not only independent beam management for different QCL-D sources with multiple TRPs, but also it can include UE behaviors handling the other QCL-types such as {average gain, delay shift and doppler shift} for various scenario and usecase support such as high speed train usecase or practical cell size.

Study independent beam managements for different QCL-D sources with multiple TRPs, and also study other QCL channel property impacts of {average gain, delay shift and doppler shift} for multi-RX chain requirements.

RX gain control of multiple RX chains

RAN4 requirements have set a restriction on RX signal reception, when RX signal level difference is less than a threshold. If signals from different QCL sources are received by different beam independently, the RX signal level will be different. From Figure 1, the beam gain can also be different too between a wide and a narrow beam that may be used in different Rx chains, therefore it is hard to apply a fixed tolerance of Rx signal level difference from multiple TRPs. Since each RX chain is assumed to operate independently, signal gain control is assumed independent per RX chain, RAN4 should discuss UE capability of multiple RX chains regarding AGC gain difference.

Study UE behaviors and capability of multiple RX chains regarding handling Rx signal level difference between two channels.

RX Timing Difference (RTD) from different QCL sources.

The next question is whether a UE can handle signals with different RTD from different QCL sources. So far, RAN4 has studied MRTD with 260 ns for FR2 intra-band CA and 8 us MRTD for FR2 inter-band CA IBM as Table 1. These numbers for intra-band were derived without considering non-collocated TRP deployment with different QCL-type D property. A UE with multi-RX chain should be able to support mTRP deployments, RAN4 needs to study UE RX capability receiving extended MRTD based on non-collocated mTRP deployment scenario operating in the same band.

Table 1 Summary of existing MRTD requirements in 38.133 considering existing rules

| Intra/inter band | Scenario | FR | Sync/async | TAE (us) | MRTD (us) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 15 | 30 | 60 | 120 |
| Inter band | EN-DC | - | Synchronous |  | 33 | 33 | 33 | 33 |
|  |  |  | Asynchronous |  | 500 | 250 | 125 | 65.2 |
|  | NE-DC | - | Synchronous |  | 33 | 33 | 33 | 33 |
|  |  |  | Asynchronous |  | 500 | 250 | 125 | 65.2 |
|  | NR-DC | FR1 | synchronous | 3 | 33 | 33 | 33 | 33 |
|  |  | FR2-1 | Synchronous | 3 | 8 | 8 | 8 | 8 |
|  |  | FR1 – FR2-1 | synchronous | 3 | 33 | 33 | 33 | 33 |
|  | CA | FR1 |  |  | 33 | 33 | 33 | 33 |
|  |  | FR2-1 |  |  | 8 | 8 | 8 | 8 |
|  |  | FR1 FR2-1 |  |  | 25 | 25 | 25 | 25 |
| Intra-band | EN-DC |  | Synchronous |  | 3 | 3 | 3 | 3 |
|  |  |  | Asynchronous |  | 500 | 250 | 125 | 62.5 |
|  | Non-contiguous CA | FR1 |  | 3 | 3 | 3 | 3 | 3 |
|  |  | FR2-1 |  | 0.26 | 0.26 | 0.26 | 0.26 | 0.26 |

Study UE RX capability receiving extended MRTD based on non-collocated mTRP deployment scenario.

RX scheduling restriction and simultaneous reception

The WID mentions simultaneous reception, but it is unclear which signal can be simultaneously received. Possible UE implementation supporting simultaneous reception using multiple RX chains are as below

- RRM and demodulation simultaneous processing capability

- RRM and CSI simultaneous measurements and processing capability

- PDCCH and PDSCH simultaneous demodulation capability

- PDSCH simultaneous RX (basic for 4 layers)

- PDCCH simultaneous monitoring for multi-DCI dual-TCI

Some of above may drop in demodulation requirement discussions, but we note that RX design between RRM and demodulation are related in a single RX architecture. Therefore, it will be good to check each bullets together in RF/RRM/Demod of the WI.  The Rel-16 existing UE capability simultaneousReceptionDiffTypeD-r16 is defined to support simultaneous RX with different type-D, however it is unclear which RX processing capability is indicated by simultaneousReceptionDiffTypeD-r16. Depending on the UE simultaneous RX capability, RX scheduling restriction rules can be discussed.

Possible scenarios supporting simultaneous reception using multiple RX chains needs to be clarified in Rel-18 WI scope, including:

- RRM and Demod simultaneous processing capability

- RRM and CSI simultaneous measurements and processing

- PDCCH/PDSCH simultaneous RX

- PDSCH simultaneous RX (basic for 4 layers)

- PDCCH simultaneous monitoring for multi-DCI dual-TCI 
( Depending on the UE simultaneous RX capability, RX scheduling restriction rules can be discussed.)

| (a) | (b) | (c) |
| --- | --- | --- |

Figure 3 Possible scenarios for use of multi Rx in DL where (a) is the current Rel 15-17 scenario, (b) measurements can be performed in multiple beams simultaneously; and (c) measurements and SSB monitoring can be performed on different beams

UE RRM measurement behavior impacts

L1 and L3 SMTC

In Rel-17 feMIMO, it is assumed that a UE is able to measure L1 and L3 sequentially in time division manner, also measure a serving cell and a non-serving cell sequentially as well. RAN4 understanding of ‘SSBs associated with different PCIs overlap’ consists of 2 cases:

- Case I: SSBs associated to the cell with PCI different from serving cell used for L1-RSRP measurements, and SSBs configured for serving cell L1 measurements (i.e. L1-RSRP, L1-SINR, RLM, BFR), are overlapped.

- Case II: SSBs associated to the cell with PCI different from serving cell used for L1-RSRP measurements, and SSBs for the serving cell L3 measurements, i.e. SSBs in SMTC, are overlapped.

Rel-17 conclusion was that a FR2 UE measures L3-measurements within SMTC, while L1 measurement is executed out of SMTC. This sets quite complex behaviors in both network and UE causing RX scheduling restriction and measurement latency.

In Rel-18, if a UE can measure L1 and L3 simultaneously, or if a UE can measure a serving cell and non-serving cells simultaneously, the SMTC window can be used for both L1 and L3 measurements, also for serving cell and non-serving-cell. Therefore, it is beneficial that RAN4 revisits the SMTC use-case for ‘L1 and L3’ measurements and ‘serving cell and non-serving cell’ to reduce measurement latency. Rel-18 FR2 multi-RX chain UE may be able to measure RSs simultaneously with different QCL-D sources or with different beam assumptions (i.e. narrow and wide beam assumption)

RAN4 to revisit the FR2 SMTC use case for ‘L1 and L3’ measurements and ‘serving cell and non-serving cell’ to reduce measurement latency. Rel-18 FR2 multi-RX chain UE may be able to measure RSs simultaneously with different QCL-D sources or with different beam assumptions (i.e. narrow and wide beam assumption)

Dual TCI switching delay requirements

Rel-17 unified TCI switching framework should be applicable to the UE requirements with multi-RX chains. Ideally, the multiple RX chains are controlled independently, so it is expected that Rel-17 TCI switching delay requirements can be applicable as Rel-18 UE requirements with multi-RX chains. We are open to discuss if Rel-17 TCI switching delay requirements cannot be applicable as Rel-18 UE requirements with multi-RX chains due to the additional UE implementation.

Each TCI switching per RX chain is assumed to be independent in aspect of TCI switching delay. RAN4 to study if Rel-17 TCI switching delay requirements can be applicable as Rel-18 UE requirements with multi-RX chains.

Active TCI states for multiple RX chains

When dual TCI is given by DCI, it means that dual TCI can be activated. Then a UE tracks time and frequency per TCI state by referring to the legacy active TCI state definition.

tci-StatePDSCH TS38.306 - maxNumberActiveTCI-PerBWP indicates the maximum number of activated TCI-states per BWP per CC, including control and data. If a UE reports X active TCI state(s), it is not expected that more than X active QCL type D assumption(s) for any PDSCH and any CORESETs for a given BWP of a serving cell become active for the UE. The UE shall include this field. Note the UE is required to track only the active TCI states.

RAN4 to study if a UE with multiple RX chains tracks time and frequency per TCI when dual TCIs are activated per RX chain.

Power saving

Since a UE has multiple RX chains, power consumption can be concerned. If a UE turns on and off arbitrarily, there may be concern of malfunction of multiple RX chain behavior cooperated with network. side effect there can be processing modes and indications per RX chain. Network controls a UE with multiple RX chains when the UE gets benefits of power saving. Power saving can be additional requirements of UE with multiple RX chains.

Power saving can be additional requirements of UE with multiple RX chains.

Conclusion

This paper has presented Nokia’s views on Rel 18 multi-RX chain reception in FR2. As part of this discussion, we have reached the following observations and proposals:

Proposal 1: Rel-18 multi-RX chain UE should be able to support both intra-cell and inter-cell operation with TRPs located within reasonable intercell distance.

Proposal 2: The scope of a RX chain architecture includes possible implementations and UE capabilities as below:

Multiple Antenna panel

Multiple Antenna panel + AGC

Multiple Antenna panel + AGC + front-end (time and frequency sync)

Multiple Antenna panel + AGC + front-end + Demod/RRM

Proposal 3: RAN4 can consider two options for beam management for UE with multiple RX chains

Option-(i) Reuse independent beam management concept (IBM) from Inter-band CA study

Option-(ii) define panel or RX chain specific behaviors with RX panel control signal for DL

Proposal 4: Study independent beam managements for different QCL-D sources with multiple TRPs, and also study other QCL channel property impacts of {average gain, delay shift and doppler shift} for multi-RX chain requirements.

Proposal 5: Study UE behaviors and capability of multiple RX chains regarding handling Rx signal level difference between two channels.

Proposal 6: Study UE RX capability receiving extended MRTD based on non-collocated mTRP deployment scenario.

Proposal 7: Possible scenarios supporting simultaneous reception using multiple RX chains needs to be clarified in Rel-18 WI scope, including:

RRM and Demod simultaneous processing capability

RRM and CSI simultaneous measurements and processing

PDCCH/PDSCH simultaneous RX

PDSCH simultaneous RX (basic for 4 layers)

PDCCH simultaneous monitoring for multi-DCI dual-TCI  ( Depending on the UE simultaneous RX capability, RX scheduling restriction rules can be discussed.)

Proposal 8: RAN4 to revisit the FR2 SMTC use case for ‘L1 and L3’ measurements and ‘serving cell and non-serving cell’ to reduce measurement latency. Rel-18 FR2 multi-RX chain UE may be able to measure RSs simultaneously with different QCL-D sources or with different beam assumptions (i.e. narrow and wide beam assumption)

Proposal 9: Each TCI switching per RX chain is assumed to be independent in aspect of TCI switching delay. RAN4 to study if Rel-17 TCI switching delay requirements can be applicable as Rel-18 UE requirements with multi-RX chains.

Proposal 10: RAN4 to study if a UE with multiple RX chains tracks time and frequency per TCI when dual TCIs are activated per RX chain.

Proposal 11: Power saving can be additional requirements of UE with multiple RX chains.

References

- RP-221753, Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Incorporated