3GPP TSG-RAN WG4 Meeting # 104-e							         R4-2212697

Electronic Meeting, August 15 – August 26, 2022

Agenda item:			11.13.2.1

Source:	Samsung

Title:	Scenarios, assumptions and analysis for SBFD coex study

Document for:	Approval

# Introduction

In 3GPP RAN#96 meeting, it is agreed that the Study on Evolution of NR Duplex Operation (940082) demand RAN4 to study the feasibility of adjacent channel co-existence with legacy operation. In addition, it is further noted that RAN4 should be involved early to provide necessary information to RAN1 as needed. Considering the work demand and urgency, in this document, we proposed the interference scenarios and simulation assumptions for the co-existence study with legacy system in adjacent channel. Moreover, we share our preliminary results of certain scenarios under the assumptions proposed for information.

# Adjacent channel interference scenarios

The objectives of the Study on Evolution of NR Duplex Operation (940082) described the duplex mode of gNB and UE and the adjacent channel scenarios. The related content is directly quoted below.

RP-221352 [Unrelated content omitted] In this study, the followings are assumed: • Duplex enhancement at the gNB side • Half duplex operation at the UE side • No restriction on frequency ranges The detailed objectives are as follows: • Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

Given the SID approved by RAN plenary, it’s RAN4’s responsibility to carry out adjacent channel co-ex studies for subband full duplex scenarios. And the RAN4 co-ex study will be the fundamental basis for discussion of checking the supporting scenarios of SBFD, checking the feasibility of existing ACIR requirements in SBFD scenario and checking the feasibility of new ACIR (ACLR/ACS) for new SBFD-capable gNBs.

Proposal 1: The RAN4 shall conduct co-ex study as the co-ex study will be the discussion basis for RAN4 to:

- Determine the supporting co-ex scenarios for SBFD operation;

- Check the adjacent channel interference with existing ACIR (ACLR/ACS) requirements in SBFD operation;

- Check the feasibility of new ACIR (ACLR/ACS) requirements for new SBFD-capable gNBs.

The interference scenario between the SBFD and legacy TDD system in adjacent band can be summarized into the figure below. The victim system is a static TDD system operating on the adjacent channel to the aggressor system, which enabling SBFD operation by having both subband UL and DL simultaneously. The bandwidths of the system and sub-bands in the figure below are just an example.

Fig.1 SBFD and Legacy TDD in adjacent band

Observation 1: The adjacent channel co-existence study can focus on the timeslot(s) where the aggressor SBFD having its subband UL and DL operating simultaneously while the victim legacy TDD system is in its DL timeslot(s). The rest timeslot(s) are legacy TDD co-existence cases, and the adjacent channel interference (ACI) of those cases were solved by legacy ACLR and ACS of each frequency range already.

In the RP-221351, it is agreed that the two subband configurations are considered in SBFD evaluation. Thus, it is proposed to RAN4 to consider these two subband configurations in co-ex study.

Agreement in RP-223151 For SBFD evaluation, consider the following for SBFD subband configurations: - SBFD Subband configuration#1 with {DUD} pattern, which means one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth. - SBFD Subband configuration#2 with {DU} pattern, which means one SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth.

Proposal 2: Proposed to consider both SBFD Subband configuration#1 with {DUD} pattern, which means one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth, and SBFD Subband configuration#2 with {DU} pattern, which means one SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth, in RAN4 co-ex study. These two configurations are shown in figure below:

- (b)

Fig.2 SBFD subband configurations: (a) #1 {DUD}, (b) #2 {DU}

Moreover, the adjacent channel interference case was described as ‘Deployment Case 4’ in RAN1#109-e meeting, and its considerations and agreements are directly quoted below. In RAN1, the scenarios considered were focused on the Macro scenario in both FR1 and FR2-1 as starting point.

Agreements in Chair’s Notes of RAN1 #109-e meeting [Unrelated content omitted] Deployment Case 4 (Adjacent-channel co-existence case): Two operators each using one carrier are considered and the two carriers are adjacent carriers. One operator uses legacy TDD operation (static TDD operation) while the other operator uses SBFD operation with the same SBFD subband configuration. For SBFD Deployment Case 4, at least consider the following scenarios for evaluation from RAN1 perspective:  FR1: Urban Macro  FR2-1: Dense Urban Macro layer  FFS: UE outdoor/indoor proportion, clustering, etc  FFS: the grid shift between two networks, e.g., 0%, 100%  FFS: Indoor hotspot, Dense Urban Micro layer

In addition, the CLI results from TR 38.828 also gives us valuable information to narrow down the interested cases to reduce the workload for RAN4 in SBFD study. The results of CLI TR 38.828 is summarized in the table below.

Table 1. Summary of CLI results in TR 38.828

| Scenario No. | Deployment Scenario (Aggressor -> Victim) | Aggressor in CLI | Victim | CLI Results |
| --- | --- | --- | --- | --- |
| 1 | Macro → Macro | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, UL100% | NR, 100 MHz, DL | UE-UE, no degradation |
| 2 |  | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, DL100% | NR, 100 MHz, UL | BS-BS, degradation. |
| 3 | Macro → Indoor | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, UL100% | NR, 100 MHz, DL | UE-UE, degradation. |
| 4 |  | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, DL100% | NR, 100 MHz, UL | BS-BS, no degradation. |
| 5 | Indoor → Macro | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, UL100% | NR, 100 MHz, DL | No degradation |
| 6 |  | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, DL100% | NR, 100 MHz, UL |  |
| 7 | Indoor → Indoor | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, UL100% | NR, 100 MHz, DL | UE-UE, no degradation |
| 8 |  | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, DL100% | NR, 100 MHz, UL | BS-BS, no degradation |
| 9 | Macro → Macro | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, UL100% | NR, 200 MHz, DL | UE-UE, no degradation |
| 10 |  | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, DL100% | NR, 200 MHz, UL | BS-BS, degradation. |
| 11 | Micro → Micro | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, UL100% | NR, 200 MHz, DL | UE-UE, no degradation |
| 12 |  | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, DL100% | NR, 200 MHz, UL | BS-BS, no degradation. |
| 13 | Indoor → Macro | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, UL100% | NR, 200 MHz, DL | No degradation. |
| 14 |  | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, DL100% | NR, 200 MHz, UL |  |
| 15 | Indoor → Indoor | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, UL100% | NR, 200 MHz, DL | UE-UE, no degradation |
| 16 |  | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, DL100% | NR, 200 MHz, UL | BS-BS, complex results, power dependent. |

Observation 2: The results from TR 38.828 reflects the cross-link adjacent channel interference impact focused on the FR1 and FR2 Macro-Macro case for BS-BS interference link, and FR1 Macro-Indoor case for UE-UE interference link. And RAN1 agreements also shows they will focus on Macro cases as starting point.

Hence, from Observation 1 and 2, we would like to propose RAN4 to consider the following scenarios, as a starting point, to conduct the adjacent channel co-existence study between SBFD and legacy TDD. And it is proposed to adopt the legacy TDD ACI as evaluation baseline which is the same as principle used in CLI study.

Proposal 3: Considering Observation 1 and 2, we propose RAN4 to consider the following scenarios in Table 2 as the starting point for SBFD co-ex study with legacy TDD system.

Table 2. SBFD adjacent channel co-existence scenarios (starting point)

| FR | Scenario No. | Deployment Scenario (Aggressor -> Victim) | Aggressor baseline | Aggressor | Victim |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Macro -> Macro | NR, 100MHz, DL ACI | NR, 100MHz SBFD1 | NR, 100MHz, DL |
|  | 2 | Macro -> Macro | SBFD Intra-system | NR, 100MHz, DL | NR, 100MHz SBFD1 |
|  | 3 | Macro -> Indoor | NR, 100MHz, DL ACI | NR, 100MHz SBFD1 | NR, 100MHz, DL |
|  | 4 | Macro -> Indoor | SBFD Intra-system | NR, 100MHz, DL | NR, 100MHz SBFD1 |
| 2 | 5 | Macro -> Macro | NR, 200MHz, DL ACI | NR, 200MHz, SBFD2 | NR, 200MHz, DL |
|  | 6 | Macro -> Macro | SBFD Intra-system | NR, 200MHz, DL | NR, 200MHz, SBFD2 |
| Note 1: For FR1, consider subband config#1 with {DUD}: 40MHz DL + 20MHz UL + 40MHz DL and subband config#2 with {DU}: 80MHz DL + 20MHz UL. Note 2: For FR2, consider subband config#1 with {DUD}: 50MHz DL + 50MHz UL + 100MHz DL and subband config#2 with {DU}: 150MHz DL + 50MHz UL. |  |  |  |  |  |

# Simulation assumptions and parameters for SBFD adjacent channel co-existence study with legacy TDD

The simulation assumptions, including deployment parameters and system characteristics, can directly adopt most assumptions used in Section 5.2 of TR 38.828 CLI study as a starting point. At the same time, we also find some assumptions in TR 38.828 requires update to fit the SBFD study, and these major changes are discussed and proposed in this section.

Proposal 4: Propose to adopt the system characteristics, deployment parameters and other assumptions from TR 38.828 as the starting point for SBFD co-ex study. But the assumptions should be updated accordingly to fulfill the SBFD co-ex purpose.

## Element number of SBFD base station

Because the SBFD operation considered antenna isolation at base station side between its DL transmitting and UL receiving ports, the number of antenna elements of the SBFD base station is different to what was assumed in CLI study. While the base station of victim legacy TDD system can still follow the element configurations as described in TR 38.828. Our understanding is that the SBFD BS could 1) utilize half of its original panel for DL and UL each, or 2) the SBFD BS could implement an extra panel with same number of elements for subband UL receiving so that the subband DL Tx will utilize same elements as legacy TDD BS. Thus, it is proposed to RAN4 to consider the above two cases in discussing co-ex assumptions.

Proposal 5: Our understanding is that the SBFD BS could 1) utilize half of its original panel for DL and UL each, or 2) the SBFD BS could implement an extra panel with same number of elements for subband UL receiving so that the subband DL Tx will utilize same elements as legacy TDD BS. Thus, it is proposed for the meeting to consider two options for SBFD BS antenna and TRP power:

- Option 1: Utilize half of its original panel for SBFD UL and DL each. In this case, the TRP and elements number for DL and UL in SBFD BS will be half of the TDD BS configuration.

- Option 2: Utilize an extra panel for subband UL operation. In this case, the TRP and element number for DL and UL in SBFD BS will be the same as TDD BS configuration.

With above two options, we propose RAN4 to consider the following new antenna configuration, BS powers for SBFD base station. The following tables for different frequency ranges and scenarios are originally from TR 38.828, and the proposed modifications are highlighted in yellow below.

For FR1

Table 5.2.1.1.1-1: Single operator layout for urban macro in FR1 (4 GHz)

| Layout | Single layer with 19 hexagonal cell with wrap around |
| --- | --- |
| Inter-BS distance | 500 m |
| Carrier frequency | 4 GHz |
| Path-loss model | - Macro(Aggressor) → Macro(Victim) - Macro-to-UE: UMa see TR 38.803 [5] - Macro-to-Macro: UMa (h_UE = 25 m) see TR 38.803 [5] - UE-to-UE: Outdoor UE – Outdoor UE see TR 36.828 [6] + penetration loss see TR 38.803 [5] |
| BS Tx power | For Legacy TDD: 49 dBm For SBFD: Option 1: 46 dBm Option 2: 49 dBm |
| UE Tx power | 23 dBm |
| BS antenna configurations | For Legacy TDD: (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ For SBFD: Option 1: (Mg,Ng,M,N,P)=(1,1,4,8,2) (dH,dV)=(0.5,0.8)λ Option 2: (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ Note 1,2 |
| BS antenna height | 25 m |
| BS antenna element gain + connector loss | 5 dBi (assuming antenna 1.8dB loss) |
| BS receiver noise figure | 5 dB |
| UE antenna configuration | Omni |
| UE antenna height | hUT=3(nfl-1)+1.5 nfl for outdoor UEs: 1 nfl for indoor UEs: nfl~uniform(1,Nfl) where Nfl = 1 |
| UE antenna gain | 0 dBi |
| UE receiver noise figure | 9 dB |
| Multi operators layout | uncoordinated operation (100% Grid Shift) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. |  |

For FR2

Table 5.2.2.5.1-1: FR2 BS antenna modelling for urban macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| (Mg, Ng, M, N, P) | For 30 GHz legacy TDD: (1, 1, 8, 16, 2) For SBFD: Option 1 (1, 1, 8, 8, 2) Option 2 (1, 1, 8, 16, 2) Note1,2 |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 65 degree element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation results. |  |

Table 5.2.2.4-1: Other simulation parameters

| Parameters | Urban macro |
| --- | --- |
| Channel bandwidth | 200MHz |
| Scheduled channel bandwidth per UE (DL) | 200MHz |
| Scheduled channel bandwidth per UE (UL) | 200MHz |
| Traffic model | Low (RU 10%) and Full buffer |
| DL power control | NO |
| UL power control | YES |
| BS max TX power in dBm TRP (Total Radiated Power) | For legacy TDD: 43dBm For SBFD: Option 1: 40 dBm Option 2: 43 dBm |
| UE Peak EIRP in dBm | 22.4 dBm |
| UE min TX power in dBm | -40dBm |
| BS Noise figure in dB | 10 (note 1) |
| UE Noise figure in dB | 10 (note 1) |
| Handover margin | 3dB |

Proposal 6: Propose to consider two options of SBFD BS power and elements based on what was assumed TR 38.828, while the victim legacy TDD BS assumption stays the same. The detailed proposed changes for SBFD BS can be found in the highlighted part of above tables.

## Downtilt angles of base stations

In TR 38.828, the mechanical downtilt angle of the BS is not provided. We would like to propose to use the following parameters as the BS mechanical down-tilt angle in different frequency ranges as a starting point. The numbers were from TR 38.803.

Table 3. BS mechanical down-tilt angle proposal

|  | FR1 | FR2 |
| --- | --- | --- |
|  | Macro | Macro |
| BS mechanical down-tilt angle | 6 | 6 |

Proposal 7: Propose to align the mechanical down-tilt angles assumptions for BS. And it is proposed to use 6 degrees for the Macro BS for FR1 and FR2 as provided in TR 38.803.

## Transmission power control

In TR 38.828 section 5.2.3.4, the uplink power control (UPC) model provided is lack of necessary input parameters, such as receiver noise figure, target SNR, etc. Hence, in this document, we would like to discuss and align the UPC assumptions with the following equations, which provides a more general equation for UPC.

Proposal 8: Propose to use the following as the general uplink power control model for SBFD co-existence study.

For downlink scenario, no power control scheme is applied. For uplink scenario, TPC model specified in Section 9.1 TR 36.942 [9] is applied with following parameters. - CLx-ile = –SNR_target + UE_maxpower – ThermalNoise – BS_NoiseFigure + 10*log10(BW) - γ = 1 Where, SNR_target for FR1 and FR2 are 15 dB.

## FR1 UE ACLR

In TR 38.808, it did not consider the ACLR1 and ACLR2 for FR1 UE. But in SBFD operation, the subband configurations: #1 {DUD} and #2{DU} will result different levels of adjacent channel interference in its adjacent legacy TDD channel, it is reasonable to consider the ACLR1 and ACLR2 for FR1 UE referring to TR 36.942.

Proposal 9: For FR1, it is proposed to adopt ACLR1 as 30 dBc and ACLR2 as 43 dBc for FR1 UE.

## Simulation description

The simulation steps in TR 38.828 should be updated to better fulfill the SBFD co-ex study purpose. Based on the simulation scenarios proposed in Table 2 in section 2 of this document. We propose the following steps for SBFD co-ex study simulation.

Proposal 10: Propose to adopt the following steps, modified from TR 38.828 and other general legacy RAN4 coex study report, as the simulation steps for SBFD coex study.

1. Aggressor and victim network are generated. - UEs are distributed randomly across the network. 2. UE associations: UEs are associated to base station based on coupling loss. - Associations are made assuming a single element at both UE and BS. 3. Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks. 4. When legacy TDD system is victim, follow steps 4a: 4a. Throughput is computed considering ACI from another static TDD system as baseline aggressor: - Thputbaselinebpshz=fSINRICI+ACI_baseline=fSN+IICI+IACI_baseline, where IICI is the inter-cell interference and IACI_baseline is the adjacent channel interference from the baseline aggressor. When SBFD system is victim, follow steps 4b: 4b. Throughput is computed in the victim system without considering ACI as below: - Thputbaselinebpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference. 5. Throughput is computed considering ACI below: - ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

Proposal 11: It is proposed to follow the TR 38.828’s evaluation criteria to check the 50% and 5% throughput loss compared to the baseline scenario as described in table 2 (Proposal 3). And the criteria is 5% throughput degradation.

When evaluating the SINR/Throughput of SBFD system itself, the noise floor of the SBFD BS will rise due to the self-interference. The specific level of the self-interference level and the performance of each component to cancel the self-interference is discussed in [6]. Considering the different implementations of the SBFD and the simulation nature of RAN4 co-ex study, it is proposed to use a simple value to represent the noise floor raise when simulating the SBFD SINR/Throughput. And according to [6], we propose RAN4 to consider use {N=noise floor+1dB} in step 4b and 5 in above proposed steps for evaluation.

Proposal 12: In the above step 4b and 5, when SBFD is victim system, it is proposed to use {N = noise floor + 1dB} for the SBFD system to simulate the self-interference impact as a simplified method to evaluate its SINR/throughput.

# Co-ex study initial results

## Simulation calibration with CLI results

Compared to the legacy RAN4 co-ex studies, CLI is the new interference sources in SBFD co-ex study to the legacy TDD system. Thus in this section, we share our CLI results and compared them with the results approved in the TR 38.828 for calibration and information purpose.

FR1:

Table 4. Samsung FR1 Macro-Macro CLI results compared with TR 38.828

| FR1: Macro-Macro case |  |  |  |  |
| --- | --- | --- | --- | --- |
| Source |  | Observation Point | Victim DL |  |
|  |  |  | Aggressor 100% UL (CLI) |  |
|  |  |  | SNR degradation (dB) | Throughput degradation (%) |
| CLI | Huawei (1905522) | 5% | -0.13 | -2.83 |
|  |  | 50% | -0.37 | -1.75 |
|  |  | 95% | -0.21 | 0.00 |
|  | LGE (1907601) | 5% | 2.5 | 31.33 |
|  |  | 50% | 0.04 | 0.2 |
|  |  | 95% | 0.2 | 0.47 |
|  | Ericsson (1906097) | 5% | <1 | 3.1 |
|  |  | 50% | <1 | 1.2 |
|  |  | 95% | <1 | 2.1 |
|  | Nokia (1907604) | 5% | -1.5 | -31.1 |
|  |  | 50% | -0.7 | -9 |
|  |  | 95% | -0.4 | -1.1 |
|  | Qualcomm (1906703) | 5% | -0.70 | -8.44 |
|  |  | 50% | -0.38 | -1.78 |
|  |  | 95% | -0.27 | 0 |
|  | Samsung | 5% | -0.58 | -5.43 |
|  |  | 50% | -0.54 | -2.02 |
|  |  | 95% | -0.20 | 0 |

Fig.3 CLI: SINR and Throughput with ACI in FR1 Macro-Macro

FR2:

Table 5. Samsung FR2 Macro-Macro CLI results compared with TR 38.828

| FR2: Macro-Macro case |  |  |  |  |
| --- | --- | --- | --- | --- |
| Source |  | Observation Point | Victim DL |  |
|  |  |  | Aggressor 100%UL (CLI) |  |
|  |  |  | SNR degradation (dB) | Throughput degradation (%) |
| CLI | Huawei (1905523) | 5% | -0.4 | -4.07 |
|  |  | 50% | -0.68 | 0.00 |
|  |  | 95% | -0.82 | 0.00 |
|  | LGE (1907062) | 5% | 5.5 | 37.08 |
|  |  | 50% | 2.9 | 9.5 |
|  |  | 95% | 2.19 | 4.96 |
|  | Ericsson (196098) | 5% | <1 | <1 |
|  |  | 50% | <1 | <1 |
|  |  | 95% | <1 | <1 |
|  | Qualcomm (1906703) | 5% | -0.38 | -1.95 |
|  |  | 50% | -0.22 | 0 |
|  |  | 95% | -0.32 | 0 |
|  | Nokia (1907604) | 5% | -1.5 | -35.0 |
|  |  | 50% | -0.7 | -7.0 |
|  |  | 95% | -0.4 | -1.2 |
|  | Samsung | 5% | -0.55 | -4.93 |
|  |  | 50% | -0.29 | 0 |
|  |  | 95% | -0.53 | 0 |

Fig.4 CLI: SINR and Throughput with ACI in FR2 Macro-Macro

Observation 3: The CLI SINR and Throughput distribution of our simulation platform seems quite aligned with the results in TR 38.828.

## Initial co-ex results of SBFD

Following the proposed assumptions, as described in proposal 2~6, we have some very initial results for SBFD Macro -> Legacy TDD Macro cases in FR1 and FR2. The results assumed subband config#2 {DU} in Table 2.

Table 6. Samsung initial results for SBFD co-ex with legacy TDD in FR1/FR2 Macro-Macro scenario

| FR | Deployment Scenario (Aggressor -> Victim) | Aggressor | Victim | Observation Point | SNR degradation (dB) | Throughput degradation (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Macro -> Macro | NR, 100MHz SBFD Subband config#2 with {DU}: 80MHz DL + 20MHz UL. | NR, 100MHz, DL | 5% | 0.15 | 1.42 |
|  |  |  |  | 50% | 0.27 | 1.03 |
|  |  |  |  | 95% | 0.44 | 0 |
| 2 | Macro -> Macro | NR, 200MHz, SBFD Subband config#2 with {DU}: 50MHz UL + 150MHz DL | NR, 200MHz, DL | 5% | -0.09 | -0.80 |
|  |  |  |  | 50% | 0.05 | 0 |
|  |  |  |  | 95% | 0.14 | 0 |

(a)

(b)

Fig 5 SBFD: SINR and Throughput losses compared to baseline: (a) FR1 Macro-Macro; (b) FR2 Macro-Macro

Observation 4: Our initial results shows the existing ACIR (ACLR/ACS) provides sufficient interference reduction in adjacent band legacy TDD. The performance degradation is below the evaluation criteria proposed in Proposal 11.

# Conclusion

In this contribution, we submit 4 observations and 12 proposals as follows:

Proposal 1: The RAN4 shall conduct co-ex study as the co-ex study will be the discussion basis for RAN4 to:

- Determine the supporting co-ex scenarios for SBFD operation;

- Check the adjacent channel interference with existing ACIR (ACLR/ACS) requirements in SBFD operation;

- Check the feasibility of new ACIR (ACLR/ACS) requirements for new SBFD-capable gNBs.

Observation 1: The adjacent channel co-existence study can focus on the timeslot(s) where the aggressor SBFD having its subband UL and DL operating simultaneously while the victim legacy TDD system is in its DL timeslot(s). The rest timeslot(s) are legacy TDD co-existence cases, and the adjacent channel interference (ACI) of those cases were solved by legacy ACLR and ACS of each frequency range already.

Proposal 2: Proposed to consider both SBFD Subband configuration#1 with {DUD} pattern, which means one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth, and SBFD Subband configuration#2 with {DU} pattern, which means one SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth, in RAN4 co-ex study. These two configurations are shown in figure below:

- (b)

Fig.2 SBFD subband configurations: (a) #1 {DUD}, (b) #2 {DU}

Observation 2: The results from TR 38.828 reflects the cross-link adjacent channel interference impact focused on the FR1 and FR2 Macro-Macro case for BS-BS interference link, and FR1 Macro-Indoor case for UE-UE interference link. And RAN1 agreements also shows they will focus on Macro cases as starting point.

Proposal 3: Considering Observation 1 and 2, we propose RAN4 to consider the following scenarios in Table 2 as the starting point for SBFD co-ex study with legacy TDD system.

Table 2. SBFD adjacent channel co-existence scenarios (starting point)

| FR | Scenario No. | Deployment Scenario (Aggressor -> Victim) | Aggressor baseline | Aggressor | Victim |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Macro -> Macro | NR, 100MHz, DL ACI | NR, 100MHz SBFD1 | NR, 100MHz, DL |
|  | 2 | Macro -> Macro | SBFD Intra-system | NR, 100MHz, DL | NR, 100MHz SBFD1 |
|  | 3 | Macro -> Indoor | NR, 100MHz, DL ACI | NR, 100MHz SBFD1 | NR, 100MHz, DL |
|  | 4 | Macro -> Indoor | SBFD Intra-system | NR, 100MHz, DL | NR, 100MHz SBFD1 |
| 2 | 5 | Macro -> Macro | NR, 200MHz, DL ACI | NR, 200MHz, SBFD2 | NR, 200MHz, DL |
|  | 6 | Macro -> Macro | SBFD Intra-system | NR, 200MHz, DL | NR, 200MHz, SBFD2 |
| Note 1: For FR1, consider subband config#1 with {DUD}: 40MHz DL + 20MHz UL + 40MHz DL and subband config#2 with {DU}: 80MHz DL + 20MHz UL. Note 2: For FR2, consider subband config#1 with {DUD}: 50MHz DL + 50MHz UL + 100MHz DL and subband config#2 with {DU}: 150MHz DL + 50MHz UL. |  |  |  |  |  |

Proposal 4: Propose to adopt the system characteristics, deployment parameters and other assumptions from TR 38.828 as the starting point for SBFD co-ex study. But the assumptions should be updated accordingly to fulfill the SBFD co-ex purpose.

Proposal 5: Our understanding is that the SBFD BS could 1) utilize half of its original panel for DL and UL each, or 2) the SBFD BS could implement an extra panel with same number of elements for subband UL receiving so that the subband DL Tx will utilize same elements as legacy TDD BS. Thus, it is proposed for the meeting to consider two options for SBFD BS antenna and TRP power:

- Option 1: Utilize half of its original panel for SBFD UL and DL each. In this case, the TRP and elements number for DL and UL in SBFD BS will be half of the TDD BS configuration.

- Option 2: Utilize an extra panel for subband UL operation. In this case, the TRP and element number for DL and UL in SBFD BS will be the same as TDD BS configuration.

Proposal 6: Propose to consider two options of SBFD BS power and elements based on what was assumed TR 38.828, while the victim legacy TDD BS assumption stays the same. The detailed proposed changes for SBFD BS can be found in the highlighted part of above tables.

|  | FR1 Macro Urban | FR2 Macro Urban |
| --- | --- | --- |
| BS antenna configurations | For Legacy TDD: (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ For SBFD: Option 1: (Mg,Ng,M,N,P)=(1,1,4,8,2) (dH,dV)=(0.5,0.8)λ Option 2: (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ | For 30 GHz legacy TDD: (1, 1, 8, 16, 2) For SBFD: Option 1 (1, 1, 8, 8, 2) Option 2 (1, 1, 8, 16, 2) |
| BS Tx power | For Legacy TDD: 49 dBm For SBFD: Option 1: 46 dBm Option 2: 49 dBm | For legacy TDD: 43dBm For SBFD: Option 1: 40 dBm Option 2: 43 dBm |

Proposal 7: Propose to align the mechanical down-tilt angles assumptions for BS. And it is proposed to use 6 degrees for the Macro BS for FR1 and FR2 as provided in TR 38.803.

Proposal 8: Propose to use the following as the general uplink power control model for SBFD co-existence study.

For downlink scenario, no power control scheme is applied. For uplink scenario, TPC model specified in Section 9.1 TR 36.942 [9] is applied with following parameters. - CLx-ile = –SNR_target + UE_maxpower – ThermalNoise – BS_NoiseFigure + 10*log10(BW) - γ = 1 Where, SNR_target for FR1 and FR2 are 15 dB.

Proposal 9: For FR1, it is proposed to adopt ACLR1 as 30 dBc and ACLR2 as 43 dBc for FR1 UE.

Proposal 10: Propose to adopt the following steps, modified from TR 38.828 and other general legacy RAN4 coex study report, as the simulation steps for SBFD coex study.

1. Aggressor and victim network are generated. - UEs are distributed randomly across the network. 2. UE associations: UEs are associated to base station based on coupling loss. - Associations are made assuming a single element at both UE and BS. 3. Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks. 4. When legacy TDD system is victim, follow steps 4a: 4a. Throughput is computed considering ACI from another static TDD system as baseline aggressor: - Thputbaselinebpshz=fSINRICI+ACI_baseline=fSN+IICI+IACI_baseline, where IICI is the inter-cell interference and IACI_baseline is the adjacent channel interference from the baseline aggressor. When SBFD system is victim, follow steps 4b: 4b. Throughput is computed in the victim system without considering ACI as below: - Thputbaselinebpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference. 5. Throughput is computed considering ACI below: - ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

Proposal 11: It is proposed to follow the TR 38.828’s evaluation criteria to check the 50% and 5% throughput loss compared to the baseline scenario as described in table 2 (Proposal 3).

Proposal 12: In the above step 4b and 5, when SBFD is victim system, it is proposed to use {N = noise floor + 1dB} for the SBFD system to simulate the self-interference impact as a simplified method to evaluate its SINR/throughput.

Observation 3: The CLI SINR and Throughput distribution of our simulation platform seems quite aligned with the results in TR 38.828.

Observation 4: Our initial results shows the existing ACIR (ACLR/ACS) provides sufficient interference reduction in adjacent band legacy TDD. The performance degradation is below the evaluation criteria proposed in Proposal 11.

# Reference

[1]. RP-221352, Revised SID: Study on evolution of NR duplex operation, CMCC.

[2]. RP-221351, Status report for study on evolution of NR duplex operation, CMCC.

[3]. Chair's notes RAN1#109-e, May 2022

[4]. TR 36.942 v17.0.0

[4]. TR 38.828 v16.1.0

[5]. TR 38.803 v14.3.0

[6]. R4-2212486, Discussion on interference modelling for duplex evolution, Samsung.