3GPP TSG-RAN WG4 Meeting #104-e                                            	R4-2212705

Electronic Meeting, 15th August-26th August, 2022

Source:	Samsung

Title:	Discussion and simulation results on demodulation requirement for Enhancement on HST-SFN deployment

Agenda item:		9.17.4.1.1

Document for:	Discussion

- Introduction

In last RAN4 103-e meeting, test setup for PDSCH requirement for HST-SFN scheme A with single carrier has agreements in WF [1] as below.  In this contribution, we share our simulation results and views for information.

Issue 2-2-1: Maximum Doppler shift - RAN4 will make final decision about the value of Maximum Doppler shift in RAN4#104-e meeting based on the simulation results, e.g. compare the required SNR to achieve 70% of peak rate with MCS17 (or MCS13) with rank 2. - Option 1: 870Hz (baseline) - Option 2: 972Hz - Encourage interested companies provide the simulation results for both Options in RAN4#104-e meeting. Issue 2-2-2: MCS & Rank - MCS17 rank 2 as the baseline. - RAN4 may consider MCS13 rank 2 according to the simulation results Issue 2-2-3: UE capability The PDSCH demodulation requirements for HST-SFN Scheme A should be applicable for UE capable of “23-6-1 SFN scheme A (scheme 1) for PDSCH and PDCCH”.

Besides, there is still no conclusion on whether to introduce PDSCH requirements for HST-SFN scheme B.  In this contribution we also provide our simulation results and views on this open issue based on the agreed TRP pre-compensation modelling in [1].

Issue 2-1-1: Whether to define PDSCH requireemnt with HST-SFN scheme B - Option 1: Introduced the PDSCH requirement with HST-SFN B under test applicability rule - FFS on the pre-compensation modeling - Option 2: Not to introduce the PDSCH requirement with HST-SFN B Issue 2-3-1: Modeling of TRP pre-compensation - Assume perfect modeling of TRP pre-compensation as baseline. - Interested companies can evaluate the following model: - The frequency shift FD,k (Hz) from kth RRH is given by: FD,k=fc×real-v×y-xky-xk×C for k*Ds≤a<k*Ds+Ds and FD,k+1=FD,k+F∆,k+1 for k*Ds≤a<k*Ds+Ds where F∆,k=+0.1 ppm*fc , k mod 2=0-0.1 ppm*fc , k mod 2=1

- Simulation results of SFN scheme A

We evaluate SFN scheme A performance based on below simulation assumptions.

Channel model we used is similar as 3GPP38.101-4 B.3.2 HST-SFN [2], removing 2 taps. Related Doppler shift trajectory and relative power level trajectory are as Fig.1.

Fig.1 Doppler shift trajectory and relative power level trajectory of maximum Doppler shift 870Hz

- Simulation results for FDD

Fig.2 Throughput performance of FDD MCS13 and MCS17

- Simulation results for TDD

Fig.3 Throughput performance of TDD MCS13 and MCS17

The SNR points at 70% of peak rate for different test cases are summarized in the following table：

Table 2.1: SNR point at 70% peak rate of different cases

| Case Number | Duplex | MCS level | Maximum Doppler(Hz) | Antenna Configuration | SNR(dB) point |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD | 13 | 870 | 2x2 | 9.5 |
| 2 | FDD | 13 | 972 | 2x2 | 10.3 |
| 3 | FDD | 13 | 870 | 2x4 | 6.1 |
| 4 | FDD | 13 | 972 | 2x4 | 6.9 |
| 5 | FDD | 17 | 870 | 2x2 | 13.1 |
| 6 | FDD | 17 | 972 | 2x2 | 16.1 |
| 7 | FDD | 17 | 870 | 2x4 | 10.0 |
| 8 | FDD | 17 | 972 | 2x4 | 12.6 |
| 9 | TDD | 13 | 1667 | 2x2 | 9.4 |
| 10 | TDD | 13 | 1667 | 2x4 | 6.3 |
| 11 | TDD | 17 | 1667 | 2x2 | 11.3 |
| 12 | TDD | 17 | 1667 | 2x4 | 8.3 |

Observation1: for FDD, compared with MCS13, MCS17 has larger SNR difference between maximum Doppler 870Hz and 972Hz.

Observation2: for FDD MCS17, the SNR points difference at 70% of peak rate is 2.6~3dB between maximum Doppler 870Hz and 972Hz.

Proposal 1: MCS17 and rank2 should be used for PDSCH requirement setup for HST-SFN scheme A with single carrier.

Proposal 2: Maximum Doppler shift for PDSCH requirement setup for HST-SFN scheme A with single carrier

- FDD, 870Hz

- TDD, 1667Hz

Proposal 3: SNR point at 70% peak rate for PDSCH requirement for HST-SFN scheme A with single carrier

| Case Number | Duplex | MCS level | Maximum Doppler(Hz) | Antenna Configuration | SNR(dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD | 17 | 870 | 2x2 | 13.1 |
| 2 | FDD | 17 | 870 | 2x4 | 10.0 |
| 3 | TDD | 17 | 1667 | 2x2 | 11.3 |
| 4 | TDD | 17 | 1667 | 2x4 | 8.3 |

- Simulation results of SFN scheme B

According the WF [1], Simulation assumption of scheme B is as same as scheme A, except the Channel model. Assume single tap channel model as the baseline, provide simulation result of perfect modeling of TRP pre-compensation, and also simulate the delta Doppler shift is  0.1 ppm*fc (Fig.4 shows the Doppler shift trajectory of FDD and TDD cases).

Table 3.1: SNR point at 70% peak rate of different cases

| Case Number | Duplex | MCS level | Antenna Configuration | Max Doppler shift | others | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD | 17 | 2x2 | 870Hz | single tap | 11.7 |
| 2 | FDD | 17 | 2x2 | 870Hz | 2 taps and 0Hz delta between 2 taps | 12.8 |
| 3 | FDD | 17 | 2x2 | 870Hz | 2 taps and 0.1ppm delta between 2 taps | 13.6 |
| 4 | FDD | 17 | 2x4 | 870Hz | single tap | 8.4 |
| 5 | FDD | 17 | 2x4 | 870Hz | 2 taps and 0Hz delta between 2 taps | 9.9 |
| 6 | FDD | 17 | 2x4 | 870Hz | 2 taps and 0.1ppm delta between 2 taps | 10.6 |
| 7 | TDD | 17 | 2x2 | 1667Hz | single tap | 11.3 |
| 8 | TDD | 17 | 2x2 | 1667Hz | 2 taps and 0Hz delta between 2 taps | 13.8 |
| 9 | TDD | 17 | 2x2 | 1667Hz | 2 taps and 0.1ppm delta between 2 taps | 14.6 |
| 10 | TDD | 17 | 2x4 | 1667Hz | single tap | 8.4 |
| 11 | TDD | 17 | 2x4 | 1667Hz | 2 taps and 0Hz delta between 2 taps | 11.2 |
| 12 | TDD | 17 | 2x4 | 1667Hz | 2 taps and 0.1ppm delta between 2 taps | 11.9 |

Observation3: the SNR point of HST-SFN scheme B with perfect modeling of TRP pre-compensation, is 1.1~2.8dB worse than that of HST single tap channel modeling.

Proposal 4:  Introduce PDSCH requirements for HST-SFN scenario with scheme B.

Proposal 5: For PDSCH requirements for HST-SFN scheme B, both perfect modelling of TRP pre-compensation and delta Doppler shift 0.1ppm*fc modelling are acceptable.

Proposal 6: SNR point at 70% peak rate for PDSCH requirement for HST-SFN scheme B with single carrier

| Case Number | Duplex | MCS level | Antenna Configuration | Max Doppler shift | others | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | FDD | 17 | 2x2 | 870Hz | 2 taps and 0Hz delta between 2 taps | 12.8 |
| 3 | FDD | 17 | 2x2 | 870Hz | 2 taps and 0.1ppm delta between 2 taps | 13.6 |
| 5 | FDD | 17 | 2x4 | 870Hz | 2 taps and 0Hz delta between 2 taps | 9.9 |
| 6 | FDD | 17 | 2x4 | 870Hz | 2 taps and 0.1ppm delta between 2 taps | 10.6 |
| 8 | TDD | 17 | 2x2 | 1667Hz | 2 taps and 0Hz delta between 2 taps | 13.8 |
| 9 | TDD | 17 | 2x2 | 1667Hz | 2 taps and 0.1ppm delta between 2 taps | 14.6 |
| 11 | TDD | 17 | 2x4 | 1667Hz | 2 taps and 0Hz delta between 2 taps | 11.2 |
| 12 | TDD | 17 | 2x4 | 1667Hz | 2 taps and 0.1ppm delta between 2 taps | 11.9 |

- Conclusion

In this contribution, we provide simulation results for PDSCH demodulation requirement for Enhancement on HST-SFN deployment, based on the agreements in RAN4 #103-e meeting. We compare different test cases under FDD and TDD mode. According to the above results, we observe that:

Observation1: for FDD, compared with MCS13, MCS17 has larger SNR difference between maximum Doppler 870Hz and 972Hz.

Observation2: for FDD MCS17, the SNR points difference at 70% of peak rate is 2.6~3dB between maximum Doppler 870Hz and 972Hz.

Proposal 1: MCS17 and rank2 should be used for PDSCH requirement setup for HST-SFN scheme A with single carrier.

Proposal 2: Maximum Doppler shift for PDSCH requirement setup for HST-SFN scheme A with single carrier

- FDD, 870Hz

- TDD, 1667Hz

Proposal 3: SNR point at 70% peak rate for PDSCH requirement for HST-SFN scheme A with single carrier

| Case Number | Duplex | MCS level | Maximum Doppler(Hz) | Antenna Configuration | SNR(dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD | 17 | 870 | 2x2 | 13.1 |
| 2 | FDD | 17 | 870 | 2x4 | 10.0 |
| 3 | TDD | 17 | 1667 | 2x2 | 11.3 |
| 4 | TDD | 17 | 1667 | 2x4 | 8.3 |

Observation3: the SNR point of HST-SFN scheme B with perfect modeling of TRP pre-compensation, is 1.1~2.8dB worse than that of HST single tap channel modeling.

Proposal 4:  Introduce PDSCH requirements for HST-SFN scenario with scheme B.

Proposal 5: For PDSCH requirements for HST-SFN scheme B, both perfect modelling of TRP pre-compensation and delta Doppler shift 0.1ppm*fc modelling are acceptable.

Proposal 6: SNR point at 70% peak rate for PDSCH requirement for HST-SFN scheme B with single carrier

| Case Number | Duplex | MCS level | Antenna Configuration | Max Doppler shift | others | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | FDD | 17 | 2x2 | 870Hz | 2 taps and 0Hz delta between 2 taps | 12.8 |
| 3 | FDD | 17 | 2x2 | 870Hz | 2 taps and 0.1ppm delta between 2 taps | 13.6 |
| 5 | FDD | 17 | 2x4 | 870Hz | 2 taps and 0Hz delta between 2 taps | 9.9 |
| 6 | FDD | 17 | 2x4 | 870Hz | 2 taps and 0.1ppm delta between 2 taps | 10.6 |
| 8 | TDD | 17 | 2x2 | 1667Hz | 2 taps and 0Hz delta between 2 taps | 13.8 |
| 9 | TDD | 17 | 2x2 | 1667Hz | 2 taps and 0.1ppm delta between 2 taps | 14.6 |
| 11 | TDD | 17 | 2x4 | 1667Hz | 2 taps and 0Hz delta between 2 taps | 11.2 |
| 12 | TDD | 17 | 2x4 | 1667Hz | 2 taps and 0.1ppm delta between 2 taps | 11.9 |

Reference

[1] R4-2210670, “WF on demodulation requirement for Enhancement on HST-SFN deployment”, Ericsson, 3GPP RAN4 #103-e meeting

[2] 3GPP TS 38.101-4 v17.3.0, “NR User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 17)”