# 3GPP TSG-RAN WG4 Meeting # 104-e                                 R4-2212717

# Electronic Meeting, Aug. 15-Aug. 26, 2022

# Title: 	Discussion on non-collocated EN-DC and NR-CA

# Source: 	ZTE Corporation

# Agenda item:	11.11.2

# Document for:	Approval

# 1 Introduction

In Rel-18, a new WID [1] to support of intra-band non-collocated EN-DC/NR-CA deployment was approved due to the demands from operator, where the core part objectives are:

- Phase I:

  - Study the feasibility to support non-co-located scenario for FR1 intra-band non-contiguous EN-DC/NR-CA except for 2-layer case of EN-DC with supporting the UE capability of interBandMRDC-WithOverlapDL-Bands-r16 already specified in Rel-16 and Rel-17.

    - Investigate the tolerable power imbalance between carriers

    - Investigate the required arrival time difference between CCs

    - Investigate the additional impacts of contiguous case, if time units are available.

    - Evaluate the UE performance under the power imbalance and arrival time difference

      - Discuss and decide reference UE architecture considering the UE capability of interBandMRDC-WithOverlapDL-Bands-r16 for 2-layer MIMO case for NR-CA, and 4-layer MIMO case for both EN-DC/NR-CA

        - NOTE 1: For UE capable of supporting 2-layer MIMO, only assuming power imbalance 25dB

        - NOTE 2: For UE capable of supporting 4-layer MIMO, 2-layer MIMO power imbalance assumption can be considered as a base line but another value or the same power imbalance with different throughput performance requirement is not precluded.

        - NOTE 3: RAN4 is recommended to start the work on 2-layer first and after that start 4-layer work based on the conclusion of 2-layer work.

    - Work is limited to CA/EN-DC for EN-DC/NR-CA for bands 42, n77/n78

    - Investigate whether the power imbalance should be explicitly (e.g. as an RF requirement) or implicitly specified (e.g. through a demodulation performance test). Specify the power imbalance based on the outcome of the investigation.

    - If any change in RAN1 or RAN2 spec is needed, it will be triggered by RAN4 LS

- Phase II:

  - Phase II work will get started after the feasibility in phase I is confirmed

  - Specify MRTD and MTTD requirements in non-collocated deployment

  - Discuss and decide if the different requirements will be specified based on UE capability of interBandMRDC-WIthOverlapDL-Bands-r16.

In this contributions, we give some initial discussions on the RF requirements for non-collocated deployment for inter-band ENDC and intra-band non-contiguous NR CA.

# 2	Discussion

First, we think some ambiguities existed in the existing objectives. In terms of the above objectives, it seems it mixed intra-band non-contiguous ENDC with inter-band non-contiguous ENDC according to the following two bullets:

  - Study the feasibility to support non-co-located scenario for FR1 intra-band non-contiguous EN-DC/NR-CA except for 2-layer case of EN-DC with supporting the UE capability of interBandMRDC-WithOverlapDL-Bands-r16 already specified in Rel-16 and Rel-17.

....

    - Work is limited to CA/EN-DC for EN-DC/NR-CA for bands 42, n77/n78

We can see that this work is limited for bands 42, n77/n78. However, it is impossible to implement intra-band non-contiguous ENDC due to band n42 is not supported in current TS38.101-1, which means there are no DC_42_n42 configurations. Note that DC_42-n77/n78 belong to inter-band ENDC, where the intra-band contiguous/non-contiguous RF requirements (associated with power imbalance), or inter-band RF requirements (associated with power imbalance) are applied which rely on the co-located or non-co-located deployment.

In addition, in terms of some background provided in [2]:

Therefore, to support non-co-located scenario, the scope should be:

1. 2-layer MIMO case for intra-band non-contiguous NR CA, i.e. NR CA n77(xA)/n78(xA) type 2 UE;

2. 4-layer MIMO case for both intra-band non-contiguous NR CA and inter-band non-contiguous ENDC, i.e. NR CA n77(xA)/n78(xA) and ENDC 42-n77/n78 new type UE(?), respectively.

Although there were several intra-band non-contiguous ENDC configurations defined in the TS38.101-3, the work in this WID is limited to the bands 42, n77/n78, which means non-co-located scenario for intra-band non-contiguous ENDC is out of the scope, instead it should be inter-band ENDC (here it is 4-layer MIMO case in the WID). In the other words, if non-co-located scenario is supported for intra-band non-contiguous ENDC, then the bands should not be limited to the bands 42, n77/n78.

Observation: Non-co-located scenario for intra-band non-contiguous ENDC is impossible if the work is limited to CA/EN-DC for EN-DC/NR-CA for bands 42, n77/n78, instead it should be inter-band EN-DC.

Proposal 1. The objectives needs to be updated.

In our understanding, the RF architecture are generally the same for ENDC and NR CA for the same band combination. Therefore, for 2 layer MIMO case for intra-band non-contiguous NR CA n77/n78 supporting non-co-located scenario, the situation is quite similar with ENDC 42-n77/n78, where no filtering between the 2 carriers which cause power imbalance between carriers [3]. Therefore, we think 25dBc power imbalance of existing inter-band ENDC type 2 UE which corresponding to 1dB REFSEN degradation can be reused. It shall be noted that same power imbalance of 6dB are applied for both inter-band ENDC and intra-band non-contiguous CA type 1 UE.

Proposal 2. For 2 layer MIMO case for intra-band non-contiguous CA supporting non-collocated deployment, 25dBc power imbalance of inter-band ENDC type 2 UE which corresponding to 1dB REFSEN degradation can be reused.

For 4 layer MIMO case for intra-band non-contiguous CA and inter-band ENDC supporting non-co-located, the power imbalance should be FFS.

By looking at the following texts for inter-band ENDC type 2 UE:

The applicability of these test configurations is shown below for type 2 UE which is indicated by capability

interBandMRDC-WithOverlapDL-Bands-r16:

When the capability interBandContiguousMRDC is indicated, test configuration 1 or 2 can be used to verify the RX

power imbalance requirement for type 2 UE.

When the capability interBandContiguousMRDC is absent, test configuration 1, 2 or 3 can be used to verify the RX

power imbalance requirement for type 2 UE.

It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE.

For type 2 UEs indicating interBandMRDC-WithOverlapDL-Bands-r16 for the following EN-DC band combinations in

Table 7.6B.2.6-2, the Rx requirements for four Rx ports do not apply for each band.

From TS38.306, the capability interBandMRDC-WithOverlapDL-Bands-r16 is only for ENDC/NEDC. Obviously, new signaling should be introduced for intra-band non-contiguous CA supporting non-co-located.

Moreover, for inter-band ENDC type 2 UE, Rx requirements for four Rx ports do not apply for each bands, i.e. 2Rx in LTE and NR band, respectively.

For band n77/n78, four Rx ports are mandatory supported. Therefore, for intra-band non-contiguous CA supporting non-collocated deployment, we think Rx requirements for four Rx ports can be applied. In addition, considering 8Rx ports are under discussing, whether or not supporting Rx requirements for 8 RX ports is FFS.

Proposal 3. To support non-collocated deployment for intra-band non-contiguous CA:

- New signaling(s) should be introduced

- Rx requirements for four Rx ports are applied

- Whether or not supporting Rx requirements for 8 RX ports is FFS

# 3 Conclusion

In this contribution, we give some initial discussions on the RF requirements for non-collocated deployment for inter-band ENDC and intra-band non-contiguous NR CA.

We think there exist ambiguities in the objectives due to intra-band non-contiguous ENDC is not corrected if the work is limits to CA/EN-DC for EN-DC/NR-CA for bands 42, n77/n78.

The conclusions are:

Observation: Non-co-located scenario for intra-band non-contiguous ENDC is impossible if the work is limited to CA/EN-DC for EN-DC/NR-CA for bands 42, n77/n78, instead it should be inter-band EN-DC.

Proposal 1. The objectives needs to be updated.

Proposal 2. For 2 layer MIMO case for intra-band non-contiguous CA supporting non-collocated deployment, 25dBc power imbalance of inter-band ENDC type 2 UE which corresponding to 1dB REFSEN degradation can be reused.

Proposal 3. To support non-collocated deployment for intra-band non-contiguous CA:

- New signaling(s) should be introduced

- Rx requirements for four Rx ports are applied

- Whether or not supporting Rx requirements for 8 RX ports is FFS

# 4	Reference

[1]  RP-221809, New WID: Support of intra-band non-collocated EN-DC/NR-CA deployment, KDDI

[2]  RP-221004, [95e-13-RAN4-R18-NonColloc] Summary of Extended discussion, Moderate(KDDI)

[3]  R4-2111732, Imbalance Requirement for Type 2 UE RX, Qualcomm