3GPP TSG-RAN WG4 Meeting #104-e	R4-2212747

Electronic Meeting, Aug. 15- Aug. 26, 2022

Agenda Item:	9.10.1.1

Source:	Ericsson

Title:	Remaining issues on inter-cell interference IRC

Document for:	Discussion

# 1	Introduction

In the last meeting, RAN4 discussed UE demodulation and CSI requirements of MMSE-IRC receiver for suppressing inter-cell interference. The WF for inter-cell interference MMSE-IRC is agreed in [1].

In this contribution, we will continue to discuss the remaining issues on MMSE-IRC receiver.

# 2	PDSCH Demodulation issues

PDCCH interference modelling

The remaining issue for interference mode PDCCH interference configuration is listed below.

- Considered options - Option 1: All the REs in control region filled with QPSK randomly modulated symbols with random precoding for the number of antenna ports. - Option 2: Assume PDCCH transmission from interference cells, and for each cell, OCNG signal is transmitted on each RE that is not occupied by the PDCCH of this cell. - Option 3: Assume PDCCH transmission from interference cells which is non-overlapping with serving cell.

From our understanding, PDCCH performance is very robust to inter-cell interference and there is no PDCCH impact on PDSCH performance. It is also impractical to expect non-overlapping PDCCH between serving cell and interference cells. Thus, we assume PDCCH transmission overlapping from interference cells.

Proposal 1: RAN4 to assume PDCCH transmission overlapping with interference cells with the following configuration.

- All the REs in the control region filled with QPSK randomly modulated symbols with random precoding for the number of antenna ports.

SNR deriving rule

In the last meeting, the rule to deriving SNR points were discussed. RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes X dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within X dB. Based on current simulation results, the span for inter-cell and intra-cell inter-user cases is mostly larger than 3dB. The best span cases are still larger than 2dB. Thus, we propose to use 2.5dB to leave some margins to consider most companies results.

Proposal 2: RAN4 to use 2.5dB span to derive the results.

# 2	CSI reporting issues

T-put gain for CSI reporting

Based on our simulation results, the relative throughput ratio is around 1 for MRC/MRC as expected since there is no gain when only MRC receiver is applied. The gain is achieved by the IRC/IRC from the UE demodulation part which can be seen in our simulation results. In legacy LTE CQI reporting, the relative throughput ratio γ is defined as 1.8 in SINR =-2dB to differentiate the different UE implementations. Based on our observations for different simulation results, more relative throughput ratio difference happens in a low SINR. Therefore, we are suggesting defining γ equaling 2 to obtain enough Throughput gain from IRC demodulation and CQI receiver compared with MRC receiver.

Proposal 3: RAN4 to define CQI reporting test metric with γ=2 for 2Rx.

Proposal 4: RAN4 to define CQI reporting test metric with γ=2 for 4Rx.

# 3	Summary

In this contribution, we further discuss the remaining issues on MMSE-IRC receiver for suppressing inter-cell interference.

Proposal 1: RAN4 to assume PDCCH transmission overlapping with interference cells with the following configuration.

- All the REs in the control region filled with QPSK randomly modulated symbols with random precoding for the number of antenna ports.

Proposal 2: RAN4 to use 2.5dB span to derive the results.

Proposal 3: RAN4 to define CQI reporting test metric with γ=2 for 2Rx.

Proposal 4: RAN4 to define CQI reporting test metric with γ=2 for 4Rx.

# References

- R4-2210681, “WF on PDSCH demod and CSI requirements for inter-cell interference MMSE-IRC”, Ericsson