3GPP TSG-RAN WG4 Meeting #104-e 	               R4-2212748

Electronic Meeting, Aug. 15- Aug 26, 2022

Agenda Item:	9.10.1.1

Source:	Ericsson

Title:	Simulation results on PDSCH demodulation for inter-cell interference

Document for:	Information

# 1	Introduction

In this contribution, the MMSE-IRC for suppressing inter-cells’ interference performance simulation results are shown.

PDSCH demodulation simulation assumption

The detailed simulation assumptions for PDSCH demodulation are listed in table below.

| Parameters | Serving cell FDD | Serving cell TDD | Interferer cell(s) |
| --- | --- | --- | --- |
| Network scenario | Sync. |  |  |
| Number of carriers | single carrier |  |  |
| Carrier frequency | [2 GHz] | [4GHz] |  |
| PDSCH allocation in frequency domain | Full PRB |  |  |
| Physical cell ID | 0 | 0 | 1 |
| Active DL BWP index | 1 | 1 | N/A |
| Channel bandwidth | 10MHz | 40MHz |  |
| TDD configuration | N/A | 7DS2U, S=6D+4G+4U | 7DS2U, S=6D+4G+4U |
| SCS | 15kHz | 30kHz |  |
| PDCCH allocation in time domain | symbols #0 and #1 of each slot for PDCCH | symbols #0 and #1 of each slot for PDCCH | symbols #0 and #1 of each slot for PDCCH |
| SSB configuration | SSB index #0 | SSB index #0 | SSB index #0 |
| TRS configuration | k0=0 for CSI-RS resource 1,2,3,4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 Density 3, 1 port FDD: Periodicity 20 slots; Offset 10 for CSI-RS resource 1 and 2; 11 for CSI-RS resource 3 and 4 TDD: Periodicity 40 slots; Offset 20 for CSI-RS resource 1 and 2; 21 for CSI-RS resource 3 and 4 |  | Same as Serving cell |
| PDSCH configuration | Type A, Start symbol 2, Duration 12​ |  | Type A, Start symbol 2, Duration 12​ |
| PRB bundling size for target PDSCH | 2 |  | 2 |
| PDSCH DMRS configuration | Type 1, 1+1 |  | Type 1, 1+1 |
| Scheduling with data multiplexed on the DMRS symbols | One single symbol front loaded and 1 additional DMRS |  | One single symbol front loaded and 1 additional DMRS |
| MCS | MCS 13 (16QAM, CR=0.5) |  | 16QAM |
| Rank | 1 |  | random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s) |
| PDSCH precoder | Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2. |  |  |
| Antenna configuration | 2x2, 2x4 |  |  |
| Antenna Correlation | ULA Low |  |  |
| Propagation condition | Homogeneous: TDLC300-100 Heterogeneous: TDLA30-10 |  |  |
| Time Offset relative to serving cell | FDD 15KHz: Cell 2 3 us, Cell 3 -1us TDD 30KHz: Cell 2 1.5 us, Cell 3 -0.5us |  |  |
| Metric | SNR to achieve 70% of maximum throughput |  |  |
| Number of HARQ Processes | 4 | 8 | N/A |
| Maximum HARQ transmission | 4 |  | N/A |

| NW Deployment | SNR (dB) for cell 2 | SNR (dB) for cell 3 | SNR for serving cell (cell 1) |
| --- | --- | --- | --- |
| Homogeneous | 7.77 | 2.29 | [-6:2:20] |
| Heterogeneous | 7.58 | - |  |

# 2	Simulation Results

The simulation results based on current agreements are summarized as follow.

Table 1. simulation results for FR1 FDD

| Parameters |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Duplex mode | Deployment | Number of UE Rx antenna | INR1 | INR2 | MCS | MMSE-MRC | MMSE-IRC | MMSE-IRC Gain |
| FDD (15 kHz, 10 MHz) | Homogeneous (TDL-C) | 2 Rx | 7,77 | 2,29 | MCS 13 | 15.1 | 14.0 | 1.1 |
|  |  | 4 Rx | 7,77 | 2,29 | MCS 13 | 11.2 | 8.1 | 3.1 |
|  | Heterogeneous (TDL-A) | 2 Rx | 7,58 | N/A | MCS 13 | 12.6 | 11.0 | 1.6 |
|  |  | 4 Rx | 7,58 | N/A | MCS 13 | 8.1 | 4.2 | 3.9 |

Table 2. simulation results for FR1 TDD

| Parameters |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Duplex mode | Deployment | Number of UE Rx antenna | INR1 | INR2 | MCS | MMSE-MRC | MMSE-IRC | MMSE-IRC Gain |
| TDD (30 kHz, 40 MHz) | Homogeneous (TDL-C) | 2 Rx | 7,77 | 2,29 | MCS 13 | 15.2 | 14.3 | 0.9 |
|  |  | 4 Rx | 7,77 | 2,29 | MCS 13 | 11.4 | 8.7 | 2.7 |
|  | Heterogeneous (TDL-A) | 2 Rx | 7,58 | N/A | MCS 13 | 12.5 | 10.5 | 2 |
|  |  | 4 Rx | 7,58 | N/A | MCS 13 | 8.5 | 4.5 | 4 |

The detailed simulation results are shown in the appendix.

# References

- R4-2120707, “Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel Corporation.

- R4-2203008, “Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel Corporation.

# Appendix

- FR1 FDD

- Homogeneous (TDL-C)

- Heterogeneous (TDL-A)

- FR1 TDD

- Homogeneous (TDL-C)

- Heterogeneous (TDL-A)