3GPP TSG-RAN WG4 Meeting #104-e	R4-2212752

Electronic Meeting, Aug 15- Aug 26, 2022

Agenda Item:	9.18.3.1.2

Source: 	Ericsson

Title:	Discussions on RedCap mobility requirements

Document for:	Discussion

# Introduction

RAN4 has discussed the impact on RRM Handover requirements due to NCD-SSB in last meeting and most issues were agreed [1].

In this contribution we further provide our view on the remaining open issues.

# Discussion

## NCD-SSB related Handover

In last meeting, RAN4 had discussed NCD-SSB based handover and sent a LS to RAN2 to confirm whether it’s possible to support NCD-SSB based handover for RedCap UE. RAN4 had also agreed to define requirements for neighbour cell measurements based on NCD-SSB.  During the handover, the UE will be configured by the old serving cell to transmit RACH either in initial BWP or in the RedCap BWP of the target cell. NW may also configure either CD-SSB or NCD-SSB measurements to UE.

Trs

In legacy handover requirement, Trs is defined as follow. If NW configures SMTC in handover command, Trs will use the SMTC configuration. Otherwise, Trs will equal the SMTC configuration in measObjectNR. If NW still not configure the SMTC in measObjectNR, UE will assume the Trs=5ms. There is no requirement if the SSB transmission periodicity is not 5ms.

Mismatch between SMTC configurations in scenario 1, 2, 3 and 4 Candidate options: - Option 1 (E///, vivo): RAN4 to further discuss the possible additional delay due to SMTC configuration mismatch, such as between CD-SSB measurement and NCD-SSB HO without default SMTC configuration. - Option 1a (CMCC, HW): Clarification needed in spec. - Option 2 (Apple, Xiaomi): FFS, some clarification may be needed. - Option 3 (MTK): No need to discuss the issue.

In RedCap, UE can perform HO based on CD-SSB or NCD-SSB. A SMTC configuration mismatch should be further considered. For example, NW configures UE handover to the specific RedCap BWP with NCD-SSB without NCD-SSB SMTC configuration in HO command. UE can further refer on the measObjectNR having the same SSB frequency and SCS. After that, for example, if there is no NCD-SSB related periodicity configuration in measObjectNR, UE can further check the information from CD-SSB.

Proposal 1: UE should check both CD-SSB and NCD-SSB configuration in the measObjectNR when NW doesn’t configure the SMTC in HO command.

# Summary

In this contribution we have discussed the mobility requirements for RedCap, and made following observations and proposals:

Proposal 1: UE should check both CD-SSB and NCD-SSB configuration in the measObjectNR when NW doesn’t configure the SMTC in HO command.

# References

- R4-2210592, “WF on RedCap RRM requirements”, Ericsson