3GPP TSG-RAN WG4 Meeting#104-e	R4-2212782

# Electronic meeting, 15 – 26 August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | CRNum | rev | - | Current version: | 15.18.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Carrier resource grid mapping to channel raster and use of UE-specific bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are different interpretations and understanding of the parameter carrierBandwidth in SIB1 in the field and whether the UE specific (regular) channel bandwidths configured by the IE ServingCellConfig have to be located on the channel raster. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is specified that the carrier resource grid with size as indicated by carrierBandwidth in SIB1 shall be mapped to the channel raster, but this need not necessarily be the case for UE-specific channel bandwidths configured by the BS within the BS transmission bandwidth configuration (the said carrier resource grid). The CA channel spacing and TX-RX separation with UE-specific channel bandwidths are also specified. Clause 5.3.1: the specification of the channel bandwidth is aligned with 38.104 not to suggest that there are multiple carriers per cell (other than for SUL in the UL). Clause 5.3.4: the requirements apply for the channel bandwidth applied by the UE (initial access) or by BS configuration. Clause 5.4.4: specification of the TX-RX for UE specific channel bandwidths. Clause 5.4.2.2: clarification that the carrier resource grid with size as indicated by carrierBandwidth in signaling of common parameters (including SIB1) shall be mapped to the channel raster for at least one numerology. That this does not apply to UE-specific channel bandwidths for any numerology is noted in an informative note. Clause 5.4A.1: the CA channel spacing for CA configurations with UE-specific channel bandwidths (configured by the BS) is specified thus defining contiguous and non-contiguous CA configurations with UE-specific bandwidths. The existing specification of nominal CA spacing is reused consistent with legacy CA operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear definitions of configurations lead to interoperability problems and UE malfunction in the field. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1, 5.3.4, 5.4.4, 5.4.2.2, 5.4A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

## 5.3	UE channel bandwidth

### 5.3.1	General

The UE channel bandwidth supports a single NR RF carrier or parts thereof in the uplink or downlink at the UE. From a BS perspective, different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the BS. Transmission of multiple carriers to the same UE (CA) or multiple bandwidth parts to different UEs within the BS channel bandwidth can be supported.

From a UE perspective, the UE is configured with one or more BWP / carriers, each with its own UE channel bandwidth. The UE does not need to be aware of the BS channel bandwidth or how the BS allocates bandwidth to different UEs.

The placement of the UE channel bandwidth for each UE carrier is flexible but can only be completely within the BS channel bandwidth.

The relationship between the channel bandwidth, the guardband and the maximum transmission bandwidth configuration is shown in Figure 5.3.1-1.

Figure 5.3.1-1: Definition of the channel bandwidth and the maximum transmission bandwidth configuration for one NR channel

### 5.3.2	Maximum transmission bandwidth configuration

< text omitted >

### 5.3.4	RB alignment

For each numerology, its common resource blocks are specified in Clause 4.4.4.3 in TS 38.211 [6], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to "Reference point A" in the unit of the numerology. The UE transmission bandwidth configuration is indicated by the higher layer parameter carrierBandwidth [7] and will fulfil the minimum UE guardband requirement specified in Clause 5.3.3 for the corresponding UE channel bandwidth applied.

< text omitted >

#### 5.4.2.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on the channel raster and the frequency location of the subcarrier of the corresponding resource element of the carrier resource grid of the RF channel is given in Table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the carrier grid of the RF channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the UE.

Table 5.4.2.2-1: Channel raster to resource element mapping

|  | NRBmod2 = 0 | NRBmod2 = 1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block number |  |  |

and nPRB are as defined in [6] and NRB is the carrier bandwidth given by carrierBandwidth indicated in downlinkConfigCommon or uplinkConfigCommon or supplementaryUplink for the relevant transmission direction.

NOTE: 	the corresponding resource element of the transmission bandwidth configuration of a UE-specific channel bandwidths as configured in the IE ServingCellConfig does not have to be mapped to an RF reference frequency on the channel raster for any numerology.

#### 5.4.2.3	Channel raster entries for each operating band

< text omitted >

### 5.4.4	TX–RX frequency separation

The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands is specified in Table 5.4.4-1.

The TX-RX frequency separation between uplink and downlink UE specific channel bandwidths configured in the IE ServingCellConfig is the frequency separation between their centre frequencies for each numerology. This also applies for the TX-RX frequency separation of asymmetric UE specific channel bandwidths as specified in sub-clause 5.3.6. The center frequency of a UE specific channel bandwidth for each numerology and transmission direction is the frequency location of the subcarrier of the resource element determined according to Table 5.4.2.2-1 but with NRB the carrier bandwidth given by carrierBandwidth configured in ServingCellConfig. For each numerology, the default TX-RX frequency separation between UE specific channel bandwidths symmetric in the downlink and uplink is specified in Table 5.4.4-1.

Table 5.4.4-1: Default UE TX-RX frequency separation

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n1 | 190 MHz |
| n2 | 80 MHz |
| n3 | 95 MHz |
| n5 | 45 MHz |
| n7 | 120 MHz |
| n8 | 45 MHz |
| n12 | 30 MHz |
| n20 | -41 MHz |
| n25 | 80 MHz |
| n28 | 55 MHz |
| n66 | 400 MHz |
| n70 | 300 MHz |
| n71 | -46 MHz |
| n74 | 48 MHz |
| NOTE 1: Void |  |

< text omitted >

## 5.4A	Channel arrangement for CA

### 5.4A.1	Channel spacing for CA

For intra-band contiguous carrier aggregation with two or more component carriers, the nominal channel spacing between two adjacent NR component carriers is defined as the following unless stated otherwise:

For NR operating bands with a 100 kHz channel raster:

while for NR operating bands without a 100 kHz channel raster:

with

n = µ0

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective NR component carriers according to Table 5.3.2-1 with values in MHz, μ0  is the largest μ value among the subcarrier spacing configurations supported in the operating band for both of the channel bandwidths according to Table 5.3.5-1 and GBChannel(i) is the minimum guard band for channel bandwidth i according to Table 5.3.3-1 for the said μ value with μ as defined in TS 38.211. In case there is no common μ value for both of the channel bandwidths, μ0=1 is selected and GBChannel(i) is the minimum guard band for channel bandwidth i according to Table 5.3.3-1 for μ=1 with μ as defined in TS 38.211.

The channel spacing for intra-band contiguous carrier aggregation can be adjusted to any multiple of least common multiple of channel raster and sub-carrier spacing less than the nominal channel spacing to optimize performance in a particular deployment scenario.

For intra-band contiguous carrier aggregation with UE specific channel bandwidth and location configurations in adjacent component carriers, the frequency separation between the center frequencies of the UE specific channel bandwidths shall be less than or equal to the nominal CA channel spacing specified above with the UE specific channel bandwidth replacing BWChannel(j) and the centre frequency of the UE specific channel bandwidth determined as in sub-clause 5.4.4. In case there is no UE specific channel bandwidth and location configurations in the adjacent carrier the RF reference frequency of the adjacent channel is used for determining the frequency separation.

For intra-band non-contiguous carrier aggregation, the channel spacing between two NR component carriers in different sub-blocks or frequency separation between UE specific channel bandwidth and location configurations in component carriers shall be larger than the nominal channel spacing defined in this clause.

### 5.4A.2	Channel raster for CA

For inter-band carrier aggregation, the channel raster requirements in clause 5.4.2 apply for each operating band.

### 5.4A.3	Synchronization raster for CA

For inter-band carrier aggregation, the synchronization raster requirements in clause 5.4.3 apply for each operating band.

### 5.4A.4	Tx-Rx frequency separation for CA

For inter-band carrier aggregation, the Tx-Rx frequency separation requirements in clause 5.4.4 apply for each operating band.

## 5.5	Void

< end of changes >