3GPP TSG-RAN WG4 Meeting # 104-e 	R4-2212790

Electronic Meeting, August 15 – August 26, 2022

Source: 	vivo

Title: 	Initial evaluation of FR2 UL 256QAM

Agenda Item:	11.7.2

Document for:	Discussion

- Introduction

The WID for FR2 UL 256QAM was agreed as follows:

UL 256QAM

- Investigate and enable UL 256QAM for FR2-1 [RAN4]

  - Study the gain, operating SNR, phase noise model and implementation aspects

  - Specify the UE RF requirements

  - First priority: Targeted power classes are PC1, PC2 and PC5

  - Second priority: Targeted power class is PC3

The FR2 DL 256QAM was introduced in R16 and a bunch of evaluation have been done. Actually, it is not easy for UE to benefit from DL 256QAM because the required operating SNR is quite high and the situation will be much critical for UL due to the implementation restriction. In this contribution, we provide our preliminary simulation results of UL 256QAM.

- Discussion

  - Simulation assumption

Our evaluation consists of two parts, link level simulation to evaluate the operating SNR that 256QAM can bring performance gain compared to 64QAM and system level simulation to show the proportion of UE that can benefit from 256QAM in specific scenario. The simulation assumptions are mainly modified from [1] and [2] which are recorded in Annex. The parameters used in this simulation are marked in red.

  - Link level simulation

- 29 GHz (n257)

Figure 1 simulation results in 29 GHz

Observation 1a: For AWGN in 29 GHz, UL 256QAM can archive performance gain when SNR >22 dB for EVM = 1% and when SNR >26 dB for EVM = 3%.

Observation 1b: For TDL-A and TDL-D in 29 GHz, UL 256QAM can archive performance gain when SNR >28 dB for EVM = 1% and when SNR >35 dB for EVM = 3%.

39 GHz (n260)

Figure 2 simulation results in 39 GHz

- 48 GHz (n262)

Figure 3 simulation results in 48 GHz

From the result of 39 GHz and 48 GHz, we can see that UL 256QAM is hard to bring any performance gain under our simulation assumption which may because the phase noise increases with frequency significantly.

Figure 4 PN difference between 30 GHz and 47GHz

Observation 2: UL 256QAM is hard to provide performance gain in both 39GHz and 48 GHz.

Another thing worth to note is that DFT-s-OFDM have better performance compared to CP-OFDM, this is because the PTRS for DFT-s-OFDM is added in time domain which means it can compensate both CPE and ICI. Considering the DFT-s-OFDM also have lower PAPR and require smaller MPR, DFT-s-OFDM is more feasible to bring enough benefit from UL 256QAM. As for CP-OFDM, if it hardly brings performance gain, we may not need to further discuss its relevant requirements.

Observation 3: DFT-s-OFDM waveform require lower operating SNR than CP-OFDM under UL 256QAM.

Proposal 1: Introduce UL 256QAM with DFT-s-OFDM first and further discuss the feasibility of CP-OFDM.

  - System level simulation

The system level simulation only provides the distribution of large scale SINR for PC3 in 29 GHz, and the results shown in follows:

Figure 5 UL SINR for PC3 in 29 GHz

Observation 4: For PC3 UE, about 20% UE can archive 26 dB SINR at BS side in Indoor scenario, and it will be further reduced to 5% if adjacent channel interference is considered.

For system simulation, it is noted that the power control model in current TR 38.803 was designed for satisfying the target SNR at BS side is 15 dB and we further modify the CLx-tile so that the target SNR changed to 26 dB which is the operating SNR that UE may benefit from UL 256QAM based on the link level simulation results. However, such power control model will lift output power of most UE and lead the result may not so “typical”. In our understanding, PC3 UE that can achieve larger than 30 dB SNR is already very rare in real scenario. Considering PC3 UE face more challenge in implementation, e.g., power limitation, heating, RF component linearity, etc., we afraid the PC3 UE cannot benefit from UL 256QAM but only increase the UE complexity.

Proposal 2: Exclude PC3 from R18 UL 256QAM discussion scope and update the WID.

- Conclusion

In this contribution, we provide our preliminary evaluation results of FR2 UL 256QAM and the observations and proposals are as follows:

Observation 1a: For AWGN in 29 GHz, UL 256QAM can archive performance gain when SNR >22 dB for EVM = 1% and when SNR >26 dB for EVM = 3%.

Observation 1b: For TDL-A and TDL-D in 29 GHz, UL 256QAM can archive performance gain when SNR >30 dB for EVM = 1% and when SNR >35 dB for EVM = 3%.

Observation 2: UL 256QAM is hard to provide performance gain in both 39GHz and 48 GHz.

Observation 3: DFT-s-OFDM waveform require lower operating SNR than CP-OFDM under UL 256QAM.

Observation 4: For PC3 UE, about 20% UE can archive 26 dB SINR at BS side in Indoor scenario, and it will be further reduced to 5% if adjacent channel interference is considered.

Proposal 1: Introduce UL 256QAM with DFT-s-OFDM first and further discuss the feasibility of CP-OFDM.

Proposal 2: Exclude PC3 from R18 UL 256QAM discussion scope and update the WID.

- References

- TR 38.883

- TR 38.803

- Annex

Table A-1 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257) /39 GHz (n260)/48 GHz(n262) |
| CBW | 50 MHz, 100MHz |
| SCS | 60kHz, 120 kHz; |
| Allocated RBs | Full allocation |
| TDD UL-DL pattern | DDDSU |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency TDL-D 30ns delay spread, 35Hz Doppler frequency Static (AWGN) |
| MCS | 64QAM(CP-OFDM): MCS 23, 24, 26, 28 in TS 38.214 Table 5.1.3.1-1, and other MCSs are not precluded 64QAM(DFT-s-OFDM): MCS 22, 23, 25, 27 in TS 38.214 Table 6.1.3.1-1, and other MCSs are not precluded 256QAM: MCS 21, 23, 25, 27 in TS 38.214 Table 5.1.3.1-2, and other MCSs are not precluded Baseline: fixed MCSs |
| Precoding | Codebook based |
| Symbol type | CP-OFDM / DFT-s-OFDM |
| HARQ | 4, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 (for U slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | CP-OFDM KPTRS: 2 (every 2 RBs), LPTRS: 1 (every 1 symbol) DFT-s-OFDM NgroupPT-RS:4, Nsampgroup: 4, LPTRS: 1 |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): example1 (BS) + example1(UE) Option b): example2 (BS) + example2(UE) Option c): example2 (BS) + example2(BS) Option d): example2 (BS) + PN model config1: example1(UE) Option e): Other phase noise models, e.g. ones extracted from commercially available components or published results, are not excluded |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM=rxEVM = [1%,3%,5%] |

Table A-2 system level simulation assumptions

| Parameters |  | Urban macro | Indoor |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | 50m x 120m, 12BSs |
| Inter-site distance |  | 200m (baseline) 300m (optional) | 20m |
| BS antenna height |  | 25 m | 3 m |
| UE location | Outdoor/indoor | Outdoor and indoor | Indoor |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 | 1 m |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m | 0 m |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Pathloss |  | UMa LOS and NLOS in table 5.2.2.1-1 of 38.803 | InH - Office LOS and NLOS in table 5.2.2.1-1 of 38.803 |
| Carrier frequency |  | 29GHz |  |
| BS antenna configuration |  | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 8 dBi | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |
| UE antenna configuration |  | PC3 UE: (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |  |
| System bandwidth |  | 200MHz |  |
| BS TX power in TRP |  | 43dBm | 23 dBm |
| UE max power |  | 23 dBm |  |