3GPP TSG-RAN WG4 Meeting#104-e                	R4-2212832

Electronic Meeting, 15 August – 26 August 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PUCCH demodulation performance of Rel-17 NR coverage enhancements: simulation results

Agenda item: 	9.16.2.2

Document for: 	Discussion

Introduction

In this contribution, we provide the results of our simulation campaign considering the agreements in RAN4#103-e captured in [1][2][3] for PUCCH DM-RS bundling.

PUCCH enhancements

Simulation parameters

We provide in this section the simulation results of PUCCH format 1 and format 3 using the parameters below:

Table 1: Simulation parameters for PUCCH JCE.

| PUCCH Parameters | Format 1 and format 3 |
| --- | --- |
| Metric | Format 1: NACK to ACK detection probability and ACK missed detection probability |
|  | Format 3: UCI block error probability |
| aTDW | (same aTDW for PUSCH JCE) |
|  | TDD: 2 slots // FDD: 8 slots |
| Number of repetitions | (same aTDW for PUSCH JCE) |
|  | TDD: 2 slots // FDD: 8 slots |
| Frequency hopping | Inter-slot frequency hopping disabled |
|  | Intra-slot frequency hopping disabled |
| TDD UL-DL pattern | • (same aTDW for PUSCH JCE) FR1 SCS 15/30 kHz: 7D1S2U, S=6D:4G:4U (TDD/FDD) FR2 60/120kHz: DDSUU, S=10G:2G:2U (TDD) |
| Phase and power offset modelling | with ideal phase offset |
| Bit payload | Format 1: 2 bits payload |
|  | Format 3: 16 bits payload |
| Resource allocation | 14 symbols 1 PRB for PUCCH format 1 and format 3 |
| Antenna configuration | 2 Rx only |
| DMRS configuration for PUCCH F3 | DMRS1+1 and DMRS1+0 |

Simulation results

PUCCH format 1

Table 2 provides the test NACK to ACK detection probability and ACK missed detection probability for PUCCH JCE format .

Table 2:Test NACK to ACK detection probability and ACK missed detection probability for PUCCH JCE format 1

| JCE over |  |  |  | SNR (dB) |
| --- | --- | --- | --- | --- |
| 8 slots | FDD 15kHz SCS 5MHz | TDLA30-10 Low | ACK missed detection 1% | -14 |
|  |  |  | NACK to ACK detection 0.1% | -11,7 |
|  | [FDD 15kHz SCS 50MHz] |  | ACK missed detection 1% | -14,1 |
|  |  |  | NACK to ACK detection 0.1% | -11,9 |
| 2 slots | TDD 15kHz SCS 5MHz |  | ACK missed detection 1% | -7,8 |
|  |  |  | NACK to ACK detection 0.1% | -4,9 |
|  | [TDD 15kHz SCS 50MHz] |  | ACK missed detection 1% | -8 |
|  |  |  | NACK to ACK detection 0.1% | -5,7 |
| 8 slots | FDD 30kHz SCS 10MHz |  | ACK missed detection 1% | -13,7 |
|  |  |  | NACK to ACK detection 0.1% | -10,6 |
|  | [FDD 30kHz SCS 100MHz] |  | ACK missed detection 1% | -13,9 |
|  |  |  | NACK to ACK detection 0.1% | -10,7 |
| 2 slots | TDD 30kHz SCS 10MHz |  | ACK missed detection 1% | -7,6 |
|  |  |  | NACK to ACK detection 0.1% | -4,8 |
|  | [TDD 30kHz SCS 100MHz] |  | ACK missed detection 1% | -7,6 |
|  |  |  | NACK to ACK detection 0.1% | -4,8 |
| 2 slots | TDD 60kHz SCS 50MHz | TDLA30-75 Low | ACK missed detection 1% | -8,3 |
|  |  |  | NACK to ACK detection 0.1% | -5,6 |
|  | TDD 120kHz SCS 50MHz |  | ACK missed detection 1% | -8,1 |
|  |  |  | NACK to ACK detection 0.1% | -5,5 |

PUCCH format 3

Table 3 gives the test UCI block error probability fo PUCCH JCE format 3.

Table 3: Test UCI block error probability fo PUCCH JCE format 3

| JCE over |  |  |  |  | JCE |
| --- | --- | --- | --- | --- | --- |
| 8 slots | FDD 15kHz SCS 5MHz | TDLA30-10 Low | DMRS 1+0 | BLER 1% | -6,8 |
|  |  |  | DMRS 1+1 | BLER 1% | -7,6 |
|  | [FDD 15kHz SCS 50MHz] |  | DMRS 1+0 | BLER 1% | -7,4 |
|  |  |  | DMRS 1+1 | BLER 1% | -8 |
| 2 slots | TDD 15kHz SCS 5MHz |  | DMRS 1+0 | BLER 1% | -1,8 |
|  |  |  | DMRS 1+1 | BLER 1% | -2,2 |
|  | [TDD 15kHz SCS 50MHz] |  | DMRS 1+0 | BLER 1% | -2 |
|  |  |  | DMRS 1+1 | BLER 1% | -2,5 |
| 8 slots | FDD 30kHz SCS 10MHz |  | DMRS 1+0 | BLER 1% | -6,6 |
|  |  |  | DMRS 1+1 | BLER 1% | -7,4 |
|  | [FDD 30kHz SCS 100MHz] |  | DMRS 1+0 | BLER 1% | -6,8 |
|  |  |  | DMRS 1+1 | BLER 1% | -7,6 |
| 2 slots | TDD 30kHz SCS 10MHz |  | DMRS 1+0 | BLER 1% | -1,6 |
|  |  |  | DMRS 1+1 | BLER 1% | -2 |
|  | [TDD 30kHz SCS 100MHz] |  | DMRS 1+0 | BLER 1% | -1,8 |
|  |  |  | DMRS 1+1 | BLER 1% | -2,4 |
| 2 slots | TDD 60kHz SCS 50MHz | TDLA30-75 Low | DMRS 1+0 | BLER 1% | -2,5 |
|  |  |  | DMRS 1+1 | BLER 1% | -3,1 |
|  | TDD 120kHz SCS 50MHz |  | DMRS 1+0 | BLER 1% | -2,2 |
|  |  |  | DMRS 1+1 | BLER 1% | -2,6 |

Conclusion

In this contribution we have provided the results of our simulation campaign on both PUCCH format 1 and format 3 for NR_Cov_enh Rel-17 BS demodulation performance requirements.

References

- R4-2203031 WF on PUCCH demodulation performance of Rel-17 NR coverage enhancement

- R4-2207211 WF on PUCCH demodulation performance of Rel-17 NR coverage enhancement

- R4-2210667 WF on PUCCH demodulation performance of Rel-17 NR coverage enhancement