3GPP TSG-RAN WG4 Meeting #104-e	R4-2212859

Online, 15th – 26th Aug, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2487 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for test case on UE UL carrier RRC reconfiguration delay for FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-Perf |  |  |  |  |  | Date: |  |  | 2022-8-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Submit formal CR for endorsed draftCR for test case on UE UL carrier RRC reconfiguration Delay and interruption at UL carrier RRC reconfiguration for FR2 in 103-e meeting (R4-2211081) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add test case on UE UL carrier RRC reconfiguration Delay for FR2, including verification for interruption at UL carrier RRC reconfiguration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case is missing for UE UL carrier RRC reconfiguration Delay for FR2 and interruption at UL carrier RRC reconfiguration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.5.x1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Formal CR for endorsed draftCR R4-2211081 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of change 1>

### A.7.5.x1	UE UL carrier RRC reconfiguration Delay

#### A.7.5.x1.1	UE UL carrier RRC reconfiguration Delay

##### A.7.5.x1.1.1	Test Purpose and Environment

The purpose of this test is to verify that when the UE receives a RRC message implying NR UL carrier configuration, the UE shall be ready to start transmission on the newly configured carrier within the time limits specified in clause 8.4.2 and 8.4.3 for configuring and deconfiguring, respectively. The test will also verify the interruption at UL carrier configuration requirements on PCell in clause 8.2.2.2.4.

There are two cells: FR2 PCell (Cell 1) on NR RF channel 1 and FR2 SCell (Cell 2) on NR RF channel 2. NR uplink is broadcast by ServingCellConfigCommonSIB. The test parameters for PCell and SCell are given in Table A.7.5.x1.1.1-1, Table A.7.5.x1.1.1-2 and Table A.7.5.x1.1.1-3 below.  The test consists of three time periods, with duration of T1, T2 and T3 respectively. During time duration T1, NR uplink of Cell 2 is not configured to UE. At the start of T2, a NR uplink of Cell 2 is configured to UE through RRCReconfiguration, then UE shall start transmission on the NR uplink of Cell 2. At the start of T3, the NR uplink is released through RRCReconfiguration.

The test equipment also verifies that potential interruption of PCell due to UL carrier configuration on SCell is carried out within the correct time span and within the correct length by monitoring ACK/NACK sent in PCell.

Table A.7.5.x1.1.1-1: Supported test configurations

| Configuration | PCell (Cell 1) | SCell (Cell 2) |
| --- | --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | DL and UL: 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.x1.1.1-2: General test parameters for SA UE UL carrier RRC reconfiguration Delay for FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2 | Two NR RF channels |
| Active cell |  | Config 1 | Cell 1: FR2 PCell Cell 2: FR2 SCell | PCell on RF channel number 1 FR2 SCell on RF channel number 2 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| Cell 2 timing offset to Cell 1 | s | Config 1 | 3 | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 5 |  |
| T3 | s | Config 1 | 5 |  |

Table A.7.5.x1.1.1-3: NR Cell specific test parameters for SA UE UL carrier RRC reconfiguration Delay for FR2

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.3.15 |  |  |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  |  | Rough |  |  |
| Frequency Range |  |  | Config 1 | FR2 |  |  | FR2 |  |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | TDD |  |  | TDD |  |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  |
| Data RBs allocated |  |  | Config 1 | 66 |  |  | 66 |  |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | DLBWP.1.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | ULBWP.1.1 |  |  |
| OCNG Patterns |  |  | Config 1 | OP.1 |  |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| SSB configuration |  |  | Config 1 | SSB.1 FR2 |  |  | SSB.1 FR2 |  |  |
| SMTC configuration |  |  | Config 1 | SMTC.1 |  |  | SMTC.1 |  |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  |  | TRS.2.1 TDD |  |  |
| TCI state |  |  | Config 1 | TCI.State.0 |  |  | TCI.State.0 |  |  |
| PUSCH parameters for NR UL carrier |  |  | Config 1 | N/A |  |  | N/A | G-FR2-A3-4 in [13] | N/A |
| PUCCH parameters for NR UL carriers |  |  | Config 1 | N/A |  |  | N/A | Table 11.3.2.3.1.2-2 in [13] | N/A |
| EPRE ratio of PSS to SSS |  |  | Config 1 | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 | Config 1 | -104.7 |  |  | -104.7 |  |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -95.7 |  |  | -95.7 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -88.7 |  |  | -88.7 |  |  |
|  |  | dB | Config 1 | 7 |  |  | 7 |  |  |
|  |  | dB | Config 1 | 7 |  |  | 7 |  |  |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.92 |  |  | -58.92 |  |  |
| Propagation Condition |  |  | Config 1 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |

##### A.7.5.x1.1.2	Test Requirements

The UE shall be ready to start transmission on the NR uplink carrier on SCell within 20ms from the start of T2.

The UE shall stop the transmission on the NR uplink carrier on SCell within 20ms from the start of T3.

During T2 and T3, the start of interruption of PCell due to UL carrier configuration or de-configuration on SCell shall not happen outside the UL carrier configuration delay which is 20ms in this test, and the length of interruption shall not exceed the length specified in clause 8.2.2.2.4.

All of the above test requirements shall be fulfilled in order for the observed UE UL carrier configuration delay and UE UL carrier release delay to be counted as correct. The rate of correct observed UE UL carrier configuration delay and UE UL carrier release delay during repeated tests shall be at least 90%.

# < End of change 1>