3GPP TSG-RAN WG4 Meeting #104-e                 	R4-2212861

Online, 15th – 26th Aug, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on FR1-FR1 NR-DC

Agenda item:	11.9.3

Document for:	Discussion

Introduction

As RAN#95e meeting, a new WID on Even Further RRM enhancement for NR and MR-DC was agreed, and in RAN#96 meeting, this WID is updated. The following objectives are listed in this WI:

### 4.1 Objective of SI or Core part WI or Testing part WI The following objectives are considered in this WI: - FR2 SCell activation delay reduction - Identify cases where FR2 SCell activation delay can be reduced (e.g., unknown target cell cases), and specify reduced delay requirements for such cases, including but not limited to [RAN4] - Study and, if feasible, enhance cell detection for unknown SCell and time/frequency tracking - Study and, if feasible, enhance L1-RSRP measurement delay reduction on target SCell - Note: Subject to RAN4’s agreement, the technical solutions can be extended to other general RRM requirements if applicable. - Specify if needed, reference signal enhancement and/or signaling enhancement for the UE to meet the enhanced delay requirements [RAN4, RAN2] - Note: No RAN1 work, i.e. introducing new RS, is expected. - Note: the technical solutions can be extended to FR1, when applicable - Define RRM requirements for FR1-FR1 NR-DC scenarios [RAN4] - RRM requirements include the number of serving carriers, PSCell addition/release delay requirement, PSCell change and conditional PSCell change delay, scheduling availability, and CSSF. Other Rel-15 requirements are not precluded and are subject to WI stage discussion. - For R16 and R17 features, RRM requirements for FR1-FR1 NR-DC including HO with PSCell, SCG activation/deactivation and CPAC. - Note: no other R16/17 features are considered

In this paper, we will provide our view on the RRM requirements for FR1-FR1 NR-DC scenarios.

Discussion

In the objective description for FR1-FR1 NR-DC in the WID, the RRM requirements in Rel-15 requirements and in Rel-16 and Rel-17 features are needed to be supported for FR1-FR1 NR-DC, we will discussion one by one.

2. 1 Rel-15 RRM requirements

As listed in the WID, For Rel-15 requirements, below RRM requirements need to be supported:

- Number of serving carriers

- PScell addition/release delay

- PSCell change/conditional PSCell change delay

- Scheduling availability

- CSSF

- Others not precluded.

Number of serving carriers

For number of serving carriers in RRM requirements, it shall be aligned with RF specification based on RRM support requirements. Currently, we only support FR1+FR2 NR-DC in RRM requirements and the number of serving carrier for NR-DC is specified for FR1+FR2 NR-DC as below:

#### 3.6.2.4 Number of serving carriers for NR-DC Requirements for NR-DC are applicable for the UE configured with the following number of serving NR CCs: - up to 2 NR DL CCs in total in FR1, up to 8 NR DL CCs in total in FR2, with 1 UL in PCell, 1 UL in PSCell, and up to 1 UL in each SCell.

To support FR1-FR1 NR-DC, RRM requirements should be updated according to the RF specification.

- RRM requirements for supported number of serving carriers for NR-DC need to be updated  for FR1-FR1 NR-DC according to RF specification.

PScell addition/release delay

NR-DC PSCell addition/release delay requirements are defined in section 8.9 in RRM requirements. In this section, it is defined only for PCell in FR1 and PSCell in FR2.

### 8.9.2 PSCell Addition Delay Requirement The requirements in this clause shall apply for the UE configured with only PCell in FR1. Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR2 no later than in subframe n + Tconfig_PSCell Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR2 no later than in slot n+Tconfig_PSCellNR slot length: ……

To support FR1-FR1 NR-DC, we need to introduce FR1 PSCell in the PSCell addition/release requirements.

- Introduce FR1 PSCell addition/release delay requirements in NR-DC PSCell addition/release delay RRM requirements to support FR1-FR1 NR-DC.

PSCell change delay

NR-DC PSCell change/conditional PSCell change delay requirements are defined in section 8.11 in RRM requirements. In this section, the requirements are defined for the two cases:

- source and target cells are in the same FR

- source and target cells are in different FRs

It means that the requirements are specified to support FR1-FR2 NR-DC and FR1-FR1 NR-DC.  The detail values except Tprocessing defined for PSCell change delay refer to 8.9.2. In the above, we have analysed the PSCell addition delay requirements and the conclusion is that the PSCell addition delay requirements need to be updated to introduce FR1-FR1 NR-DC PSCell addition. Since section 8.9.2 will be updated to support FR1-FR1 NR-DC, current PSCell change delay requirements still work and no need for change.

- Reuse current PSCell change delay requirements since the requirements already support FR1-FR1 NR-DC.

Conditional PSCell change delay

NR-DC conditional PSCell change delay requirements are defined in 8.11B in RRM requirements. In this section, the requirements are defined for the two cases:

- source and target cells are in the same FR

- source and target cells are in different FRs

It means that the requirements are specified to support FR1-FR2 NR-DC and FR1-FR1 NR-DC.

- Reuse current conditional PSCell change delay requirements since the requirements already support FR1-FR1 NR-DC.

Scheduling availability

Scheduling availability during radio link monitoring and link recovery requirements in section 8.1.7, 8.5.7 and 8.5.8 are defined only for CA and FR1-FR2 NR-DC. To support FR1-FR1 NR-DC, the scheduling availability requirements need to be updated.

- Scheduling availability requirements need to be updated to support FR1-FR1 NR-DC

CSSF

CSSF requirements for NR-DC are defined in section 9.1.5.1.3, 9.1.5.2.4 and 9.1.5.2.7. NR-DC CSSF outside gaps defined in 9.1.5.1.3 are only specified for FR1+FR2 NR-DC as below:

Table 9.1.5.1.3-1: CSSFoutside_gap,i scaling factor for NR-DC mode Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PSCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required; CSSFoutside_gap,i for inter-frequency MO with no measurement gap. FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell) Note 1; 1+NPCC_CSIRS; 2×( NSCC_SSB +Y+2xNSCC_CSIRS); 2x(1+ NPSCC_CSIRS) Note 2; 2x(NSCC_SSB +Y+2x NSCC_CSIRS ); 2x(NSCC_SSB +Y+2x NSCC_CSIRS ). Note 1: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2. Note 2: CSSFoutside_gap,i =1 if no SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on PSCC; CSSFoutside_gap,i =2 if no SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on PSCC. Note 3: Y is the number of configured inter-frequency SSB based frequency layers without MG that are being measured outside of MG; otherwise, it is 0. Note 4: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. Note 5: NPSCC_CSIRS=1 if PSCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPSCC_CSIRS =0. Note 6: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured Note 8: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG..

To support FR1-FR1 NR-DC, CSSF outside gaps need to be updated to add FR1-FR1 NR-DC scenario.

- CSSF outside gaps for NR-DC requirements need to be updated to support FR1-FR1 NR-DC

Others RRM requirements

RRM requirements for NR-DC was introduced in Rel-15, even though RRM requirements are only limited to FR1-FR2 NR-DC since from Rel-15, many RRM requirements for NR-DC scenario are defined to consider FR1-FR1 NR-DC also, for example, MRTD/MTTD requirements, conditional PSCell addition, PSCell change, conditional PSCell change, etc. To fully support FR1-FR1 NR-DC, we still have much RRM requirements need to be updated.

Besides the listed RRM requirements in the WID, some sections in RRM requirements are clarified that only support FR1-FR2 NR-DC, since FR1-FR1 NR-DC will be introduced and supported since Rel-18, this limitation or clarification should be removed from Rel-18 RRM requirements.

In NR-DC interruption requirements, only up to 1 SCell in FR1 and up to 7 SCell(s) in FR2 are supported, which means FR1+FR2 NR-DC. To support FR1-FR1 NR-DC, NR-DC interruption requirements also need to be updated.

- Rel-18 RRM requirements need to remove the clarification or limitation of only supporting FR1-FR2 NR-DC

- NR-DC interruption requirements need to be updated to support FR1-FR1 NR-DC.

2.2 Rel-16 and Rel-17 features

As listed in the WID, For Rel-16 and Rel-17 features, below RRM requirements need to be supported:

- HO with PSCell

- SCG activation/deactivation and CPAC

HO with PSCell

In Rel-17 HO with PSCell, we have defined the requirements for HO with PSCell from NR-DC to NR-DC, and only FR1-FR2 NR-DC is supported.  Since we start to support FR1-FR1 NR-DC, we will have another 3 cases need to consider in NR-DC to NR-DC scenario:

- FR1-FR2 NR-DC to FR1-FR1 NR-DC

- FR1-FR1 NR-DC to FR1-FR1 NR-DC

- FR1-FR1 NR-DC to FR1-FR2 NR-DC

With considering these three cases for the HO with PSCell from NR-DC to NR-DC requirements, current requirements for PCell handover defined for FR1 to FR1 still be appliable, PSCell change need to be updated to consider FR1 to FR2 or FR2 to FR1 or FR1 to FR1 PSCell change. Beside this, since PSCell could be possible in FR1, we need to consider parallel or sequential process in the HO with PSCell NR-DC to NR-DC scenario. Hence, the processing time in PCell HO and PSCell change should also be updated.

- HO with PSCell from NR-DC to NR-DC requirements need to be updated to support FR1-FR1 NR-DC

SCG activation/deactivation and CPAC

SCG activation/deactivation requirements are defined in section 8.17. In the SCG activation delay requirements, Tprocessing, Tsearch and cell known condition are defined only for FR2 PSCell. To support FR1-FR1 NR-DC, the SCG activation delay requirements need to be updated to include FR1 PSCell.

- SCG activation/deactivation requirements need to be updated to support FR1-FR1 NR-DC

Conclusion

In this contribution we have discussed the RRM requirements for FR1-FR1 NR-DC. We have made the following proposals:

- RRM requirements for supported number of serving carriers for NR-DC need be updated  for FR1-FR1 NR-DC according to RF specification.

- Introduce FR1 PSCell addition/release delay requirements in NR-DC PSCell addition/release delay RRM requirements to support FR1-FR1 NR-DC.

- Reuse current PSCell change delay requirements since the requirements already support FR1-FR1 NR-DC.

- Reuse current conditional PSCell change delay requirements since the requirements already support FR1-FR1 NR-DC.

- Scheduling availability requirements need to be updated to support FR1-FR1 NR-DC

- CSSF outside gaps for NR-DC requirements need to be updated to support FR1-FR1 NR-DC

- Rel-18 RRM requirements need to remove the clarification or limitation of only supporting FR1-FR2 NR-DC

- NR-DC interruption requirements need to be updated to support FR1-FR1 NR-DC.

- HO with PSCell from NR-DC to NR-DC requirements need to be updated to support FR1-FR1 NR-DC

- SCG activation/deactivation requirements need to be updated to support FR1-FR1 NR-DC

References

- RP-221696	Revised WID on Rel-18 even Further RRM enhancement for NR and MRDC, Apple, Vivo, RAN#96

- TS38.133, h60