3GPP TSG-RAN WG4 Meeting#104-e                	R4-2212877

E-meeting, August 15th – 26th, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Final aspects on Efficient activation/de-activation mechanism for SCells

Agenda item:	9.20.1.1

Document for:	Discussion

Introduction

In last meeting RAN4#103, the work progressed leaving only a few open issues for further discussion [1]. One issue related Temporary RS based SCell activation delay (Issue 1-1) and three issues related to SCG activation/deactivation delay (Issues 2-1-3, 2-1-4 and 2-1-5).

In this paper we address issue related Temporary RS based SCell activation delay (Issue 1-1).

Discussion

In Rel-17 3GPP have been working on a WI Further Multi-RAT Dual-Connectivity enhancements. Most aspects have been discussed and agreed and only a few remain opens. In the following we address these and give our views.

Temporary RS based SCell activation delay

For the discussion related to temporary RS based SCell activation delay 1 issue remained open after RAN4#103 meeting.

Additional condition for scenario #3

Two options have been listed in the WF [1] from RAN4#103:

Issue 1-1: Additional condition for scenario #3

  - Option 1(Qualcomm, Huawei, Apple, MTK, OPPO, vivo): Add one additional condition

          - UE receives the SCell activation command and TCI state activation command at the same time.

  - Option 2 (Nokia, Ericsson): Allow the UE the additional activation time for receiving the TCI state activation command if not provided in the same MAC command as the SCell activation command.

    - Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to SCell activation command for known case, if the UE does not receive the SCell activation command and TCI state activation command simultaneously. Otherwise, Tuncertainty_MAC = 0.

As stated in last meeting, we do not see a problem in allowing the UE the additional time for receiving the TCI state activation command if this is not provided in the same MAC command as the SCell activation command.

However, we also see no reason not to have any UE requirements if the UE does not receive the SCell activation command and TCI state activation command at the same time.

Allowing such extra time if the TCI state activation command if not provided in the same MAC command as the SCell activation command is also applied in other RAN4 activation delay requirements.

Additional activation time is allowed, if UE is not provided with the TCI state activation command in the same MAC command as the SCell activation command.

This could be captured as follows:

If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is:

-	3ms + max(TFirstATRS + 2ms + Tuncertainty_MAC, Tuncertainty_SP), where Tuncertainty_SP=0 if UE receives the SCell activation command and semi-persistent CSI-RS activation command at the same time.

If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

-	max(TFirstATRS + 5ms + Tuncertainty_MAC, Tuncertainty_RRC + TRRC_delay-THARQ).

where:

- Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to SCell activation command for known case, if the UE does not receive the SCell activation command and TCI state activation command simultaneously. Otherwise, Tuncertainty_MAC = 0.

Alternative is to capture in the same way as done for existing SCell activation delay requirements.

We have captured this proposal in our CR [2].

Conclusion

For the discussion related to temporary RS based SCell activation delay 1 issue remained open after RAN4#103 meeting, Issue 1-1: Additional condition for scenario #3, for which we propose following:

- Additional activation time is allowed, if UE is not provided with the TCI state activation command in the same MAC command as the SCell activation command.

This is aligned with legacy SCell activation delay principles. We have captured this proposal in our CR [2].

References

- R4-2210605, WF on R17 further Multi-RAT Dual-Connectivity enhancements, Huawei, HiSilicon.

- R4-2212878, CR on Efficient activation/de-activation mechanism for SCells, Nokia, Nokia Shanghai Bell