3GPP TSG-RAN WG4 Meeting #104-e	R4-2212892

Electronic Meeting, 15 August – 26 August 2022

Agenda Item:	9.18.5.1.1

Source:	Ericsson

Title:	PDSCH demodulation requirements for RedCap

Document for:	Discussion

# 1	Introduction

RAN4#103-e agreed with the WF on RedCap UE demodulation and CQI reporting requirements [1]. This contribution provides our simulation results and discusses the remaining open issues on PDSCH demodulation requirements for RedCap.

# 2	HD-FDD

Agreement: UL/DL pattern used for FDD tests for 1Rx UE - Introduce demodulation/CSI requirements covering both FD-FDD and HD-FDD. - DDDSU applied for HD-FDD - Existing pattern applied for FD-FDD - Same demodulation requirement applied for FD-FDD and HD-FDD with different FRCs - Test applicability rule: - If RedCap UE support only HD-FDD in a FDD band, this UE is tested with HD-FDD mode otherwise UE is tested with FD-FDD mode - FFS for the UE demodulation and CSI reporting requirements for 2Rx RedCap UE supporting HD-FDD in FDD bands

According to the latest Rel-17 RAN1 feature list [2], the half-duplex FDD feature is independent of the number of receive antennas, that is, 2Rx UE may support HD-FDD in an FDD band instead of full-duplex FDD.

| Features | Index | Feature group | Components | Prerequisite feature groups | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- | --- | --- |
| 28. NR_redcap | 28-3 | Half-duplex FDD operation type A for RedCap UE | 1. Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE | 28-1 | UE is assumed to support FD-FDD in that band |

Although we think the HD-FDD is mainly supported by 1Rx UE, from the test coverage point of view, it is important to ensure the UE performance if 2Rx RedCap UE supports only HD-FDD in an FDD band. Our preference is therefore to define HD-FDD also for 2Rx UE, and apply the same UE demodulation and CSI reporting requirements regardless of HD-FDD or FD-FDD.

For reference, the below is an example of specification of PDSCH demodulation requirements of 1Rx and 2Rx. In this example, R.PDSCH.x-y.z HD-FDD is the FRC with HD-FDD pattern (i.e., DDDSU) where the MCS etc. are same as R.PDSCH.1-1.1 HD-FDD.

## 5.2 PDSCH demodulation requirements ### 5.2.1 1RX requirements #### 5.2.1.1 FDD ##### 5.2.1.1.1 Minimum requirements for RedCap UE Table 5.2.1.1.x-y: Minimum performance for UE supporting full-duplex FDD Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; TDLB100-400; 2x1, ULA Low; 70; TBD. Table 5.2.1.1.x-z: Minimum performance for UE supporting half-duplex FDD Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 2-1; R.PDSCH.x-y.z HD-FDD; 10 / 15; QPSK, 0.30; TDLB100-400; 2x1, ULA Low; 70; Same as 1-1. ### 5.2.2 2RX requirements #### 5.2.2.1 FDD […] ##### 5.2.2.1.X Minimum requirements for RedCap UE Table 5.2.2.1.x-y: Minimum performance for UE supporting full-duplex FDD Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; TDLB100-400; 2x2, ULA Low; 70; -0.8. Table 5.2.2.1.x-z: Minimum performance for UE supporting half-duplex FDD Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 2-1; R.PDSCH.x-y.z HD-FDD; 10 / 15; QPSK, 0.30; TDLB100-400; 2x2, ULA Low; 70; Same as 2-1.

Proposal 1: Specify the UE demodulation and CSI reporting requirements for 2Rx RedCap UE supporting HD-FDD in FDD bands as well as 1Rx UE. The requirements with HD-FDD should be the same as the corresponding (full-duplex) FDD requirements.

# 3	PDSCH demodulation

## 3.1	256QAM PDSCH requirements

The remaining open issues on PDSCH demodulation requirements for RedCap is whether to specify 256QAM demodulation requirements or not for 1Rx UE.

For 256QAM requirements for 1Rx, - Option 1: Define 256QAM demodulation requirements for 1Rx in FR1 - Option 1a: MCS20 - Option 1b: MCS is based on the operating SNR - Option 2: Not to define 256QAM demodulation requirements for 1Rx in FR1

Since RAN4 has already agreed to define PDSCH demodulation requirements with 256QAM for 2Rx RedCap UE, it is straightforward to define the requirements for 1Rx RedCap UE. Moreover supporting 256QAM is an optional feature for RedCap UE; UE does not need to be tested if it does not support this feature.

Table 3 shows the ideal simulation results of 256QAM PDSCH according to the way forward. According to the simulation results, it is observed that the required SNR to achieve 70% of peak rate for MCS25 is more than 25dB. We think it is still feasible even if we consider the RF impairment margin, but we also understand the concern from chipset vendors discussed in the last meeting. If we observe the ideal results with MCS20 with 1Rx, it is close to MCS24 with 2Rx which is used for 2Rx RedCap requirements. We are also fine to define 256QAM PDSCH requirements with MCS20 for 1Rx RedCap UE considering the same operating point as MCS24 for 2Rx UE.

Table 3	Ideal PDSCH simulation results with 256QAM for 1Rx/2Rx RedCap UEs.

|  | MCS 20 (CR=0.67) | MCS22 (CR=0.74) | MCS23 (CR=0.78) | MCS24 (CR=0.82) |
| --- | --- | --- | --- | --- |
| FDD SCS=15kHz, 1Rx | 21.8 | 22.9 | 24.4 | 25.3 |
| FDD SCS=15kHz, 2Rx (reference) |  |  |  | 20.8 |
| TDD SCS=30kHz, 1Rx | 21.8 | 23.6 | 24.7 | 25.8 |
| TDD SCS=30kHz, 2Rx |  |  |  | 21.1 |

Proposal 2: RAN4 defines 256QAM demodulation requirements for 1Rx RedCap UE.

Proposal 3: For 1Rx RedCap UE, set MCS20 for 256QAM demodulation requirements.

## 3.2	Simulation results

Table 1 and Table 2 summarizes our simulation results for FDD 15kHz and TDD 30kHz, respectively, according to the simulation assumption in [1].

Table 1	Summary of PDSCH simulation results for FDD 15kHz.

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | 1.0 | 3.0 |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | 9.1 | 11.1 |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | 13.6 | 15.6 |
| 2Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | Reuse | Reuse |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | 5.1 | 7.1 |
| 2Rx UE | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | Reuse | Reuse |
| 2Rx UE | 10MHz / 15kHz | 256QAM 0.82 Rank 1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-3 | Reuse | Reuse |

Table 2	Summary of PDSCH simulation results for TDD 30kHz.

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | 0.7 | 2.7 |
| 1Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | 9.2 | 11.2 |
| 1Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | 13.8 | 15.8 |
| 2Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | -2.8 | -0.8 |
| 2Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | 4.9 | 6.9 |
| 2Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | 16.5 | 18.5 |
| 2Rx UE | 20MHz / 30kHz | 256QAM 0.82 Rank 1 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-3 | 21.1 | 23.1 |

# 4	Summary

Proposal 1: Specify the UE demodulation and CSI reporting requirements for 2Rx RedCap UE supporting HD-FDD in FDD bands as well as 1Rx UE. The requirements with HD-FDD should be the same as the corresponding (full-duplex) FDD requirements.

Proposal 2: RAN4 defines 256QAM demodulation requirements for 1Rx RedCap UE.

Proposal 3: For 1Rx RedCap UE, set MCS20 for 256QAM demodulation requirements.

# References

- R4-2210672 “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.

- R1-2205609 “LS on updated Rel-17 RAN1 UE features list for NR”, RAN1.