3GPP TSG-RAN WG4 Meeting #104-e	R4-2212893

Electronic Meeting, 15 August – 26 August 2022

Agenda Item:	9.18.5.1.2

Source:	Ericsson

Title:	PDCCH/PBCH demodulation requirements for RedCap

Document for:	Discussion

# 1	Introduction

RAN4#103-e agreed with WF on RedCap UE demodulation and CQI reporting requirements [1]. This contribution provides our simulation results to set the requirements according to the way forward.

# 2	PDCCH demodulation

1Rx UE: - FR1 FDD 15kHz: Define AL8 only - FR1 TDD 30kHz: Define AL4 only 2Rx UE: - FR1 FDD 15kHz: Define both AL4 and AL8 - FR1 TDD 30kHz: Define both AL4 and AL8 - FR2 TDD 120kHz: Define both AL4 and AL8

Table 2 and Table 3 summarize PDCCH simulation results for FDD 15kHz and TDD 30kHz, respectively, according to [1].

Table 2	Summary of PDCCH simulation results for FDD 15kHz.

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x1 low | 1% | 5.3.2.1.2-1 Test 3 (R.PDCCH.1-1.3 FDD) | 3.2 | 5.2 |
| 2Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 low | 1% | 5.3.2.1.1-1 Test 3 (R.PDCCH.1-2.4 FDD) | Reuse | Reuse |
| 2Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 low | 1% | 5.3.2.1.2-1 Test 3 (R.PDCCH.1-1.3 FDD) | Reuse | Reuse |

Table 3	Summary of PDCCH simulation results for TDD 30kHz.

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | 1 | 4 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 2 (R.PDCCH.2-1.2 TDD) | 5.6 | 7.6 |
| 2Rx UE | 20 | 48 | 1 | 4 | TDLC300-100 | 1x2 low | 1% | 5.3.2.2.1-1 Test 2 (R.PDCCH.2-1.2 TDD) | 1.2 | 3.2 |
| 2Rx UE | 20 | 48 | 1 | 8 | TDLC300-100 | 2x2 low | 1% | 5.3.2.2.2-1 Test 1 (R.PDCCH.2-1.3 TDD) | -1.8 | 0.2 |

# 3	PBCH demodulation

Table 4 summarizes our PBCH simulation results for the case SS/BPCH block index is unknown.

Table 4	Summary of PBCH simulation results.

| 1Rx UE or 2Rx UE | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | SS/PBCH block index | Reference from TS38.101-4 | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Unknown | 5.4.2.1-2 Test 1 | -4.2 | -2.2 |
| 1Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Unknown | 5.4.2.2-2 Test 1 | -3.2 | -1.2 |

# References

- R4-2210672 “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.