3GPP TSG-RAN WG4 Meeting #104-e	R4-2212896

Electronic Meeting, 15 August – 26 August 2022

Agenda Item:	9.24.6.1.3

Source:	Ericsson

Title:	Test setup of NB-IoT channel quality reporting requirements

Document for:	Discussion

# 1	Introduction

RAN4 has agreed with the way forward on UE demodulation requirements for Rel-17 NB-IoT [1][2]. In [2], RAN4 has agreed to define CQI reporting requirements for TDD in addition to half-duplex FDD.

CQI reporting test for TDD RAN4 defines the CQI reporting test for TDD

This contribution discusses the detailed test setup for NB-IoT CQI reporting tests based on the existing NPDSCH demodulation requirements in TDD and agreed CQI test setup for HD-FDD.

# 2	CQI reporting requirements for TDD

TDD UL-DL configuration and special subframe configuration

RAN4 has defined NPDSCH demodulation requirements for TDD from Rel-15. Rel-15 NPDSCH demodulation requirements use Uplink downlink configuration 4 (DSUUD DDDDD) and special subframe configuration 4, to allocate more downlink subframes and allocate 2 consecutive uplink subframes for NPUSCH format 2 (HARQ-ACK). Since NB-IoT CQI request and reporting is the MAC-based procedure and it uses NPDSCH/NPUSCH format 1, we propose to use the same TDD configuration as Rel-15 NPDSCH demodulation requirements.

Proposal 1: For NB-IoT CQI reporting test in TDD, set the UL-DL configuration and special subframe configuration parameters same as the NPDSCH demodulation requirements:

- Uplink-downlink configuration: 4 (DSUUD DDDDD)

- Special subframe configuration: 4

CQI request and reporting

For NB-IoT CQI reporting test in HD-FDD, it was agreed to configure the following parameters for NPDCCH/PDSCH scheduling [2]:

- Rmax (npdcch-NumRepetitions): 4

- G (nPDCCH-startSF-USS): 2

- Repetition numbers of NPDCCH and NPUSCH: 1

- ISF=1 for NPDSCH and IRU=NPUSCH format 1 to transmit MAC CE

With this configuration, the UE-specific search space is configured every 8ms and then the CQI reporting period becomes 40ms. Figure 1 illustrates the CQI request and report pattern.

Figure 1	Scheduling of CQI request and report in HD-FDD.

In the case of TDD, if we reuse the same NPDCCH/PDSCH parameters and consider TDD UL-DL configuration 4, the CQI request and report pattern is illustrated as Figure 2, and the CQI reporting period becomes 80ms. This configuration also ensures UE monitors at least 4 subframes for CQI measurements. We therefore propose to set the same NPDCCH/NPDSCH parameters for TDD case also.

Figure 2	Scheduling of CQI request and report in TDD with UL-DL configuration 4.

Proposal 2: For NB-IoT CQI reporting test in TDD, configure the following NPDCCH/NPDSCH parameters:

- Rmax (npdcch-NumRepetitions): 4

- G (nPDCCH-startSF-USS): 2

- Repetition numbers of NPDCCH and NPUSCH: 1

- ISF=1 for NPDSCH and IRU=NPUSCH format 1 to transmit MAC CE

- CQI reporting period: 80ms

# 3	Summary

Proposal 1: For NB-IoT CQI reporting test in TDD, set the UL-DL configuration and special subframe configuration parameters same as the NPDSCH demodulation requirements:

- Uplink-downlink configuration: 4 (DSUUD DDDDD)

- Special subframe configuration: 4

Proposal 2: For NB-IoT CQI reporting test in TDD, configure the following NPDCCH/NPDSCH parameters:

- Rmax (npdcch-NumRepetitions): 4

- G (nPDCCH-startSF-USS): 2

- Repetition numbers of NPDCCH and NPUSCH: 1

- ISF=1 for NPDSCH and IRU=NPUSCH format 1 to transmit MAC CE

- CQI reporting period: 80ms

# References

- R4-2210674, “Way forward for performance requirements of Rel-17 NB-IOT and eMTC”, Huawei, HiSilicon.

- R4-2207200, “Way forward for performance requirements of Rel-17 NB-IOT and eMTC”, Huawei, HiSilicon.