3GPP TSG-RAN WG4 Meeting#104-e                	R4-2212898

Electronic Meeting, 15 – 26 August 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On RRM Measurement accuracy for Rel-17 NR NTN

Agenda item:	9.11.6.10

Document for:	Discussion

Introduction

This contribution discusses the RRM measurement accuracy requirements. The WF [1] lists several open issues, which we discuss several in the following sections.

Impact of measurement accuracy on performance

RSRP and RSRQ measurements are used for mobility decisions and their accuracy is critical for the handover performance. Therefore, 3GPP has defined the following requirements for the absolute and relative SS-RSRP accuracy (for terrestrial networks) [2]:

Table 10.1.2.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| ±4.5 | ±9 | ³-6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| ±8 | ±11 | ³-6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

Table 10.1.2.1.2-1: SS-RSRP Intra frequency relative accuracy in FR1 [2]

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±2 | ±3 | ³-3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| ±3 | ±3 | ³-6 | Note 3 | Note 3 | Note 3 | N/A | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

As can be seen from the Tables the relative accuracy is tighter than the absolute accuracy and as we are focussing on handover performance in this document, this is what we focus on.

For NTN the difference in RSRP between neighbouring cells is typically rather small due to the propagation channel. An example of this can be seen in Figure 1, which shows the RSRP of different cells, considering earth moving cells, measured by a stationary UE when the cells move above it (the different colours represent the different cells). In this scenario the cells have a 50 km cell diameter and measurements errors corresponding to 2 dB as in the above table are included. The figure shows that from maximum RSRP (when being in the cell centre) to minimum RSRP (when at the cell border) there is about 2 dB, which is significantly lower as the RSRP differences one can expect in terrestrial networks.

Observation 1: The differences between RSRP levels of neighbouring cells in NTN are small.

Figure 1 RSRP of neighboring cells in NTN with Earth moving cells for with 50 km cell diameter.

While the smaller differences in RSRP may lead to usage of smaller hysteresis for handover decisions it also means that errors on top of the measurements have a large impact and may lead to suboptimal handover decisions. This is one of the reasons why CHO with enhancements is a good alternative for NTN. However, as networks may use conventional handover in NTN, conventional handover algorithms should be used as baseline when evaluating the impact of measurement errors on the performance

Proposal 1: Conventional handover should be used as baseline when evaluating the handover performance in NTN.

When evaluating the handover performance, we assume that the measurement errors are uncorrelated between the different cells. The error is modelled as gaussian distribution with zero mean and a 90% accuracy as used in the Tables of [2]. For evaluation of the impact of the measurement accuracy on the mobility performance we use the following scenario (see set-1 satellite parameters from Table 6.1.1.1-1 in [4] for further details):

- LEO satellite at 600 km with beam pattern from section 6.4.1 in [3]

- Cell size: 50 km diameter

- S-band (i.e. 2GHz)

- Qin/Qout thresholds: -6/-8 dB; T310 timer: 1000 ms; N310/N311: 1

- L3 filter coefficient K: 4

- Time to trigger:0 ms

- HO Margin: 0 dB

- UE speed: 0 m/s

Figure 2 show the handover operations per UE per minute, where the following operations can be seen:

- HO attempt, i.e. NR A3 event, per minute per UE

- Unnecessary handover per UE: a handover is defined as unnecessary when the UE stays connected to the new cell less than a second.

- Radio Link Failures, based on the parameters shown above.

From the figure we can conclude the following

- For this scenario the minimum number of handover attempts per UE is approximately 13.6 per minute, as this is the number without measurement error.

- The measurement error has a large impact on the number of handover attempts and number of unnecessary handover. The current accuracy requirements of 2 dB double the number of handover attempts compared to the minimum. This is caused by the fact that the measurement error leads to non optimal handover decisions, and unnecessary handovers as also can be seen.

- Also the number of RLF increases with the measurement error which is likely caused by the increased number of handovers.

Figure 2 HO operations per UE per minute vs measurement accuracy

Observation 2: Increased measurement error leads to a larger number of handovers and higher RLF rates.

Proposal 2: RAN4 should decide to keep the measurement accuracy at least as accurate as in terrestrial networks and should consider tightening the requirements further.

Conclusion

We propose

Proposal 1: Conventional handover should be used as baseline when evaluating the handover performance in NTN.

Proposal 2: RAN4 should decide to keep the measurement accuracy at least as accurate as in terrestrial networks and should consider tightening the requirements further.

References

[1]    R4-2210612, WF on timing requirements and performance requirements for NR NTN, Xiaomi

[2]    3GPP, “Requirements for support of radio resource management”, TR 38.133, V17.6.0, June 2022

[3]    3GPP, “Study on New Radio (NR) to support non-terrestrial networks (Release 15),” TR 38.811 V15.2.0, Sept. 2019.

[4]    3GPP, “Solutions for New Radio (NR) to support non-terrestrial networks (Release 16),” TR 38.821 V16.0.0, Dec. 2019.