3GPP TSG-RAN WG4 Meeting # 104-e                	R4-2212912

Electronic Meeting, August 15th – 26th, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on general RRM requirements for extension to 71 GHz

Agenda item: 	9.14.6.1

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on RRM aspects related to the operation between 52.6 GHz and 71 GHz.

Discussion

Measurement procedures

During the RAN4 #103 meeting, the following was agreed regarding RX beam sweeping scaling factor [1]:

Rx beam sweeping scaling factor Agreement: Define an Rx beam sweeping scaling factor of 12 for FR2-2 Agreement: If Rx Beam Sweeping Scaling factor is extended, maxNumberRxBeam should be extended by the same number

Apart from the change in the Rx beam sweeping factor, the time period  for cell-detection and SSB index acquisition were also changed as follows:

Measurement procedures Agreement: RAN4 to relax the cell-detection (PSS/SSS detection) delay requirements by a factor of 2 for 480kHz SCS and by a factor of 3 for 960kHz SCS to avoid increasing the HW and memory requirements Agreement: The SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended to - 4*NRxBeam samples for power class 2 and 3 - 6*NRxBeam samples for power class 1 - NRxBeam is the Rx beam sweeping factor under discussion Agreement: When deriveSSB-IndexFromCell is not enabled, the requirements of SSB index detection for FR2-2 for intra-frequency measurement can be defined as below: Time period for time index detection for intra-frequency measurements for FR2-2 DRX cycle; Without measurement gaps; With measurement gaps. No DRX; Max(200ms, ceil(MSSB_index_intra ´ Kp) ´ SMTC period) ´ CSSFintra; Max(200ms, ceil(MSSB_index_intra ´ Kp) ´ Max(MGRP, SMTC period)) ´ CSSFintra. DRX cycle≤ 320ms; Max(200ms, ceil(1.5 ´ MSSB_index_intra s ´ Kp) ´ Max(SMTC period, DRX cycle)) ´ CSSFintra; Max(200ms, ceil(1.5 ´ MSSB_index_intra ´ Kp) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFintra. DRX cycle>320ms; ceil(MSSB_index_intra ´ Kp) ´ DRX cycle ´ CSSFintra; ceil(MSSB_index_intra ´ Kp) ´ DRX cycle ´ CSSFintra. - MSSB_index_intra to apply agreements from inter-frequency SSB index identification time

Cell detection

Considering the agreements in RAN4 #103, our view is that they could be implemented in the same tables current in the specification for FR2-1, but with the introduction of a scaling factor KFR, to relax the cell detection delay requirements for SCS equal to 480 kHz or 960 kHz.

Introduce the factor KFR to account for the cell-detection relaxation agreed in RAN4 #103, where: For FR2-1, KFR = 1. For FR2-2: KFR =  1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz.

One example of the implementation of proposal 1 in the specification is:

Table 9.2.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(KFR x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(KFR x 1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(KFR x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: KFR is a scaling factor depending on the frequency range and SSB SCS. For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

Conclusion

In this paper, we present Nokia’s view on how to implement the cell detection relaxation agreed in the last RAN4 meeting.

Introduce the factor KFR to account for the cell-detection relaxation agreed in RAN4 #103, where: For FR2-1, KFR = 1. For FR2-2: KFR =  1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz.

One example of the implementation of proposal 1 in the specification is:

Table 9.2.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(KFR x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(KFR x 1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(KFR x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: KFR is a scaling factor depending on the frequency range and SSB SCS. For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

References

- R4-2210590, WF on NR extension to 71 GHz RRM requirements (Part 1), Qualcomm