3GPP TSG-RAN WG4 Meeting #104-e     	R4-2212914

E-meeting, August 15th – 26th, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RRM test configurations for RedCap

Agenda item:	9.18.4.2.1 Applicability rule, configurations and side conditions

Document for:	Discussion

- Introduction

In RAN4 #103, the RRM performance for RedCap UEs was discussed, and the following was agreed [1]:

### Sub-topic 6-2 Test configurations and type of tests Applicability requirements for RedCap Applicability requirements for RedCap test cases are introduced in A.3x. Duplex modes All RRM test cases are introduced for FDD, TDD and HD-FDD UEs. Accuracy of SS-RSRQ RedCap SS-RSRQ accuracy level is derived by relaxing the legacy SS-RSRQ accuracy level by the same level as agreed for RedCap SS-RSRP measurement compared to legacy SS-RSRP measurement. Accuracy of SS-SINR RedCap SS-SINR accuracy level is derived by relaxing the legacy SS-SINR accuracy level by the same level as agreed for RedCap SS-RSRP measurement compared to legacy SS-RSRP measurement.

In this document, Nokia’s views regarding RRM test configurations are presented.

# 2 General test cases configurations

Our view is that, in general, the RRM tests configurations from NR test cases can be reused for RedCap UEs, with the following observations: bandwidth configuration in FR1 and duplex modes.

Test case bandwidth configuration in FR1

In general, the RRM test cases in FR1 are defined for two bandwidths: 10 MHz and 40 MHz. For RedCap, in FR1 the maximum bandwidth is 20 MHz, therefore there is need to define test configurations with 20 MHz.

NR test case configurations are used as baseline for RedCap UEs. In FR1, NR test cases defined with bandwidth equal to 40 MHz are replaced by test cases with bandwidth equal to 20 MHz for RedCap test cases.

Our proposal is that the test configurations with bandwidth equal to 20 MHz consider an SSB SCS equal to 30 kHz. The proposed configuration uses SSB pattern 2 as baseline.

Define a new SSB pattern for SSB SCS = 30 kHz in 20 MHz channel as:

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 20 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 |
| Slot numbers containing SSB Note 3 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. Note 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves |  |

FDD and TDD duplex mode

In general, the FDD and TDD RedCap test cases can be defined with the same configurations as the NR test cases, except for the configuration of the bandwidth discussed above. One exception, is the RLM OOS test cases for 1 Rx RedCap UE which require a higher aggregation level to be considered, taking into account the PDCCH parameters defined in Table 8.1B.2.1-1 and Table 8.1B.3.1-1 of TS 38.133.

Existing configurations of reference channels in FDD and TDD duplex modes for NR UEs can be reused for RedCap test cases. In addition to NR configurations, new configurations of RMSI CORESET and Control Channel RMC are needed considering an aggregation level of 16 CCE for RLM OOS and BFD test cases for 1 Rx RedCap.

- An example of a configuration with an aggregation level of 16 CCE is given below:

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.1.1 FDD | CR.1.2 FDD |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case |  |  |  |  |  |
| Subcarrier spacing for RMSI CORESET | kHz | 15 | 15 |  |  |  |  |  |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 | 48 |  |  |  |  |  |
| Subcarrier spacing for SSB | kHz | 15 | 15 |  |  |  |  |  |
| SSB and RMSI CORESET multiplexing configuration Note 7 |  | Pattern 1 | Pattern 1 |  |  |  |  |  |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note8) | 0 (Note8) |  |  |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 | Index 4 |  |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |  |  |
| Duration of RMSI CORESET | symbols | 2 Note 7 | 2 Note 9 |  |  |  |  |  |
| DCI Format Note 1 |  | Note 2 | Note 2 |  |  |  |  |  |
| Aggregation level | CCE | 8 | 16 |  |  |  |  |  |
| DMRS precoder granularity |  | 6 | 6 |  |  |  |  |  |
| REG bundle size |  | 6 | 6 |  |  |  |  |  |
| Mapping from REG to CCE |  | Distributed | Distributed |  |  |  |  |  |
| Cell ID |  | Note 5 | Note 5 |  |  |  |  |  |
| Payload (without CRC) | bits | Note 6 | Note 6 |  |  |  |  |  |
| Note 1: DCI formats are defined in TS 38.212. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [3]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [3] Note 8: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. Note 9: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 8 in Table 13-1 in TS 38.213 [3] |  |  |  |  |  |  |  |  |

HD-FDD duplex mode

In the last RAN4 meeting, it was agreed that all test cases are to be defined for TDD, FDD and HD-FDD duplex modes. Currently in TS 38.133 there is no test configuration for HD-FDD duplex mode, therefore a new configuration needs to be defined.

It is our view, that there is no need to define HD-FDD tests for both 15 kHz and 30 kHz SCS, therefore, it is proposed that HD-FDD configurations are defined for SCS = 15 kHz. The following reference measurement channels / signals will need an HD-FDD configuration:

- PDSCH

- CORESET for RMSI scheduling

- CORESET for RMC scheduling

- CSI-RS for tracking

New HD-FDD configurations are needed for: PDSCH, CORESET for RMSI scheduling, CORESET for RMC scheduling and CSI-RS for tracking.

- Conclusions

In this contribution, Nokia’s views regarding general RRM test configurations for RedCap UEs are presented. The following proposals are discussed:

- NR test case configurations are used as baseline for RedCap UEs. In FR1, NR test cases defined with bandwidth equal to 40 MHz are replaced by test cases with bandwidth equal to 20 MHz for RedCap test cases.

- Define a new SSB pattern for SSB SCS = 30 kHz in 20 MHz channel as:

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 20 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 |
| Slot numbers containing SSB Note 3 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. Note 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves |  |

Existing configurations of reference channels in FDD and TDD duplex modes for NR UEs can be reused for RedCap test cases. In addition to NR configurations, new configurations of RMSI CORESET and Control Channel RMC are needed considering an aggregation level of 16 CCE for RLM OOS and BFD test cases for 1 Rx RedCap.

New HD-FDD configurations are needed for: PDSCH, CORESET for RMSI scheduling, CORESET for RMC scheduling and CSI-RS for tracking.

References

- R4-2210592, “WF on RedCap RRM requirements”, Ericsson, RAN3 #103-e