3GPP TSG-RAN WG4 Meeting #104-e     	R4-2212915

E-meeting, August 15th – 26th, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Test cases for RRC connection mobility control for RedCap

Agenda item:	9.18.4.2.4 Test cases for RRC_CONNECTED state mobility

Document for:	Discussion

- Introduction

According to the work split agreed in RAN4 #103-e [1], the following test cases are to be defined for RRC_connected state mobility in FR1:

| #### A.6.3.2 RRC Connection Mobility Control #### A.6.3.2.1 SA: RRC Re-establishment ##### A.6.3.2.1.x1Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE A.6.3.2.1.x2 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE | Nokia |
| --- | --- |
| ##### A.6.3.2.1.x1 Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE A.6.3.2.1.x2 Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE | Nokia |
| ##### A.6.3.2.1.x1 Intra-frequency RRC Re-establishment in FR1 without serving cell timing for 1 Rx UE A.6.3.2.1.x2 Intra-frequency RRC Re-establishment in FR1 without serving cell timing for 2 Rx UE | Nokia |
| #### A.6.3.2.2 Random Access ##### A.6.3.2.2.x1 4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE A.6.3.2.2.x2 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | Xiaomi |
| ##### A.6.3.2.2.x1 4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE A.6.3.2.2.x2 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE | Xiaomi |
| ##### A.6.3.2.2.x1 2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE A.6.3.2.2.x2 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | vivo |
| ##### A.6.3.2.2.x1 2-step RA type non-contention based test in FR1 for NR standalone for 1 Rx UE A.6.3.2.2.x2 2-step RA type non-contention based test in FR1 for NR standalone for 2 Rx UE | vivo |
| #### A.6.3.2.3 SA: RRC Connection Release with Redirection ##### A.6.3.2.3.x1 Redirection from NR in FR1 to NR in FR1 for 1 Rx UE A.6.3.2.3.x2 Redirection from NR in FR1 to NR in FR1 for 2 Rx UE | Nokia |
| ##### A.6.3.2.3.x1 Redirection from NR in FR1 to E-UTRAN for 1 Rx UE A.6.3.2.3.x2 Redirection from NR in FR1 to E-UTRAN for 2 Rx UE | Nokia |

And in FR2:

| A.7.3.2 RRC Connection Mobility Control A.7.3.2.1 SA: RRC Re-establishment A.7.3.2.1.x Intra-frequency RRC Re-establishment in FR2 | Nokia |
| --- | --- |
| A.7.3.2.1.x Inter-frequency RRC Re-establishment in FR2 | Nokia |
| A.7.3.2.1.x Intra-frequency RRC Re-establishment in FR2 without serving cell timing | Nokia |
| A.7.3.2.2 Random Access A.7.3.2.2.x 4-step RA type c ontention based random access test in FR2 for NR Standalone | Xiaomi |
| A.7.3.2.2.x 4-step RA type n on-contention based random access test in FR2 for NR Standalone | Xiaomi |
| A.7.3.2.2.x 2-step RA type contention based random access test in FR2 for NR Standalone | vivo |
| A.7.3.2.2.x 2-step RA type n on-contention based random access test in FR2 for NR Standalone | vivo |
| A.7.3.2.3 SA: RRC Connection Release with Redirection A.7.3.2.3.x Redirection from NR in FR2 to NR in FR2 | Nokia |

Additionally, the following agreements were done in RAN4 #103 regarding RRM tests for RedCap UEs [2]:

### Sub-topic 6-2 Test configurations and type of tests Applicability requirements for RedCap Applicability requirements for RedCap test cases are introduced in A.3x. Duplex modes All RRM test cases are introduced for FDD, TDD and HD-FDD UEs. Accuracy of SS-RSRQ RedCap SS-RSRQ accuracy level is derived by relaxing the legacy SS-RSRQ accuracy level by the same level as agreed for RedCap SS-RSRP measurement compared to legacy SS-RSRP measurement. Accuracy of SS-SINR RedCap SS-SINR accuracy level is derived by relaxing the legacy SS-SINR accuracy level by the same level as agreed for RedCap SS-RSRP measurement compared to legacy SS-RSRP measurement.

In this document, Nokia’s views regarding the RRC mobility control test cases is presented.

- Test setup for RRC re-establishment and RRC connection release with redirection

In Annex A.6.3.2.1.1.1 the test purpose and environment for the RRC re-establishment tests in FR1 are described. Currently, for NR, the test cases consider the following configurations:

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In RedCap, the UEs are limited to a BW of 20 MHz. Therefore, a new TDD configuration with 20 MHz can be defined for replacing the current configuration 3. Additionally, in the last RAN4 meeting it was agreed that all RRM test cases are introduced for HD-FDD UEs as well, so a new configuration is needed.

To support the HD-FDD duplex mode, new configurations are needed for:

- PDSCH RMC configuration

- RSMI CORESET RMC configuration

- Dedicated CORESET RMC configuration

- TRS configuration

To the 20 MHz bandwidth in the test cases, a new SSB configuration is needed.

Since these configurations are common to other test cases, we discuss the specific proposals in our contribution in [3]. In this contribution, we discuss the specific test configurations for the RRC re-establishment and RRC redirection.

The following test configurations are defined for RRC re-establishment test cases in FR1:

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In a similar manner, new configurations are needed for the redirection from NR to NR in FR1 and from NR to E-UTRAN.

The following test configurations are defined for the Redirection from NR to NR test cases in FR1:

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 4 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
|  | Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

The following test configurations are defined for Redirection from NR to E-UTRAN test cases:

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| 7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE FDD |
| 8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

The changes in comparison to the NR test cases are highlighted in yellow in the proposals above.

In FR2, the maximum bandwidth for RedCap UEs is supported by current test configurations. Furthermore, the RRM core requirements for RedCap UEs and NR UEs are the same in this frequency band. Therefore, there is no need to introduce new test configurations.

NR test configurations for RRC re-establishment and RRC release with redirection in FR2 are reused for RRM test configurations for RedCap UEs.

- Conclusions

- In this contribution we discuss the test cases for RRC re-establishment and RRC redirection for RedCap UEs. The following observations and proposals are presented:

- To support the HD-FDD duplex mode, new configurations are needed for:

- PDSCH RMC configuration

- RSMI CORESET RMC configuration

- Dedicated CORESET RMC configuration

- TRS configuration

To the 20 MHz bandwidth in the test cases, a new SSB configuration is needed.

- The following test configurations are defined for RRC re-establishment test cases in FR1:

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

The following test configurations are defined for the Redirection from NR to NR test cases in FR1:

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 4 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
|  | Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

The following test configurations are defined for Redirection from NR to E-UTRAN test cases:

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| 7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE FDD |
| 8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

NR test configurations for RRC re-establishment and RRC release with redirection in FR2 are reused for RRM test configurations for RedCap UEs.

References

- R4-2210595, “Test case list for RedCap RRM performance part”, Ericsson, RAN4 #103-e

- R4-2210592, “WF on RedCap RRM requirements”, Ericsson, RAN3 #103-e

- R4-2212914, “Discussion on RRM test configurations for RedCap”, Nokia, Nokia Shanghai Bell