3GPP TSG-RAN4 Meeting #104-e	R4-2212918

# Electronic meeting, August 15 – August 26, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | dCR | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.133 TRP-specific beam failure recovery on NSC (EN-DC, FR1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Perf |  |  |  |  |  | Date: |  |  | 2022-8-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In Rel-17, core requirement for TRP specific beam failure recovery on NSC was introduced. It has been agreed that UE behaviours to TRP specific BFR should be tested for FR1 UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding the support of accuracy requirement for TRP specific BFR on cell with different PCI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - There is no way to verify if a UE supports TRP specific BFR on cell with different PCI is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | - A.4.5.5. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st version |  |  |  |  |  |  |  |  |

==========================Start of 1st change =============================

#### A.4.5.5.X	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode

##### A.4.5.5.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set (q0,0), (q0,1), (q1,0) and (q1,1) configured for a serving PSCell and a cell with PCID different from the serving cell, and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.X.5.5.1.1-1, A.X.5.5.1.1-2, A.X.5.5.1.1-3 and A.X.5.5.1.1-4 below. There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.X.5.5.1.1-1 shows the variation of the downlink SNR of the PCell and the SNR of the SSB in set q0 in the active PSCell to emulate SSB based beam failure. Figure A.X.5.5.1.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test 1.

Table A.X.5.5.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.X.5.5.X.1-2: General test parameters for FR1 PSCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 |  |  |  |  |
| Active E-UTRA PCell |  |  |  |  |  | Cell 1 |  |  |  |  |
| E-UTRA RF Channel Number |  |  |  |  |  | 1 |  |  |  |  |
| Active PSCell |  |  |  |  |  | Cell 2 |  | Cell 3 |  | Different PCID |
| RF Channel Number |  |  |  |  |  | 2 |  | TBD |  | Different QCL-D |
| Duplex mode |  |  |  | Config 1, 4 |  | FDD |  | FDD |  |  |
|  |  |  |  | Config 2, 3, 5, 6 |  | TDD |  | TDD |  |  |
| BWchannel |  |  |  | Config 1, 4 | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |  |
|  |  |  |  | Config 2, 5 |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |  |
|  |  |  |  | Config 3, 6 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |  |
| DL initial BWP configuration |  |  |  | Config 1, 2, 3, 4, 5, 6 |  | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration |  |  |  | Config 1, 2, 3, 4, 5, 6 |  | DLBWP.1.1 |  | DLBWP.1.1 |  |  |
| UL initial BWP configuration |  |  |  | Config 1, 2, 3, 4, 5, 6 |  | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| UL dedicated BWP configuration |  |  |  | Config 1, 2, 3, 4, 5, 6 |  | ULBWP.1.1 |  | ULBWP.1.1 |  |  |
| TDD |  |  |  | Config 1, 4 |  | Not Applicable |  | Not Applicable |  |  |
| Configuration |  |  |  | Config 2, 5 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  |  |  | Config 3, 6 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| CORESET |  |  |  | Config 1, 4 |  | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
| RMSI CORESET |  |  |  | Config 1, 4 |  | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
| Reference |  |  |  | Config 2, 5 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
| Channel |  |  |  | Config 3, 6 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  |  |  | Config 1, 4 |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
| Reference |  |  |  | Config 2, 5 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
| SSB Configuration |  |  |  |  |  | Config 1, 4 | SSB.3 FR1 | Config 7,10 | SSB.3 FR1 |  |
|  |  |  |  |  |  | Config 2, 5 | SSB.3 FR1 | Config 8, 11 | SSB.3 FR1 |  |
|  |  |  |  |  |  | Config 3, 6 | SSB.4 FR1 | Config 9, 12 | SSB.4 FR1 |  |
| SMTC Configuration |  |  |  | Config 1, 2, 4, 5 |  | SMTC.1 |  | SMTC.1 |  |  |
|  |  |  |  | Config 3, 6 |  | SMTC.1 |  | SMTC.1 |  |  |
| PDSCH/PDCCH subcarrier |  |  |  | Config 1, 2, 4, 5 |  | 15 KHz |  | 15 KHz |  |  |
| spacing |  |  |  | Config 3, 6 |  | 30 KHz |  | 30 KHz |  |  |
| PRACH Configuration |  |  |  | Config 1, 2, 4, 5 |  | Table A.3.8.2.2-1 |  | Table A.3.8.2.2-1 |  |  |
|  |  |  |  | Config 3, 6 |  | Table A.3.8.2.2-1 |  | Table A.3.8.2.2-1 |  |  |
| SSB Index assigned as BFD RS (q0,0) |  |  |  |  |  | 0 |  | - |  |  |
| SSB Index assigned as CBD RS (q1,0) |  |  |  |  |  | 1 |  | - |  |  |
| SSB Index assigned as BFD RS (q0,1) |  |  |  |  |  | - |  | TBD (3) |  |  |
| SSB Index assigned as CBD RS (q1,1) |  |  |  |  |  | - |  | TBD (4) |  |  |
| OCNG parameters |  |  |  |  |  | OP.1 |  | OP.1 |  |  |
| CP length |  |  |  |  |  | Normal |  | Normal |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  | 2x2 Low |  | 2x2 Low |  |  |
| Beam failure |  |  | DCI format |  |  | 1-0 |  | 1-0 |  |  |
| detection transmission parameters |  |  | Number of Control OFDM symbols |  |  | 2 |  | 2 |  |  |
|  |  |  | Aggregation level |  | CCE | 8 |  | 8 |  |  |
|  |  |  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |  | 0 |  |  |
|  |  |  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |  | 0 |  |  |
|  |  |  | DMRS precoder granularity |  |  | REG bundle size |  | REG bundle size |  |  |
|  |  |  | REG bundle size |  |  | 6 |  | 6 |  |  |
| DRX |  |  |  |  |  | OFF |  | OFF |  |  |
| Gap pattern ID |  |  |  |  |  | gp0 |  | gp0 |  |  |
| gapOffset |  |  |  |  |  | 0 |  | 0 |  |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent |  | absent |  | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | Config 1, 2, 4, 5 |  |  | dBm/SCS kHz | -98 |  | -98 |  | Threshold used for Qin_LR_SSB |
|  |  | Config 3, 6 |  |  |  | -95 |  | -95 |  |  |
| powerControlOffsetSS |  |  |  |  |  | db0 |  | db0 |  | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 |  | n1 |  | see TS 38.321 [7], clause 5.17 |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 |  | pbfd4 |  | see TS 38.321 [7], clause 5.17 |
| CSI-RS |  |  |  |  |  | Config 1, 4 | CSI-RS.1.1 FDD | Config 7, 10 | CSI-RS.1.1 FDD |  |
| configuration for |  |  |  |  |  | Config 2, 5 | CSI-RS.1.1 TDD | Config 8, 11 | CSI-RS.1.1 TDD |  |
| CSI reporting |  |  |  |  |  | Config 3, 6 | CSI-RS.2.1 TDD | Config 9, 12 | CSI-RS.2.1 TDD |  |
| CSI-RS for |  |  |  |  |  | Config 1, 4 | TRS.1.1 FDD | Config 7, 10 | TRS.1.1 FDD |  |
| tracking |  |  |  |  |  | Config 2, 5 | TRS.1.1 TDD | Config 8, 11 | TRS.1.1 TDD |  |
|  |  |  |  |  |  | Config 3, 6 | TRS.1.2 TDD | Config 9, 12 | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  |  |  | 0,1 |  | 3,4 |  |  |
| T310 timer |  |  |  |  | ms | 1000 |  | 1000 |  |  |
| N310 |  |  |  |  |  | 2 |  | 2 |  |  |
| T1 |  |  |  |  | s | 0.2 |  | 0.2 |  | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | 0.37 |  | 0.37 |  |  |
| T3 |  |  |  |  | s | 0.24 |  | 0.24 |  |  |
| T4 |  |  |  |  | s | 0 |  | 0 |  |  |
| T5 |  |  |  |  | s | 0.17 |  | 0.17 |  |  |
| D1 |  |  |  |  | s | 0.13 |  | 0.13 |  |  |
|  | Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |  |  |  |

Table A.X.5.5.X.1-3: Cell specific test parameters for FR1 PSCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0,0 | Config 1, 4 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2, 5 |  | 5 | -3 | -12 | -12 | -12 |
|  | Config 3, 6 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q0,1 | Config 7,10 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 8, 11 |  | 5 | -3 | -12 | -12 | -12 |
|  | Config 9, 12 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1,0 | Config 1, 4 | dB | -10 | -10 | 10 | 10 | 10 |
|  | Config 2, 5 |  | -10 | -10 | 10 | 10 | 10 |
|  | Config 3, 6 |  | -10 | -10 | 10 | 10 | 10 |
| SNR_SSB of set q1,1 | Config 7, 10 | dB | -10 | -10 | 10 | 10 | 10 |
|  | Config 8, 11 |  | -10 | -10 | 10 | 10 | 10 |
|  | Config 9, 12 |  | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1,0 | Config 1, 4 | dBm/SCS | -108 | -108 | -88 | -88 | -88 |
|  | Config 2, 5 | kHz | -108 | -108 | -88 | -88 | -88 |
|  | Config 3, 6 |  | -105 | -105 | -85 | -85 | -85 |
| SSB_RP of set q1,1 | Config 7, 10 | dBm/SCS | -108 | -108 | -88 | -88 | -88 |
|  | Config 8, 11 | kHz | -108 | -108 | -88 | -88 | -88 |
|  | Config 9, 12 |  | -105 | -105 | -85 | -85 | -85 |
|  | Config 1, 4 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2, 5 |  | -98 |  |  |  |  |
|  | Config 3, 6 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Figure A.X.5.5.1.1-1: SNR and L1-RSRP variation SSB for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.X.5.5.1.2	Test Requirements

Test requirements are applied to TRP specific report respectively on (q0,0), (q0,1) for cell-1 and (q1,0), (q1,1)  for cell-2 repectively as Figure A.X.5.5.1.1-1.

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 120+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

==========================End of 2th change =============================