GPP TSG-RAN WG4 Meeting # 104-e	R4-2212951

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.8.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance requirements for SRS antenna port switching

Document for:	Discussion

# Introduction

In last RAN4 meeting, the performance requirements of SRS antenna port switching were discussed with agreements captured in [1]. In this paper, we further provide our views on remaining issues on test cases design to verify the corresponding requirements.

# Discussion

One of the controversial issues discussed in last meeting is how to define test cases for scenario one sync case. It is agreed to verify symbol level requirements for scenarios 1 sync case. However, it is still FFS on how to design the corresponding test cases. The status are summarized as follows:

Issue 3-1-2: scenarios for SRS antenna port switching testing Agreement in GTW (May 13th) - Interruption requirement (symbol-level) for scenario 1 sync case - Interruption requirement (slot-level) for scenario 2 Issue 3-2-5: SRS configuration for scenario 1 sync case (symbol-level interruption) Agreements: - Counting missed ACK/NACK number (it’s a slot level interruption counting) - Option 1: - Define test cases for scenario 1 sync case by allocating the SRS resource at the last but one symbol of slot and the interruption on victim CCs should be no longer than 1 slot (subframe for E-UTRA) when the SCS of aggressor CC and victim CC is 15/30 KHz. - Option 2: - Define test cases for scenario 1 sync case by allocating the SRS resource before the last two symbols or more of slot and the interruption on victim CCs should be no longer than 1 slot (subframe for E-UTRA) when the SCS of aggressor CC and victim CC is 15/30 KHz.

Regarding how to verify the symbol level interruption, it is agreed to use the same mechanism as slot level interruption by counting the missing ACK/NACK, and the location of SRS resource is chosen that only one slot will be interrupted. It is proposed in last meeting that the SRS resource can be allocated at the last but one symbol of a slot. Companies comment that for some SCS combinations, considering TA and MRTD, the interruption may span 2 slots. Thus, in this paper, we analysis the issue in a case by case manner.

When evaluating the impact on UL of victim cells, it could be assumed that both aggressor Cell and victim Cell are within the same TAG. Then, the UL transmit timing is aligned (based on same reference cell and same NTA and NTA_offset) as shown below. It means if the SRS resource is allocated as the last but one symbol of aggressor Cell, it can only impact one slot of victim Cell for all SCS combinations.

Fig.1 Interruption on UL

For the impact to DL of victim Cell, the interruption location may depend on the TA and MRTD as shown below. For simplification, we can assume NTA is zero, then the timing difference between aggressor Cell UL and victim Cell DL is (NTA_offset + MRTD, NTA_offset - MRTD) which is (46 us, -20 us). To guarantee that the interruption will not span two slots on victims CCs, additional 1~2 symbols are enough. Then it is proposed to allocate the SRS resource in the 10th symbol in a slot.

Fig.2 Interruption on DL

Proposal 1: 	Define test cases for scenario 1 sync case by allocating the SRS resource at the 10th symbol of a slot and the interruption on victim CCs should be no longer than 1 slot (subframe for E-UTRA) when the SCS of aggressor CC and victim CC is 15/30 KHz.

Another issue is about the duplex/SCS configurations in the test. The status are summarized as follows:

Issue 3-2-3: General configuration – SCS and BW Agreements: - for configuration of test cases for SRS antenna port switching, to consider following cases: - 15 kHz SSB SCS with 10 MHz bandwidth for FDD - 15 kHz SSB SCS with 10 MHz bandwidth for TDD - 30 kHz SSB SCS with 40 MHz bandwidth for TDD - FFS: - Option 1: above configurations applies for aggressor cell, FFS for victim cell - Option 2: above configurations applies for both aggressor cell and victim cell, and such configuration may be different between aggressor cell and victim cell. - Option 3: above configurations applies for both aggressor cell and victim cell, and such configuration is same between aggressor and victim

It could be observed from analysis above that the interruption requirements can be different for different SCS combinations. It is quite restricted to only considering same duplex mode with same SCS. It means for FDD+TDD CA or 15 kHz + 30 kHz combinations, there is no test cases can apply to such BC.

Observation 1: It is restricted to only define test cases for same duplex mode with same SCS.

However, after checking existing Rel-16/17 test cases, also most all of them are only considering same duplex mode with same SCS.

Observation 2: In legacy test cases, only same duplex mode with same SCS are considered in test configurations.

Thus, we think this is a generic issue not only for the test cases for SRS antenna port switching. We propose to use a generic method to fix the issue and minimize the impact on the spec. One feasible approach is to state in the test configurations table that the configurations for aggressor cell and victim cell can be chosen independently. For instance, for SRS AS for EN-DC, we propose to define the test configurations as follows. In this, it is possible to test 15 kHz SCS + 30 kHz as the configuration for LTE PCell+ NR PSCell and NR SCell are chosen independently.

| Config | Description |  |
| --- | --- | --- |
|  | LTE PCell + NR PSCell Note 2 | NR SCell Note2 |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |  |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |  |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Test configuration for LTE PCell+ NR PSCell and NR SCell are chosen independently. |  |  |

Proposal 2: Configuration for aggressor cell and victim cell can be different. The configuration for different cells can be chosen independently in the test.

# Conclusions

Proposal 1: 	Define test cases for scenario 1 sync case by allocating the SRS resource at the 10th symbol of a slot and the interruption on victim CCs should be no longer than 1 slot (subframe for E-UTRA) when the SCS of aggressor CC and victim CC is 15/30 KHz.

Observation 1: It is restricted to only define test cases for same duplex mode with same SCS.

Observation 2: In legacy test cases, only same duplex mode with same SCS are considered in test configurations.

Proposal 2: Configuration for aggressor cell and victim cell can be different. The configuration for different cells can be chosen independently in the test.

# References

[1] R4-2210996 On test case list for SRS antenna port switching, Apple