3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212989

Electronic Meeting, August 15 – August 26, 2022

Title: 	Discussion on	Mobility requirements due to UE complexity reduction

Source: 	Huawei, HiSilicon

Agenda item:	9.18.3.1.2

Document for:	Discussion

Introduction

This contribution further provides analysis on the remaining issue in mobility requirements for RedCap UE.

Discussion

Handover

One open issue on Trs in handover requirement is captured in WF [R4-2210592].

Mismatch between SMTC configurations in scenario 1, 2, 3 and 4 Candidate options: - Option 1 (E///, vivo): RAN4 to further discuss the possible additional delay due to SMTC configuration mismatch, such as between CD-SSB measurement and NCD-SSB HO without default SMTC configuration. - Option 1a (CMCC, HW): Clarification needed in spec. - Option 2 (Apple, Xiaomi): FFS, some clarification may be needed. - Option 3 (MTK): No need to discuss the issue.

It is agreed in last meeting that handover scenario 2, 3 and 4 are precluded in R17 RedCap WI. The handover requirements are specified for the following scenarios:

-	Handover to a target cell’s initial BWP associated with CD-SSB;

-	Handover to a target cell’s specific Redcap BWP associated with NCD-SSB besides to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP).

The first is the legacy scenario and the second is new added scenario where directly handover to NCD-SSB.

In legacy requirements, Ts is specified as below. The logic is:

- If NW configures SMTC in handover command, Trs equals the SMTC configuration;

- If NW does not configure SMTC in handover command, Trs is the SMTC configured in measObjectNR;

- If the UE has been provided with smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

- If no SMTC is configured in handover command or no measObjectNR on this frequency, UE will assume the Trs=5ms; There is no requirement if the SSB transmission periodicity is not 5ms;

Legacy Trs definition in TS38.133 Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cellin the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

Regarding Redcap, RAN2 has made some clarification on SMTC in handover command (ReconfigurationWithSync) for Redcap UE.

ReconfigurationWithSync ::= SEQUENCE { spCellConfigCommon ServingCellConfigCommon OPTIONAL, -- Need M newUE-Identity RNTI-Value, t304 ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000}, rach-ConfigDedicated CHOICE { uplink RACH-ConfigDedicated, supplementaryUplink RACH-ConfigDedicated } OPTIONAL, -- Need N ..., [[ smtc SSB-MTC OPTIONAL -- Need S ]], [[ daps-UplinkPowerConfig-r16 DAPS-UplinkPowerConfig-r16 OPTIONAL -- Need N ]], [[ sl-PathSwitchConfig-r17 SL-PathSwitchConfig-r17 OPTIONAL -- DirectToIndirect-PathSwitch ]] }

smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change and NR PCell change. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon. For case of NR PCell change, the smtc is based on the timing reference of (source) PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If both this field and targetCellSMTC-SCG are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message. For a RedCap UE, if the first active DL BWP included in this RRC message is configured with nonCellDefiningSSB-r17, this field corresponds to the NCD-SSB indicated by nonCellDefiningSSB-r17, otherwise, this field corresponds to the CD-SSB indicated by absoluteFrequencySSB in frequencyInfoDL.

The above SMTC description specified for Redcap UE can be interpreted as:

- When SMTC is configured in ReconfigurationWithSync, if the first active DL BWP included in handover command is configured with NCD-SSB, SMTC corresponds to the NCD-SSB, otherwise, SMTC corresponds to the CD-SSB indicated by absoluteFrequencySSB in frequencyInfoDL.

- When SMTC is not configured in in ReconfigurationWithSync, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing.

  - The MO is the NCD-SSB MO if the NCD-SSB is configured in first active DL BWP included in handover command, otherwise the MO is CD-SSB MO.

  - The SMTC in MO may also have SMTC2 as legacy.

Based on the above understanding, the Trs in handover requirements for RedCap UE shall be made some accordingly clarification. Trs for RedCap UE can be specified as below.

- Trs is the SMTC periodicity of NCD-SSB indicated by nonCellDefiningSSB-r17 if the first active DL BWP included in handover command is configured with nonCellDefiningSSB-r17, otherwise, Trs is the SMTC periodicity of the CD-SSB indicated by absoluteFrequencySSB in frequencyInfoDL in handover command.

- If the UE is not provided SMTC configuration in handover command, Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

  - If the UE has been provided with smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

- If the UE is not provided SMTC configuration in handover or measurement object on this frequency, Trs is 5ms assuming the SSB transmission periodicity is 5ms. There is no handover requirement if the SSB transmission periodicity is not 5ms.

It shall be noted that dual connectivity is not supported for RedCap UE. Therefore the related description shall be removed for RedCap UE.

Proposal: Trs in handover requirements for RedCap UE is specified as:

Trs is the SMTC periodicity of NCD-SSB indicated by nonCellDefiningSSB-r17 if the first active DL BWP included in handover command is configured with nonCellDefiningSSB-r17, otherwise, Trs is the SMTC periodicity of the CD-SSB indicated by absoluteFrequencySSB in frequencyInfoDL in handover command. If the UE is not provided SMTC configuration in handover command, Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

Conclusions

This contribution provides analysis on remaining issue in mobility requirement. The definition for Trs for RedCap UE is provided:

Proposal: Trs in handover requirements for RedCap UE is defined as:

Trs is the SMTC periodicity of NCD-SSB indicated by nonCellDefiningSSB-r17 if the first active DL BWP included in handover command is configured with nonCellDefiningSSB-r17, otherwise, Trs is the SMTC periodicity of the CD-SSB indicated by absoluteFrequencySSB in frequencyInfoDL in handover command. If the UE is not provided SMTC configuration in handover command, Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

Reference

- R4-2210592, WF on RedCap RRM requirements, Ericsson