3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212991

Electronic Meeting, August 15 – August 26, 2022

Title: 	Discussion on	signaling characteristics due to UE complexity reduction

Source: 	Huawei, HiSilicon

Agenda item:	9.18.3.1.4

Document for:	Discussion

Introduction

This contribution will further discuss the remaining issues on the signaling characteristics for RedCap UE.

Discussion

The core requirements for signalling characteristics are completed in last meeting. One open issue is on whether to define further RLM/BFD relaxation for RedCap UE.

Whether to define further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17 Candidate options: - Option 1 (QC): RAN4 to define further relaxations (e.g., larger K values) to RLM/BFD measurements for a Rel-17 RedCap UE that is configured with and satisfies the stationary criterion. - Option 2 (HW, Apple, CMCC, vivo, OPPO): Relaxed RLM/BFD based on Rel-17 UE power saving WI is out of RedCap WI scope. - Option 3 (E///, MTK, Xiaomi, MTK): FFS under maintenance If further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI is defined for RedCap in Rel-17 Candidate options: - Option 1 (QC): RAN4 to consider defining the applicability of power-saving enhancements to longer DRX cycles (up-to 160ms) to allow stationary UEs save significant power.

In R17 RLM/BFD WI, it had taken several meeting cycles to evaluate the power saving gain with RLM/BFD relaxation. The conclusion is agree to relax RLM/BFD evaluation period for short DRX cycles (not larger than 80ms DRX). It is not necessary to relax for larger DRX cycles as no outstanding gain is observed.

For RedCap UE with 1RX, the evaluation period for Qout for both SSB-based and CSI-RS based RLM and BFD are doubled. It can be expected that less power saving gain is achieved with further relaxation on these requirements. Therefore we think not to define further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17.

Proposal 1: Not to define further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17.

Conclusions

This contribution provides analysis on the signaling characteristics due to complexity reduction for RedCap UE. The following proposal is provided:

Proposal 1: Not to define further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17.

Reference

- R4-2210592, WF on RedCap RRM requirements, Ericsson