3GPP TSG-RAN WG4 Meeting #104-e	R4-2213004

Electronic Meeting, August 15 – August 26, 2022

Title: 	Discussion on handover test for RedCap UE

Source: 	Huawei, HiSilicon

Agenda item:	9.18.4.2.4

Document for:	Discussion

Introduction

This contribution provides consideration on handover test case for RedCap UE.

Discussion

The handover requirements specified for RedCap UE are applicable for the following handover scenarios:

Handover to a target cell’s initial BWP associated with CD-SSB;

Handover to a target cell’s specific Redcap BWP associated with NCD-SSB besides to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP).

The first scenario is the same as legacy. Therefore we propose to verify the handover functionality to NCD-SSB for RedCap UE.

Proposal 1: Verify the functionality of intra-NR handover NCD-SSB for RedCap UE.

Regarding LTE-NR handover for RedCap UE, as the 1RX RedCap UE handover requirements have been verified in intra-NR, it is proposed that only E-UTRA-NR handover for 2RX RedCap UE test is to be verified. Similar the functionality of handover to NCD-SSB has already been verified in intra-NR test, therefore the simple handover to CD-SSB scenario is proposed to be tested.

Proposal 2: E-UTRA-NR handover to CD-SSB for 2RX RedCap UE test is to be verified.

The maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. However the current SSB configurations are only defined for (channel bandwidth 10MHz with SCS 15KHz) and (channel bandwidth 40MHz with SCS 30KHz). It is proposed to add new SSB configuration for (channel bandwidth 20MHz with SCS 30KHz). In addition, as the SSB periodicity for NCD-SSB is not less than CD-SSB. From network configuration overload perspective, NCD-SSB with 20ms periodicity would degrade about 5% overload loss. Therefore 80ms periodicity of NCD-SSB (Tssb) is a good trade-off.

Proposal 3: Add the following new SSB patterns and SMTC pattern in FR1 to consider 20MHz and 80ms NCD-SSB periodicity.

SSB.x FR1: SSB Pattern x for SSB SCS=15 kHz in 10 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 10 MHz |
| SSB SCS | 15 kHz |
| SSB periodicity (TSSB) | 80 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 2 | 2-5 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

SSB.y FR1: SSB Pattern for SSB SCS=30 kHz in 20 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 20 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 80 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 |
| Slot numbers containing SSB Note 3 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. Note 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves |  |

SMTC.x: SMTC Pattern x for SMTC period = 80 ms and duration = 1 ms

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 80 ms |
| SMTC offset | 0 ms |
| SMTC duration | 1 ms |

As the L3 measurement accuracy of 1RX RedCap UE is agreed to be relaxed by 1dB compared to legacy requirements [1]. Therefore to guarantee1RX RedCap UE handover to target cell, the SSB-RSRP level difference between serving cell and target cell shall be larger than 4dB.

Proposal 4: To guarantee 1RX RedCap UE handover to target cell, the SSB-RSRP level difference between serving cell and target cell shall be larger than 4dB in test.

Conclusions

This contribution provides the analysis on handover test for RedCap UE. The following proposals are provided:

Proposal 1: Verify the functionality of intra-NR handover NCD-SSB for RedCap UE.

Proposal 2: E-UTRA-NR handover to CD-SSB for 2RX RedCap UE test is to be verified.

Proposal 3: Add the following new SSB patterns and SMTC pattern in FR1 to consider 20MHz and 80ms NCD-SSB periodicity.

Proposal 4: To guarantee 1RX RedCap UE handover to target cell, the SSB-RSRP level difference between serving cell and target cell shall be larger than 4dB in test.

Reference

[1]R4-2210592, WF on RedCap RRM requirements, Ericsson