3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213017

# Electronic Meeting, Aug 15- Aug 26, 2022

Agenda item:	9.20.2.3

Source: 	vivo

Title: 	Discussion on test case for conditional PSCell addition and change

Document for:	Discussion

- Introduction

The conditional PSCell addition and change requirements have been introduced. And accordingly, the test case list of conditional PSCell addition have been agreed in RAN4 #103-e meeting [1]. On the other hand, the test cases of conditional PSCell change have not been discussed and defined so far. In this contribution, we will provide the discussion on the details of test cases for conditional PSCell addition and a test case list for conditional PSCell change for reference.

- Discussion

2.1.	Test cases of Conditional PSCell addition and release

In the last meeting, test cases of conditional PSCell addition and release have been agreed. According to the work split, we are responsible for the test case on conditional PSCell addition and release delay in FR1 ENDC and in FR2 NR-DC. The concrete content of the corresponding CR is provided in [2][3].

In this paper, we would like to give some analysis here. In general, the basic procedure for conditional PSCell addition and release is similar between scenario 1 in FR1 EN-DC and scenario 2 in FR2 NR-DC. Specifically, compared with PSCell addition, we understand that conditional PSCell addition shall be inclined to the configuration and satisfaction of condition. And the remaining procedures are performed similarly to legacy tests for PSCell addition and release. Furthermore, considering the UE does not have any information of PSCell during T1, both the cell detection procedure and SSB based measurement need to be performed. Further, the calculation process and details are shown in the table below.

Table 1: Test case list for conditional PSCell addition and release for 38.133

| Test case number | Test case | calculation | Description |
| --- | --- | --- | --- |
| 1 | A.4.5.X1 Conditional PSCell addition and release delay (FR1 ENDC) A.4.5.X1.1 Addition and Release Delay of NR PSCell | T2: 600 (TPSS/SSS_sync_intra) + 120 (TSSB_time_index_inter) + 320 (TSSB_measurement_period_inter) +10 (TUE_preparation) + 62 (Tinterrupt) = 1040 + 10 + 62 ms=1112 ms Tinterrupt = Tprocessing + TPSCell_ DU + T∆ + Tmargin = 20 + 20 + 20 + 2 ms = 62 ms | At the start of time duration T1, the UE does not have any timing information of Cell 2. NR shall configure a condition implying addition to cell 2 during T1, at a time earlier than TRRC_delay before the beginning of T2. At the start of T2, cell 2 becomes detectable and meets the addition condition. Reception by the test system of the PRACH preamble defines the start of T3. During T3, the UE shall send periodic CSI reports in PSCell. After having received at least one such report, the test system shall send an RRC message instructing the UE to release the PSCell. Reception by the UE of the RRC message defines the start of T4. During T4, the UE shall release the PSCell. |
| 2 | A.7.5.X3 Conditional PSCell addition and release delay (FR2 SA) A.7.5.X3.1 Addition and Release Delay of NR PSCell | T2: 2560 (TSSB_measurement_period_inter) + 1600 (TSSB_time_index_inter) +2560 (TSSB_measurement_period_inter) +10 (TUE_preparation) + 62 (Tinterrupt) = 6720 + 10 + 62 ms= 6792 ms (power class 1) or 4160+10+62ms = 4232 ms (power classes 2,3 and 4) Tinterrupt = Tprocessing + TPSCell_ DU + T∆ + Tmargin = 20 + 20 + 20 + 2 ms = 62 ms | At the start of time duration T1, the UE does not have any timing information of Cell 2. NR shall configure a condition implying addition to cell 2 during T1, at a time earlier than TRRC before the beginning of T2. At the start of T2, cell 2 becomes detectable and meets the addition condition. Reception by the test system of the PRACH preamble defines the start of T3. During T3, the UE shall send periodic CSI reports in PSCell. After having received at least one such report, the test system shall send an RRC message instructing the UE to release the PSCell. Reception by the UE of the RRC message defines the start of T4. During T4, the UE shall release the PSCell |

2.2.	New Test case list proposal (conditional PSCell change)

Compared with the current requirement on conditional PSCell addition, conditional PSCell change do not only focus inter-frequency measurement but also intra-frequency measurement. Given this difference, we propose that the test cases for conditional PSCell change need to be considered separately and differentiated with the test cases for conditional PSCell addition. For conditional PSCell change, the network preconfigures change conditions and corresponding threshold. When the condition is satisfied, the UE shall perform source Cell release and target Cell reconfiguration. From the basic procedure perspective, it is not difficult to find that the procedure for conditional PSCell change is similar with conditional handover. From this perspective, the current test requirement on intra-frequency conditional handover can be used as the baseline.

As listed above, we propose that conditional PSCell change tests should include both intra-frequency and inter-frequency tests for EN-DC and NR-DC.

Table 2: Test case list for conditional PSCell change for 38.133

| Test case number | Test case | Description |
| --- | --- | --- |
| 1 | A.4.5.X1 Conditional PSCell change delay (FR1 ENDC) A.4.5.X1.1 Conditional PSCell change Delay of NR PSCell | Focus on intra-frequency measurement on PSCell: There are two cells in the test, Cell 1 and a FR1 neighbour cell (Cell 2) on the same frequency as the Cell 1. |
| 2 | A.5.5.X2 Conditional PSCell change delay (FR2 ENDC) A.5.5.X2.1 Conditional PSCell change Delay of NR PSCell | Same as above |
| 3 | A.7.5.X3 Conditional PSCell change delay (FR2 SA) A.7.5.X3.1 Conditional PSCell change Delay of NR PSCell | Focus on inter-frequency measurement on PSCell: There are two cells in the test, Cell 1 and a FR1 neighbour cell (Cell 2) on the different frequency with the Cell 1. |

Proposal 1:

- Given that conditional PSCell change includes both inter-frequency and intra-frequency measurements, RAN4 shall develop new test cases for conditional PSCell change based on the legacy tests for conditional handover.

- Conclusions

Proposal 1: Given that conditional PSCell change includes both inter-frequency and intra-frequency measurements, RAN4 shall develop new test cases for conditional PSCell change based on the legacy tests for conditional handover.

- Reference

[1] R4-2210605 WF on R17 further Multi-RAT Dual-Connectivity enhancements, Huawei

[2] R4-2213020 Draft CR on test case for conditional addition and release of PSCell in FR1 EN-DC, vivo

[3] R4-2213021 Draft CR on test case for conditional addition and release of PSCell in FR2 SA, vivo